WESTERN DIGITAL

OS/2 REQUESTER DRIVER FOR

"PLUS" FAMILY LAN ADAPTER BOARDS

INSTALLING OS/2 REQUESTER

-------------------------

MINIMUM REQUIREMENTS:

1) 80286, 80386, or 80486-Based System

2) 2.5 MB System RAM

3) 10 MB Hard Disk

4) OS/2 Installation Package (Version 1.10 or later)

5) NetWare for OS/2 (Version 1.1 or later)

a) OS/2 Requester Software

b) OS/2 Public Software

6) Driver Software for OS/2

a) wdplus.sys

b) net.cfg

INSTALLATION PROCEDURE:

1) Install OS/2 using the instructions provided with

the OS/2 package.

2) Install NetWare using the instructions provided with

the NetWare package.

3) Copy 'WDPLUS.SYS' to either the root directory or to a

subdirectory off the root, such as C:\DRIVER or C:\NETWARE.

4) Copy NET.CFG to the root directory if one does not

already exist.  If one does exist, follow these instructions:

a)	Copy the new NET.CFG to the root directory making sure

to give the new NET.CFG a different name from the existing

NET.CFG.  For example:

copy a:\net.cfg c:\netware\newnet.cfg

b)	Append the new configuration file to the end of the

existing NET.CFG file.  For example:

append c:\net.cfg;c:\newnet.cfg

This file is used to configure the entire operating system,

not just the driver.

5) Modify CONFIG.SYS

Note:  The following instructions are also provided

in the documentation files included in the

NetWare installation diskette.

a)	After installing the NetWare package, the files

CONFIG.PST and AUTOEXEC.PST (for version 1.2 only)

will appear in the root directory. These files must be

appended to the CONFIG.SYS and AUTOEXEC.BAT files

respectively.  For example:

append c:\config.sys c:\config.pst

append c:\autoexec.bat c:\autoexec.pst

b) Use your text editor to insert 'C:\NETWARE' as the first

directory path in your LIBPATH statement.

c) Use your text editor to insert 'C:\NETWARE' as the last

directory path in your DPATH statement.

d) Add the statement which loads the LAN interface

board driver just before the IPX.SYS statement.  For

example:

device=C:\wdplus.sys

or

device=C:\DRIVER\wdplus.sys

or

device=C:\NETWARE\wdplus.sys

depending on whether WDPLUS.SYS is loaded from the

root, the \DRIVER, or the \NETWARE directory.

USING NET.CFG

1) There is no need to use the NET.CFG file if the

Western Digital PLUS adapter is an AT adapter

configured with the default settings:

Base I/O Address: 280

Interrupt IRQ: 	  3

RAM Base Address: D000

2) If the adapter being used is software configurable

then only the Base I/O Address (if it is not 280)

must be specified.

3) To change the Base I/O Address, locate the line in

NET.CFG which reads

Port #1 280 20

and change '280' to the appropriate Base I/O Address.

3) To change the Interrupt IRQ, locate the line in

NET.CFG which reads

Int #1 3

and change '3' to the appropriate Interrupt IRQ.

4) To change the RAM Base Address, locate the line in

NET.CFG which reads

Mem #1 000D0000 2000/10

and change '000D0000' to the appropriate RAM Base

Address.

For example,

Mem #1 000C8000 2000/10

causes the driver to use C8000 as the RAM Base Address.

5) When using a Micro Channel machine, the PS/2 Slot

entry in the NET.CFG file must be specified.  Locate

the line which reads

PS/2 Slot ?

and change the '?' to the slot number desired.

Note: The ';' in the first column of the NET.CFG file

denotes an ignored line.  Delete this symbol

from all used lines and insert it into all unused

lines, or delete unused lines altogether.

Western Digital is a registered trademark and the PLUS family of adapters

are trademarks of Western Digital Corporation. Other trademarks mentioned

herein belong to their respective companies.

COPYRIGHT (c) 1991 Western Digital Corporation

All rights reserve