� Brief docs for PnP Resource lister & diagnostic tool.

This program reports on the internals of Plug and Play (PnP) BIOS equippped

PC's. It displays all the PnP data the machine contains, allowing you to more

easily fault-find hardware conflicts as well as locate settings assigned to PnP

hardware. This code won't do anything on a machine that doesn't have a PnP

BIOS.

This tool is best used if piped to a file ie: "pnp > logfile.txt" then use any

text reader/editor to view the results as they may span several pages. Since

PNP supprtos pipes, you can also do PNP | more and the like.

This tool only works properly if run from a plain boot - it will crash if run

under a memory manager or win95... probably because the system is in V86 mode

at those times, yet we always use real mode versions of all the function calls.

Comments to the author are welcomed:   chart@hyperlink.net.au

home page for updates:                 http://members.hyperlink.net.au/~chart

PLEASE get in contact with me if you wish to further the exploration and

documentation of PnP programming. Full source code is included so you too can

learn how to do PnP Programming... it's not that hard!!

� PnP Books.

I have two to recommend: "Programming Plug and Play" by James Kelsey, Sams

Publishing, 1995, ISBN 0-672-30703-0 and "Plug and Play System Architecture" by

Tom Shanley, Mindshare/Addison Wesley, ISBN 0-201-41013-3. Don't expect to

learn PnP programming without these books!

� "demo.log" is a sample file generated by my Pentium 133 system; included so

you can see how things are supposed to look -- in case you have problems

getting pnp to work on your machine, or wonder if what you're seeing is

'normal'.

� Commandline format:    PNP [N]

N is optional and specifies that you'd like to see the pnp data assigned for

the [N]ext boot, not the current boot (which is the default). Normally the two

are the same, but not always, espescially if you've just been messing around

with PnP card setup disks and the like, but have not yet re-booted.

� "Understanding" what you see on the screen.

PnP outputs 4 'sections' worth of information; each of which should 'agree'

with the other sections... if they don't, you may have a machine with stange

BIOS bugs... try "Reset ESCD data" option in your BIOS, if it has it.

The first 'section' displays the PnP info available from the PnP BIOS. Normally

this covers the "built into" motherboard devices only. This info is fairly

straight forward to interpret; just read it! Anything listed between "Alternat

config start" and "Alternate config end" represents other possible

configurations of that device - but they are NOT currently in use.

The second section displays info from PnPISA cards, if any are present.

Firstly, all the possible settings for that device are listed (good for finding

out if a certain card can be set to certain settings you might wish to use),

then the actual 'current' settings are displayed. PnPISA cards can have

several, possibly unrelated devices on the came card; for example most PnP

sound cards have the SoundBlaster compatable registers as one logical device,

the joystick port as another, and the Midi port as a third. PNP can now cope

with these cards correctly.

The third section tells you wether a static resource configuration exists or

not - This is used on PC's without an ESCD. At this time, the static resource

info is not displayed (Mostly because I never found a machine that actually

uses this function!).

The fourth section displays the ESCD. ESCD is a method of tying all PnP info

from ALL devices (regardless of type) into one massive data structure.. the

concept bieng that if all info is in one place it will be easiest to work

with.. In reality the ESCD fails in this task (although it does tie most info

together); it is necessary to 'double-check' it's contents for accuracy, etc as

some programs (espcially Win95) do rely on the ESCD. The ESCD should list PCI,

EISA, MCA & PnPISA devices, as well as systemboard resources and possibly even

non-PnP devices in some cases.

� Other Notes

At this time, PnP is incomplete. Some information is not yet decoded (you see a

message saying "tripped out on (big|small) code xx"), and there may be bugs in

the program. Please let me know of anything that doesn't work for you, and I'll

try to fix it. Please include an output of the program with your message - what

messages may appear should help me to work out 'what went wrong'.

Recently I've accomplised a lot - found a copy of the 'real' EISA specs (Thanks

Tony - you know who you are) and started work on the PnP database... this

project gets ever closer to completion, but still needs YOUR help!!! Please -

tell me if this programs helps you, nor not.. either way I'd like to know!!!

� PnP & Win95

On machins with an ESCD, Win95 may insert references to non-PnP devices without

your knowledge!!! This is, of course, to help the BIOS come up with a 'better'

working boot configuration (by helping the BIOS know about used resources that

the BIOS couldn't normally detect). This explains why (A) re-booting a

non-working win95 PC sometimes makes it work (and also why win95 wants to

reboot all the goddamn time when adding new hardware), and (B) also explains

how those entries got there in the first place.

Win95 removes entries from the ESCD when you delete a device from Device

Manager; *however* win95 won't remove ESCD entries it doesn't have device

manager entries for. Thus, the ESCD can be left in a state where 'false'

entries remain if a non-PnP device is removed from the system and, say, win95

is deleted, re-installed, etc at the same time without actually deleting the

existing non-PnP device from device manager first.

Some BIOS's have a "Reset Configuration data" CMOS setup option to erase the

ESCD and 'start over'. In this case, USE IT!!  If the BIOS doesn't have such a

function, you are in trouble!! I hope to offer a 'fix' for this soon. Stay

tuned!

Why does this happen?

It's a Catch 22 situation. Win95 only alters the ESCD according to changes made

in Device Manger. The BIOS only updates PnP info in the ESCD, not non-PnP info.

Win95 trusts the BIOS to have the ESCD info 'right' for devices it does not

have listed in it's device manager registry, and similarly the BIOS doesn't

delete any non-PnP entires other programs put there, as it is trusting that

those devices really do exist and need to be taken into account.

Each is trusting the other and nobody is doing the hosekeeping. Except us, when

we are armed with this program! Remember, this only applies to non-PnP devices

- namely Legacy VESA & ISA devices - it can never apply to PnP compatable

devices.

In my system, for example, my non PnP sound card gets it's resources listed in

the ESCD.. in turn, the BIOS knows not to use those resources anywhere else.

� Revision history

Version 0.03�

Added more pnpxxxx codes.. this may need a text database in future?!?!

Tidied up some display formatting errors

More decoding of ESCD info.. still need EISA specs to do this properly!!

A bug with one too many ESCD records being decoded fixed

Fixed up some ugliness with ESCD data structure code

General housekeeping

Version 0.04�

Made ESCD loop only as many times as header indicates devices... avoids

runtime error 201 on some systems. Increased ESCD buffer to 32kb max; just in

case as there seem to be a few 16kb ESCD's out there...

Version 0.05�

Added PnPISA freeform info partial decode & display; figured out how to read

PnPISA ID's finally!

Version 0.06�

Added ability to list alternate configuration info for PnP BIOS devices, and to

select the "now" or "next boot" option at runtime.

Tidyed up the ESCD display; raw hex-dump of data refined to hex-dump for each

device - much easier to read and understand.

Added 'h' hexadecimal indicator to all numbers displayed in hex format.

Added decoding of PnPISA card resources & general info.

A lot of 'behind the scenes' adjustments went on in this version, including

display reformatting, bugfixing, additional information, and also testing. I

feel that this is the most reliable version yet of this tool.. probably the

first version that could actually be useful in fault finding.

Version 0.07�

Added pnpid.txt; list of pnp resources that pnp.exe reads. Having the pnp

info in an easily updated seperate text file makes updating the list so much

easier.  We should read this during pnpisa operations to recongise pnpisa

'compatable' devices, but we don't. yet.

Version 0.08�

Fixed small bug with decoding LogicalDevID - found when testing a Dlink PnPISA

network card.

Fixed up display of allocated resources for PnPISA devices - no more long lists

of 'Not Assigned' confusing everyone.

Version 0.09�

Major work on ESCD info - I have 'deduced' the decoding of the majority of ESCD

data using the results of an ESCD exploration program written by Phoenix (BIOS).

I still don't have everything worked out, however it's MUCH better than what we

started with!!

Version 0.10�

Fixed bug with PnPISA cards that have more than one logical device; now we list

ALL the resources for all the logical devices... phew, that was confusing!

I have gotten hold of the 'offical' EISA standards (from BCPR Services); many

updates, corrections, blah blah from these standards. The program is now mostly

complete!

Program now attempts to identify any EISA type device ID's discovered

throughout the program, not just in the first section. Serious work on

maintaining pnpid.txt has begun and will get it's own webpage soon.

Version 0.12�

Fixed more bugs, added support for "ISA Memory range" PnPISA info type.

Version 0.13�

A quick-release to trial a bug-squash; helped but didn't fix all the problems.

Version 0.14�

Fixed a serious bug that caused all hell to break loose on most modern

motherboards - the PnP device node numbers are not always contiguous and so

a "for x=0 to numnodes" loop doen't work - one must test the tempnode variable

(see source) - the BIOS tells you the next valid node number to use.

Fixed up some display formatting issues - piping the program's output to a

file now works as per all my other programs, I/O ranges are more accurately

presented now.