..mb 11

.he TurboDOS/PC Diskless Download ROM Configuration

.fo Copyright 1985 (C) Intercontinental Micro Systems, Inc.

The following information is a guide for configuration of your

system to use the Intercontinental Micro Systems diskless

download ROM.

The diskless download ROM allows you to configure a PC or PC-

compatible system so that DOS is loaded from the TurboDOS file

server instead of a local drive attached to the PC.  This

procedure is dependent on the versions of DOS that exist, so the

standard IBM PC-DOS is used here for the purpose of discussion.

OVERVIEW:

--------

The diskless download ROM is used in the ICM LAN-PC controller

for the purpose of downloading DOS from a ICM TurboLAN file

server.  This allows the PC to be free of local storage drives

and become a lower cost "PC workstation" in the TurboLAN network.

The ROM is located on the LAN-PC controller in the provided ROM

socket.  This ROM socket should be configured for a 2k (2716)

type EPROM, refer to the LAN-PC hardware documentation for the

proper configurations on the board.

The diskless download ROM requires that the LAN-PC be configured

for the IBM PC default memory and I/O addresses.

LAN memory segment: E000 hex

LAN I/O address:    200  hex

The diskless download ROM links itself into the bootstrap

interrupt vector (INT-19) during initialization.  This will

require compatibles to execute an INT-19 instruction to begin the

bootstrap procedure as in the IBM PC.  DOS is loaded through the

use of a psuedo file "PCIMAGE.DSK" located on your file server

and a simulation of the PC's disk I/O interrupt vector (INT-13).

Once the image file is opened, the first sector is loaded at the

normal boot sector location and control transfer to it.  The

bootstrap continues from this point as if it were loading from a

local floppy disk.  A Utility program "BOOTFILE.COM" is provided

to create the psuedo file.

A special version of the TurboDOS/PC driver must be configured

that defines all drives to be remote.  It is recomended that all

16 drives be mapped remote on your file server allowing changes

in the file server to be immediatly recognized by your diskless

workstation.  Once executed from your psuedo file, TurboDOS/PC

gives your workstation access to remote drives, printers, and

queues, thus completing the download procedure.

.cp 6PSUEDO FILE CREATION:

--------------------

Creation of the psuedo file requires a PC (with at least one

local floppy disk drive) be supported on your TurboLAN network.

1.   Prepare a floppy disk for bootstrap operations (preferably a

fresh disk with the use of the formatter supplied with your

system.  Copy your configuration file "CONFIG.SYS" along

with any additional drivers needed for your PC.  Copy the

cold start components of your TurboDOS/PC system, usually

"TURBOPC.COM" and "TURBOPC.DRV".  An "AUTOEXEC.BAT" file can

be used also but this imposses curtain limitations on the

downloading device in your network (see below).

2.   Use the utility program "BOOTFILE.COM" to transfer the image

of the boot disk into the psuedo file on your file server

"PCIMAGE.DSK".  For example:

BOOTFILE A: E:PCIMAGE.DSK

This could transfer the image of the local PC-DOS boot disk

in drive A to the psuedo image file on remote drive E.  The

psuedo image file must be located in user area 0 on the file

server.  This most often is the root directory of remote

drives.

.cp 6

DESTINATION CIRCUIT/NODE CONFIGURATION:

--------------------------------------

The circuit and node ID for the file server is configured with

the use of the 8-position switch located on the LAN controller.

Positions 1-2 of this switch denote the circuit number (one of 4)

and positions 3-8 denote the node number (one of 63, node 0 is

invalid).

.cp 6

DOWNLOAD ROM PROCEDURE:

----------------------

The following steps outline the diskless download procedure.

1.   The diskless download ROM is given control during the ROM

BIOS initialization phase, where it will alter the bootstrap

interrupt vector (INT-19) in order to gain control later,

and return control to the ROM BIOS.

2.   When the ROM BIOS has completed its initialization phase, it

executes the bootstrap interrupt, where the diskless

download ROM regains control.  Once at this point, the

download ROM displays its sign-on message as follows:

TurboDOS/PC Network Download ROM

Copyright 1985, Intercontinental Micro Systems, Inc.

3.   The download ROM will now initialize the LAN-PC controller

and read its own node ID out of its memory buffer.  The

sense switch is also read to determine the circuit and node

ID of the file server.  If errors are determined during

initialization, the download ROM displays the following

error message:

LAN-PC Controller not present

4.   Network requests are formed and sent to the file server to

attemp an open of the psuedo file.  Open file requests are

attemped on devices A through P (user area 0) of the file

server before the download ROM gives up and displays the

following message:

Can't open download file

5.   Once the psuedo file is opened, the download ROM alters the

disk I/O vector (INT-13) to point to its own simulation of

disk I/O.  The disk I/O vector is used from here on to read

sectors within the psuedo file.  Sector 2 (first FAT sector)

is then read into memory and the format of the psuedo image

file is determined (i.e. single or double sided, 8 or 9

sector disk).  This allows any of these formats to be used

when the local DOS boot disk is prepared.  If an error

occurs when reading this sector, the download ROM displays

the following message:

Can't read FAT sector

6.   Sector 1 (DOS boot sector) is then read into the normal

memory location 0000:7C00 and control is transfered to it.

On errors, the download ROM will display the following

message:

Can't read boot sector

7.   Loading of the DOS now proceeds in the normal fashion except

that all physical disk I/O operations are translated to

random record reads of the psuedo file and made over the

network.  Errors that occur over the network are reported by

the following messages:

Network send error

Network receive error

Diskless download errors are all considered fatal by nature.

After reporting errors on the console, the download ROM displays

the following message:

Download aborted, processor halted

The PC must be reset to perform any retries.  The CTRL-ALT-DEL

sequence will accomplish this on most PC's.

.cp 6

DOWNLOAD ROM CREATION:

---------------------

The diskless download ROM has been configured for standard IBM PC

operation.  Most PC clones and compatibles should operate with

this configuration without the need for alterations.  If changes

must be made to the standard configuration, the following steps

may be used to produce the ROM.

The object code for the diskless download ROM is distributed in

the file "TPCLOD.O".  A "GEN" and "PAR" file exist for to link

the object file and provide examples of the patchable options.

Refer to "TPCLOD.PAR" for current information on patch options.

The following is the list of utilities and the steps required to

produce the ROM native code.

TLINK.CMD      - Object linker

MONITOR.COM    - S/W 2000 Z80 monitor (other equivalent)

CHECKSUM.CMD   - ROM checksum utility

TZ80.CMD       - Z80 emulator (if Z80 machine unavailable)

1.   Link the relocatable object file:

TLINK TPCLOD.ROM

* TPCLOD

2.   Strip off the .CMD header (first 128 bytes of file):

MONITOR        - or (TZ80 MONITOR) for 8086 processors

*LTPCLOD.ROM

100-97F

*STPCLOD.ROM 180-97F

180-97F

3.   Add ROM checksum byte to end of file:

CHECKSUM TPCLOD.ROM

.cp 6

EFFECTS OF DISKLESS DOWNLOADING

-------------------------------

The following section explains certain characteristics of

diskless ROM downloading that you may encounter when configuring

your system.

An "AUTOEXEC.BAT" file can be created and placed into the psuedo

file to execute the startup sequence desired with your PC, but it

should be realized that when TURBOPC.COM is executed, device A

will usually be re-defined as a TurboDOS remote drive, where the

continuation of the file cannot be found.  If drive A (remote)

exists, and is ready for operation, the batch procedure will

terminate normally and the drive prompt will appear.  However, if

drive A is not ready, DOS will fall into a continuous loop trying

to access drive A to finish the batch job.

An "AUTOEXEC.BAT" file will work best if drive A (remote) is the

drive containing the psuedo file "PCIMAGE.DSK" which downloaded

your PC.  In fact, if an identical copy of this batch file exists

on drive A (remote) under the user area defined as the root

(usually user area 0), then the batch job will continue normally

even after TURBOPC.COM executes and terminates.

A situation will arise when DOS 3.xx and some versions of 2.1x

are used.  These versions require execution of TURBOPC3.COM

directly after execution of TURBOPC.COM.  If a batch file is used

with this case, DOS becomes disabled after TURBOPC.COM executes

and terminates because program loads from remote drives will

require TURBOPC3.COM.  This ambiguity can only be solved by

configuring your TURBOPC driver and retaining drive A as local

(the psuedo file).  TURBOPC3.COM is then executed from the psuedo

file.  The supplied LOCAL.COM utility should be executed last

during startup, to avoid network conflicts that arise when the

psuedo file is referenced later.  The "AUTOEXEC.BAT" file can be

used throughout startup but CANNOT execute the LOCAL.COM utility.

This must be done manually.

#e to end of file:

CHECKSUM TPCLOD.ROM

.cp 6

EFFECTS OF DISKLESS DOWNLOADING

--------------