ADQ(TM) Version 1.01 Shareware/commercial release

1.0 Introduction

1.1 History

2.0 System requirements

2.1 OS independence

2.2 Running ADQ from multitasking operating systems

3.0 Getting started

3.1 Command line options

1.0 Introduction:

This file contains information regarding ADQ version 1.01. Make sure you

have read terms.doc before using ADQ.

1.1 History

Version 1.01      Optimized the drive detection algorithm

Fixed a bug in the 'cross-read supported' test

Fixed a bug which made ADQ "hang" in the test section

with some BIOSs

2.0 System requirements:

* DOS version 3.3 or higher

* A 286+ processor

* An ATA/IDE drive + controller

* 200 KB of free memory

2.1 OS independence:

ADQ does not test OS dependent information, it can be used on every harddisk

regardless of the operating system (OS) when placed on a bootable DOS disk.

There is no need to enter the drive to test in CMOS setup.

2.2 Running ADQ from multitasking operating systems:

ADQ runs under DOS so theoretically it can be used under every multitasking

OSs that support a DOS box. Since ADQ accesses the drive using register

I/O it's not recommended to do this, since it may result in corrupted data.

ADQ has a build in detection which has been tested under Windows 3.11,

Windows 95, Windows NT version 3.51 and OS/2 which prevents running it under

a multitasking environment. This detection can be overruled by using

the command line parameter "nowindows", however as stated earlier this is not

recommended. The best place for ADQ is on a bootable disk.

3.0 Getting started:

ADQ is an easy to use program, it supports the keyboard and the mouse

The program has it's own help system, this document will only

explain the command line parameters.

3.1 Command line options

Overview of command line options:

floppy            Keep the engine running at the help system (default no)

This option will significantly speedup browsing the help

system when running ADQ from floppy disk.

timeout:xxx       Set the time-out to xxx ms (default 5000ms)

This value will define the maximum time to wait after

issuing a command.

force             Report all drives present.

Use this option when the build in detection failed.

register          Register this copy (needs key-code)

This will change the shareware version in a commercial one.

nowindows         Disable detection of multitasking environments. Use the

option nowindows only if you are sure the detection failed.