CCB-100 Documentation File                   01-23-85

This  file  explaines the CCB-100 software modules  found  on

this  disk and their functions.  Software for the  CCB-100  clock

calendar  board  may be run on either the CPZ-4800x,  the  CPS-MX

slave CPU cards,  the CPZ-186 16 bit master, or the CPS-16 16 bit

slave parallel ports. Under TurboDOS operation, the fact that the

CCB-100  is  on  the any of these cards makes  no  differance  to

TurboDOS as the O.S.  will find out where the CCB-100 is and  use

it to set TurboDOS's internal Date/Time function.

Below  is an explanation of the files needed to  support  the

CCB-100.  Necessary source files and in some cases, the COM files

are provided on this disk.

CLKSETM  - This driver allows the setting of the CCB-100 from the

CPZ-4800x CPU card for proper Date/Time.

CLKSETS - This driver allows the setting of the CCB-100 from  the

CPS-MX  Slave processor card for proper  Date/Time.

If  either  CLKSETM or CLKSETS does not find a clock  on  that

particular processor,  an error message will be generated to that

affect.

CPZEQU.x - Support library for the MASTER I/O ports  and  ascii

definations.

CPSEQU.x -  Support library for the SLAVE I/O ports.

DREQUATE - Ascii support library.

DSPCLKM  - Master Version. (CPZ-4800x or CPZ-186)

Stand alone module to read the Date/Time directly from

the  CCB-100 clock calendar board.  Useful under  CP/M

and in some cases under TurboDOS. Command options:

DSPCLKM<cr> will display the Date/Time on console.

DSPCLKM R<cr> will display the Date/Time until any key

is pushed to abort.

If CCB-100 is not present,  and error message will  be

generated to that affect.

DSPCLKS  - Slave Version (CPS-MX or CPS-16)

Stand alone module to read the Date/Time directly from

the  CCB-100 clock calendar board. Command options:

DSPCLKS<cr> will display the Date/Time on console.

DSPCLKS R<cr> will display the Date/Time until any key

is pushed to abort.

If CCB-100 is not present,  and error message will  be

generated to that affect.

MSTRCLK  -  Driver module to include into OSMASTER.SYS  generated

module  for TurboDOS.  Can be included in either  the

Single User or the Multi-User version of OSMASTER.SYS

This   TurboDOS  driver  module  sets  the   Internal

Date/Time of TurboDOS. NOTE!!! only for 8-Bit Master

SLVCLK  -   This module is used to set TurboDOS Date/Time from  a

Slave  processor running under  Multi-User  TurboDOS.

This module is NOT included into the Slave GEN file as

it  is  run as an AUTOLOAD file after the  Slave  has

come  up.  The SLVCLK.MAC file is assembled,  Linked,

and then renamed to SLVCLK.AUT and placed into USER 0

of the boot drive.

#########################################