IN case of a need to use the Bigboard 2 for a mixed configuration,

(5 1/4 & 8 inch) the following changes must be implemented.

1.	add a 74LS157 on the Bigboard 2 experiment area.

as follows:

___ ___

2MC osc   JB35-3    ______2|1a " 1y|4______ 1MC/2MC clock   JB35-2

1Mc osc   JB35-1    ______3|1b     |

Ready/    JB4-3     ______5|2a   2y|7______ Ready/          JB4-2

Gnd	  JB4-1     ______6|2b     |

Wd(U28-11)JB8-3     _____11|3a   3y|9______ WD              JB8-2

Wd(U10-31)JB8-1     _____10|3b	   |

|	   |

Select    JB5-1     ______1|sel    |

|       |

|74LS157|

|_stb___|

|15

Gnd       __________|

pin 16    Vcc

pin 8     gnd

pin's  12,13,14  not used (yet)

2.	Make use of a modified Bigboard monitor rom.

This one is used to initialize a location in the monitor

ram to start with the floppy drives as indicated by the

dip switch on the Bigboard 2.

This location may be altered after boot by the "config"

program.

Rom monitor modification:

Make some room (if needed) by deleting (for example "crcgen").

In the module "init"  add the following:

at the point where the disk parameters are set up

;

;    set disk parameters for 5 or 8 inch drives.

;

in   a,(sense)      ;read dip switch

bit  5,a            ;test drive type

jr   z,skip2        ;and jump if 8 inch drives

ld   a,00000011b

l䠠   (dsktyp),�  ;se� dis� typ� for min� drives

;next entry's are added�

xor  a              ;indicate all drives are 5 1/4 inch

ld   (typeflag),a   ;this is new

ld   a,8            ;set mux (74LS157) to 5 1/4 inch

out  (od9h),a

skip2:

call flipdens       ;this is existing code

;

;    initialize the z-80 for interrupt mode #2

This is all to be changed in the coding.

Next changes must me made in the following files:

memory.mac

memdef.mac

at the end of the ram definitions add the following:

typeflag: defb 00001111b      ;default set to 8 inch

speed8:�  defb 5�             ;8 inch seek rate in miliseconds

speed5:   defb 3              ;5 1/4 inch seek rate in miliseconds

Both speed bytes may be modified by the "config" program

and are used by "autobios" when selecting a drive.

Assemble the new monitor source, and make a new rom (2732)

by using the provided "program.com" transient program.

In order to burn a 2732 eprom,

Enter: "program rom(cr)."

Where "rom" is the name of the "com" file to burn in (max 4k)

3.	Assemble the file "autobios" for a mixed configuration

setup.

This is done by specifying:

in "system variables"

EIGHT	EQU	FALSE

FIVE	EQU	FALSE

MIXED	EQU	TRUE

The output may then be mixed with a 59K cpm version 2.2

or smaller if you like.

The new created bios is to big for a 60K cpm version.

Restrictions.

All drive selections should be made trough the bios module.�

The following programs are known not to do so:

BB2FORMT.COM

INIT5.COM

In absence of the source code these programs are not modified.

These programs must be executed from the type of drive the program

is written for.

I.E.

Drive "a" = 5 1/4 inch (with your programs bb2formt, init5 etc.)

Drive "c" = 8 inch

And you want to initialize an 8 inch floppy in drive "c".

First insert an initialized 8 inch floppy in drive "c",

Then log it in.

Next replace the floppy in "c" by a floppy you want to initialize.

Then type:

C>A:BB2FORMT(CR)

And follow the format menu instructions.

For 5 inch drives I wrote a format program that will execute

from either type of drive.

Thi� wil� b� calle� 'format40.com�� o� 'format80.com'�

depending on a 40 tracks or 80 tracks drive.

You also cannot do an direct sysgen from an 5 1/4 inch drive to

an 8 inch drive.

This is due to the fact that the memory layout for the ccp and

bdos modules are different.

(see bios.doc)

You can overcome this problem by reading in either format by the

corresponding sysgen program, save it as a file, read it back

using ddt or zid, move the ccp and bdos to the right boundary

and start the right sysgen to write it back.

Good luck.

L.Koopman

Waarbekenplein 13

7553 KK Hengelo (o)

Tel. 074-913022�	Nederland

##########