Purpose:

CACHE establishes disk caches. The default cache is for

all PC, XT, and AT real (not virtual) drives other than the AT hard

disk.

Format:  [d:][path]CACHE [[size] disk]

Remarks: Specify the parameters:

[d:][path] before CACHE to specify the drive and the path that

contains the CACHE command file.

[size]     Size of the cache in 512 byte sectors. This number must be

between 24 and 124, or it will be adjusted. The default is 64.

The storage occupied by the cache is less than 256 bytes plus

518 bytes per sector.

[disk]    Disk to be cached, if a single disk cache is being allocated.

The default is to cache all real disks, but there may be occasions

where it is desirable to avoid cacheing a specific disk, or a

cache larger than 62K bytes is desired.

CACHE can be called more than once to create a larger cache or

set individual disk caches for each disk. The caches are searched in

reverse order of allocation. If you are allocating more than one cache

allocate the most heavily used cache LAST. If you are allocating

a general cache, it makes sense to allocate it FIRST in most cases,

so it becoems a "cache of last resort". The overhead for skipping

individual caches is small, but the overhead for a search failure is

large.

Only single sector reads and writes are cached. Therefore, loading

large programs will not be affected. What will be affected are

databases with records smaller than 512 bytes, batch files, and

source files for language processors that scan the input file

more than once. The disk file directory and file allocation tables

are cached, and always resident once read from disk. Other files

have their sectors purged if they are not frequenly accessed.

When the cache is allocated, its segment location is listed out.

The DOS DEBUG command can be used to read out data areas of interest,

while the cache is running. The data areas start at offset 114h.

They are each one word (two bytes) long:

offset           data

114      Cache size in sectors

116      Total Number of disk I/O's

118      Number of cache hits

11A      Number of free slots in cache

Remember that these numbers are hexadecimal and stored low order byte

first, so the default size of 64 is stored as:

40 00

To access this data, first write down the sector address typed

out when CACHE initializes. Let's say it is 909. Type the following

commands:

DEBUG

d 909:114

(analyze the displayed data)

q

Typically, the about half of all disk I/O's will result in cache hits.

All data in cache is backed up on disk immediately, so only disk reads

are assisted by cache. The cache should be sized so that you will not

reduce the number of free slots to zero, the first time you perform

your most frequent operation. Size your cache by booting your

computer, issuing the CACHE command for a 124 sector buffer,

performing your "typical" batch file, compile or data base access,

running DEBUG, determining how much CACHE remains free, and downsizing

as required. If you can't allocate a single large cache that does the

job, then create caches for heavily used disks.

Place the CACHE commands first in your AUTOEXEC.BAT file because CACHE

will make it run faster. Also notice that DIR commands will run

without pausing, and the disk will stop running about halfway through

the second time it is issued.

From: Quick and Dirty Software, Inc.

Origional logic by Steven Holzer of PC magazine (8/85)