ZIP.DOC

-------

Instructions for ZIP.COM

IBM PC Fast Serial File Transfer Utility

Version 1.2 (05/88)

(c)1988 E. Meyer

Requires:  100% IBM PC compatible computers;  MS/PCDOS 2.x or 3.x

ZIP is a small (2K!), fast utility to transfer files between two IBM

compatible computers over an ordinary serial cable connection.  This is most

useful when the computers don't share a common disk format, as in the case of

a desktop PC with 5" drives and a laptop (or PS/2) with only a 3" drive.  ZIP

is easier to use, and over 6 times faster, than any ordinary telcom program.

Telcom programs: modem       2400 bps      0.3 K/sec

Telcom programs: cable      19200 bps      2.3 K/sec

--->  ZIP: cable                 115200 bps     14.1 K/sec

Most computer manufacturers offer additional disk drives or other

hardware accessories to provide access to data on both 3" and 5" disks; but

these can cost hundreds of dollars.  They can also make a laptop computer a

lot less than portable.  Direct serial connection isn't quite as fast, but it

does allow a data rate of 115,200 baud -- that's over 14K per second.

There are commercial programs for this purpose (of which LapLink may be

the best known).  I'm sure they work very well, and have many extra

conveniences that ZIP lacks.  But even they are priced around $129, which is

far more than I was willing to pay.  ZIP transfers files just as well, and is

free.

BASIC USE

Obviously, you will need a copy of ZIP.COM on both computers.  (You can

accomplish this using your serial cable, if both machines have telcom software

with XMODEM protocol transfers.)

ZIP makes transferring a file as easy as copying it from one disk to

another.  To transfer any MSDOS file, once the cable is properly connected to

the COM1 ports, from any DOS prompt:

(1) On the sending machine, type:        C>zip FILENAME

(2) On the receiving machine, type:      C>zip

or  C>zip DIRECTORY

(Don't try to hit RETURN at exactly the same time on both ends; this actually

makes an initial handshaking error more likely.)

"FILENAME" can be a single file, or a GROUP specified by wildcards

and/or commas (see examples below).  Each name may include a directory.  Each

file will appear with the same name on the receiving end.  During the transfer

you should see a blinking asterisk.  When you see the message "Verified", you

know that the received copy exactly matches the original.

"DIRECTORY" may optionally be specified on the receiving end; otherwise,

files are received in the current directory.  Note: the directory MUST end

with either a colon ":" or a backslash "\", otherwise ZIP will think it's a

FILE to SEND!  See examples below.

ZIP can be aborted by pressing any key:

(1) on either end, during initial handshaking

(2) on sending end, during file transfer

(will abort once current file is finished)

SELECTING THE SERIAL PORT

Most IBM computers have anywhere from two to four serial (RS232) ports.

The first two, COM1 and COM2, are implemented at standard addresses; COM3 and

COM4, if present, can vary from one computer to another.  To use them you will

have to install the appropriate address in ZIP.COM (see below).

ZIP has two command line options that you can specify (once, after any

filename(s) present) on either end, to determine which of the two standard

serial ports is used:

/1   = Use the COM1 port.  (This is normally the default.)

/2   = Use the COM2 port.

USAGE EXAMPLES

C>zip \work\my         sends file C:\WORK\MY over COM1

C>zip gort,klaatu      sends files C:GORT, C:KLAATU over COM1

C>zip my.* /2          sends all files C:MY.* over COM2

C>zip                  receives files over COM1 into current directory

C>zip \work\           receives files over COM1 into directory C:\WORK\

C>zip d: /2            receives files over COM2 into directory D:

TECHNICAL DETAILS

ZIP does full CRC (Cyclic Redundancy Checksum) verification on the

received file.  Being designed for direct serial transfer at maximum hardware

speed, ZIP is not compatible with other software, including ordinary telcom

programs.  Do not disturb the cable when ZIP is running, as this may cause the

program to stall, requiring a reboot of your computer(s).

Be sure to use the SAME version of ZIP on both ends: the transfer

protocol may have changed.  Otherwise, if you can't get ZIP to transfer files,

your computer(s) or your cable are incompatible.  ZIP is written entirely in

assembly language; it will only work on fully IBM compatible computers.  I

have tested it myself on 8088 PC clones with clock speeds of 4.77 to 8 MHz; I

rely on users' reports that it works well on faster systems.

Serial cables can be wired in many different ways.  The one I use was

labeled as an "IBM AT adapter", and had female connectors on both ends (one

DB9, one DB25).  Any cable that doesn't work by itself should with a null

modem adapter.  If you find that you need to build a cable, don't ask me for

advice: I don't know any more about this than you do!

MODIFYING THE DEFAULT SERIAL PORT

If no command-line option is given, ZIP will use its built-in "default"

serial port.  This value appears as a hexadecimal port address in the sign-on

message at the start of ZIP.COM; when you first run ZIP, you should see:

ZIP 1.2 (5/88) (c)1988 E. Meyer   [3F8]

The "3F8" is the standard address for COM1; "2F8" would indicate COM2.  For

the addresses needed for COM3 or COM4, if present, see your computer manual.

The default value in ZIP.COM can be modified with a patching utility like

DEBUG.  Just change the address appearing in the signon message to whatever

you require.  First, this lets you change the default to COM2 if you wish;

just change the "3F8" to "2F8".  Second, this lets you use your COM3 or COM4

port, by making it the default.  After modifying the default, you can still

use the command-line options above to select COM1 or COM2 instead.

Here is how the DEBUG session might look; we Display the signon message,

Enter a "2" in place of the "3" to change to a default of COM2, Display again

to see the new value, Write the change to the file, and then Quit:

C:\WORK>debug zip.com

-d100 12f

xxxx:0100  E8 1F 06 0D 0A 5A 49 50-20 31 2E 32 20 28 35 2F   .....ZIP 1.2 (5/

xxxx:0110  38 38 29 20 28 63 29 31-39 38 38 20 45 2E 4D 65   88) (c)1988 E.Me

xxxx:0120  79 65 72 20 20 20 5B 33-46 38 5D 00 E8 EB 06 8B   yer   [3F8].....

-e127 32

-d100 12f

xxxx:0100  E8 1F 06 0D 0A 5A 49 50-20 31 2E 32 20 28 35 2F   .....ZIP 1.2 (5/

xxxx:0110  38 38 29 20 28 63 29 31-39 38 38 20 45 2E 4D 65   88) (c)1988 E.Me

xxxx:0120  79 65 72 20 20 20 5B 32-46 38 5D 00 E8 EB 06 8B   yer   [2F8].....

-w

Writing 07C2 bytes

-q

For further information on DEBUG see your MSDOS manual.  Please DO NOT

distribute modified copies of ZIP.COM; this could confuse other users.  They

are for your personal use only.

ZIP 1.2 TRANSFER PROTOCOL

This information is provided for programmers interested in writing

applications compatible with ZIP on other computer systems.  The version 1.2

protocol may need to be modified with future development of ZIP.  All transfer

takes place at 115200 bps; be aware that very tight polling loops, minimal

handshaking, and fixed packet size are needed to avoid loss of data.

The initial handshaking involves the receiver sending a version code byte

(120 decimal, for 1.20) at intervals until it is acknowledged (ACK, 6 decimal)

by the sender.  From this point on all data transfer is in fixed-size packets.

The receiver, when ready, requests each packet by sending an ENQ (5

decimal), and acknowledges correct receipt with an ACK.  Each packet has a 3-

byte header, consisting of an SOH (1 decimal) and a 16-bit count to indicate

the number of significant bytes in the packet; then 512 bytes of data; then a

concluding ETX (3 decimal).

The first packet contains the file name; an empty name packet signifies

the end of the transmission.  Subsequent packets contain the file data; a

packet that is less than full (count less than 512) signifies the end of the

file.  One final packet contains the 16-bit CCITT CRC value for the entire

file, the DOS time stamp word, and the DOS date stamp word, in that order.

The receiver looks for the SOH and ETX bounding each packet, but does no

further checking.  There are no timeouts.  No attempt is made to resend

packets; under normal circumstances, transmission errors should not occur.

Upon any serious error, a NAK (21 decimal) is sent to the other end before

exiting to DOS.  Upon receipt of this signal, the other end also terminates.

ERROR MESSAGES

<ERROR>  invalid command (probably illegal option).

<OUT OF MEMORY>  less than 64K free RAM, or too many files.

<FILE ERROR>  trouble reading or writing a disk file.

<COMM FAILURE>  possible cable or compatibility trouble;

or, error occurred at the other end.

<CRC ERROR>  received file CRC did not match; transmission error.

<ABORTED>  you pressed a key.

HISTORY

ZIP 1.0 (04/88) - trial release.  Worked well on many systems.

1.1 (04/88) - improved transmission algorithm; batch transfers allowed

using wildcards and/or commas; receive directory option;

original file time/date preserved.

1.2 (05/88) - improved error handling; patchable default port also

allows use of COM3,4; small bug fixes.

ZIP and its documentation are (c)1988 E. Meyer, all rights reserved.

They may be freely distributed, but not modified or sold for profit without

my written consent.  The user takes full responsibility for any damages

resulting from the use (or misuse) of this program.  Please report any

problems encountered.

Eric Meyer

427 N. Washington #4             CompuServe [74415,1305]

Bloomington, IN  47401