[73307,2453]

STROKE.ARC

Keywords: KEYBOARD INTERRUPT ASSEMBLER C LANGUAGE TOOLS

With this package, (and an assembler and a "C" compiler), you can

perform keyboard interrupt handling functions in the "C"

language. No assembler coding is required -- everything you need

can be written in "C". Works on IBM PC/XT, AT&T 6300, and Compaq.

Compiled using AZTEC C. Source included, which can be easily

modified to use MS or other C.  Author can be reached here or via

EMAIL.  See also TICK.ARC for timer interrupt handling in "C".

Graeme McRae [73307,2453]

In this package, you will find:

stroke.doc   Explanation of this package

stroke.asm   Source code for the program that installs the

interrupt handler

strokeg.c    Source code for a C program that uses this facility

strokeg.exe  Compiled program demonstrating this facility

cli.asm      Source code for a trivial little routine

brkkey.asm   Source code for break key interrupt processing

Calling Sequence to install keyboard interrupt

#define STACKSIZ 512

int stack[STACKSIZ];

int stroke();

int myrtn(scan_code) int scan_code; {

}

stroke(1,myrtn,&stack[STACKSIZ-1]);

These statements will install the function myrtn as the keyboard

interrupt.  At each interrupt, myrtn will be called, passing it

the scan code generated by the keyboard.

Calling Sequence to remove keyboard interrupt

stroke(0,myrtn,&stack[STACKSIZ-1]);

You *must* remove the keyboard interrupt prior to leaving your

program.  If you don't, the next keystroke will pass control to a

function that no longer exists, and your system will go down the

tubes.  This means you have to either disable or trap the break

key.  The example program, strokeg, illustrates this point.

Explanation of brkkey()

Calling sequence

brkkey(brkrtn);

brkkeyl(brkrtn);

int brkrtn();

Brkrtn identifies an interrupt routine to be invoked when the

break key is pressed.  It automatically goes away when the

program ends.  It should set flags that will be tested by the

main program, and then the main program should process the

"break" action as appropriate.