DigiBoard Port Authority

Introduction

The DigiBoard Port Authority (DPA) program is a software tool which provides

a means to monitor the status of the Front End Processor/Operating System

(FEP/OS), which is the on-board software run by DigiBoard intelligent serial

communications products.

The DPA software also shows the status of the individual ports on a concen-

trator or PORTS module by displaying a simulated concentrator front panel.

Each of the eight supported RS-232 signals is displayed, along with input and

output flow control status and the status of the host adapter to concentrator

connection status.

The DPA program can be run from any terminal on the system, and supports the

following DigiBoard hardware and device drivers:

Hardware:	DigiBoard C/X, EPC/X and Xem systems, Acceleport, and PC/Xx products

Software:	UNIX System V, Release 3 & 4 device drivers version 2.1 and up,

Solaris x86 & Solaris Sparc device drivers, version 1.0 and up

The DPA software is installed automatically when you install this device driver.

Using the DPA Software

To run the DigiBoard Port Authority software, enter the following command from

any terminal, or the system console: dpa [-f logfile]

The -f logfile option specifies the file path for screen dumps. If this option

is not specified, the default log file path is /tmp/dpalog.

The DPA Main Screen

After reading the initial information screen, press any key to see the DPA main

screen, which will be similar to the following:

+------------------------------------------------------------------------------+

|                     Digi Port Authority  version 2.3.16                      |

|                 Copyright (c) 1994, Digi International Inc.                  |

+------------------------------------------------------------------------------+

+----------------------------------------------------------------------------+

|         Type       State      I/O Addr     Mem Addr   Mem Size     #Ports  |

|----------------------------------------------------------------------------|

|  1     ISA - PCXE    OK           220        C0000        8K          8    |

|  2    EISA - C/X     OK          6005        C0000       32K         48    |

|  3     ISA - Xem     OK           204        C0000       32K         41    |

|                                                                            |

|                                                                            |

|                                                                            |

|                                                                            |

|                                                                            |

|                                                                            |

|                                                                            |

|                                                                            |

|                                                                            |

+----------------------------------------------------------------------------+

'?'=Help     <ESC>=Quit     <Return>=Accept     j/k=Move     ^P=Print Screen

The fields in the above screen have the following meanings:

Type:	The bus type (EISA, ISA, MC) and board type (C/X, Xem, EPC/X)

detected by the DigiBoard device driver.

State:	The current state of the board.  Possible values are:

OK      	Indicates the adapter is functioning normally.

NOT FOUND	The device driver could not identify this type of board.

NO IO		The device driver was unable to communicate with

the board via the I/O port. Switch settings could be incorrect.

NO MEMORY	The device driver was unable to allocate the necessary memory

for this board.

NO BIOS		Unable to download the BIOS image. Possible hardware failure.

NO FEP/OS	Unable to download the FEP/OS image. Possible hardware failure.

HD/W FAIL	An unrecoverable error with the FEP/OS has been detected.

Possible hardware failure.

NO PORTS	(Xem system only)  The BIOS on the Xem host adapter could not

detect any ports.  Possible hardware failure.

I/O Addr:	The adapter's I/O port address (hexadecimal).

Mem Addr:	The starting address of the adapter's dual ported memory

(hexadecimal).

Mem Size:	The size of the adapter's dual ported memory.

#Ports		The number of ports for which the device driver has been

configured (this may be different than the actual number

of ports physically attached).

To get more info about one of the displayed adapters, use the up and down arrow

keys to move the highlighted bar to the desired board, then press space bar.

NOTE: If the arrow keys do not map correctly, the DPA program will display the

following message: Invalid Key. (Key Number) If this occurs, substitute the

standard UNIX cursor movement keys for the up and down arrows: Down = J or j;

Up = K or k.

Viewing the Adapter Configuration

After you press the space bar, you will see the current configuration of the

selected board.

Xem Adapters:

+------------------------------------------------------------------------------+

|                     Digi Port Authority  version 2.3.16                      |

|                 Copyright (c) 1994, Digi International Inc.                  |

+------------------------------------------------------------------------------+

+----------------------------------------------------------------------------+

|                                                                            |

|                                                                            |

|  +---------------+  +    +  +    +  +    +                                 |

|  |Adapt EBI Line |--|16em|--|16em|--|8emp|                                 |

|  +---------------+  +    +  +    +  +    +                                 |

|                                                                            |

|                                                                            |

|                                                                            |

|                                                                            |

|                                                                            |

|                                                                            |

|                                                                            |

|                                                                            |

|                                                                            |

|                                                                            |

|                                                                            |

+----------------------------------------------------------------------------+

'?'=Help    <ESC>=Quit    <Return>=Accept    h/j/k/l=Move    ^P=Print Screen

The above display shows the status of the selected Xem host adapter.  Attached

PORTS modules are identified by [16em] for PORTS/16em modules, [8em]

for PORTS/8em modules and [8emp] for PORTS/8emp modules.

C/X and EPC/X Host Adapters:

+------------------------------------------------------------------------------+

|                     Digi Port Authority  version 2.3.16                      |

|                 Copyright (c) 1994, Digi International Inc.                  |

+------------------------------------------------------------------------------+

+----------------------------------------------------------------------------+

| +-------------+   +  +  +  +  +  +  +  +                                   |

| |Adapt Line 1 |---|AC|--|AC|--|AC|--|AC|                                   |

| +-------------+   +  +  +  +  +  +  +  +                                   |

| Line 1 Config:                                                             |

|   Adapt  to Node 1 Speed = 10M, 8 wire internal clock, Direct Connection   |

|   Node 1 to Node 2 Speed = 10M, 8 wire internal clock, Direct Connection   |

|   Node 2 to Node 3 Speed = 10M, 8 wire internal clock, Direct Connection   |

|   Node 3 to Node 4 Speed = 10M, 8 wire internal clock, Direct Connection   |

|                                                                            |

| +-------------+   +  +   +  +                                              |

| |Adapt Line 2 |---|AC|---|AC|                                              |

| +-------------+   +  +   +  +                                              |

| Line 2 Config:           [32]                                              |

|   Adapt  to Node 1 Speed = 10M, 8 wire internal clock, Direct Connection   |

|   Node 1 to Node 2 Speed = 10M, 8 wire internal clock, Direct Connection   |

|                                                                            |

+----------------------------------------------------------------------------+

'?'=Help   <ESC>=Quit   <Return>=Accept   h/j/k/l=Move   'S'=Status   ^P=Print

The above display shows the status of the selected host adapter. Concentrators

are represented by [AC] if active, or [DN] if inactive (turned off, defective,

etc.). In this case, the board is an EPC/X host adapter, with four concentra-

tors connected to line 1, and two concentrators on line 2. Note the number [32]

below the second concentrator on Line 2.  This indicates that the second concen-

trator has a PORTS/16em module (or two PORTS/8em modules) attached, for a total

of 32 ports on that concentrator.

The display also shows the speed and wiring method between the host adapter and

the first concentrator (node), and between subsequent interconnected concentra-

tors. To see a C/X or EPC/X host adapter's configuration string, use the arrow

keys (or H, J, K and L keys, if your arrow keys aren't correctly mapped) to

highlight either Adapter Line box, and press the space bar.  You will see the

following display:

+------------------------------------------------------------------------------+

|                     Digi Port Authority  version 2.3.16                      |

|                 Copyright (c) 1994, Digi International Inc.                  |

+------------------------------------------------------------------------------+

+----------------------------------------------------------------------------+

|                           Adapter Configuration                            |

|----------------------------------------------------------------------------|

|                                                                            |

| Config=000e100e000e100e100eff                                              |

| Line 1 Config:                                                             |

|   Adapt  to Node 1 Speed = 10M, 8 wire internal clock, Direct Connection   |

|   Node 1 to Node 2 Speed = 10M, 8 wire internal clock, Direct Connection   |

|   Node 2 to Node 3 Speed = 10M, 8 wire internal clock, Direct Connection   |

|   Node 3 to Node 4 Speed = 10M, 8 wire internal clock, Direct Connection   |

|                                                                            |

| Line 2 Config:                                                             |

|   Adapt  to Node 1 Speed = 10M, 8 wire internal clock, Direct Connection   |

|   Node 1 to Node 2 Speed = 10M, 8 wire internal clock, Direct Connection   |

|                                                                            |

|                                                                            |

|                                                                            |

+----------------------------------------------------------------------------+

'?'=Help            <ESC>=Quit            ^P=Print Screen

The configuration string shown is what the FEP/OS has stored in memory during

system configuration.  The FEP/OS uses this string to determine line speeds and

connection types between the host adapter and concentrators, and between

interconnected concentrators.  This is useful for diagnosing configuration

problems.

Monitoring Channel Activity

The DigiBoard Port Authority program allows the user to view the current

activity of any C/X, EPC/X or Xem port in the system.  To view a channel's

activity, use the arrow keys (or H, J, K and L keys, if your arrow keys aren't

correctly mapped) to highlight the desired concentrator or PORTS module

(represented by [AC], [16em], [8em] or [8emp]), and press the space bar. The

software will display a graphic simulation of the concentrator's front panel,

as shown below:

+------------------------------------------------------------------------------+

|                     Digi Port Authority  version 2.3.16                      |

|                 Copyright (c) 1994, Digi International Inc.                  |

+------------------------------------------------------------------------------+

+----------------------------------------------------------------------------+

| Adapter Config:                                                            |

|       Line 1 - 1                                     Sync Packets TxD : |  |

|       Line 2 - 2                                     Sync Packets RxD : \  |

|                                                                            |

|                    Line 1, Concentrator 1, Channel  1                      |

|      +--------------------------------------------------------------+      |

|      |                                                              |      |

|      |                                                              |      |

|      |     TxD  RxD  RTS  CTS  DSR  DCD  DTR  RI  OFC  IFC      (AC)|      |

|      |      _    X   [ ]  [ ]  [ ]   _   [ ]   _   _    _           |      |

|      |                                                              |      |

|      +--------------------------------------------------------------+      |

|                       Signal Active = [ ]   Inactive = _                   |

|  Input Modes : BRKINT:IGNPAR:IXON:IXOFF                                    |

| Output Modes : TAB1:TAB2:TAB3                                              |

|Control Modes : 115.2 Baud:8 Char Bits:1 Stop Bits:No Parity:Fast Baud      |

+----------------------------------------------------------------------------+

'?'=Help   <ESC>=Quit   h/j/k/l=Move   'N'=Change Line   'T'=LoopBack   ^P=Print

This screen is dynamically updated at half second intervals, and shows the

current status of the channel selected.

Activity on the synchronous channel is indicated by "helicopters" after the

terms "Sync Packets TxD:" and "Sync Packets RxD:", in the upper lefthand corner

of the screen (helicopters are rotating lines, made by successively displaying

the "/", "-", "\" and "|" characters).  The box in the center of the screen

shows the state of the RS-232 signals TxD, RxD, RTS, CTS, DSR, DCD, DTR and RI,

plus input and output flow control states (IFC and OFC, respectively). An

inactive signal is indicated by a dash (-) under its name. An active signal is

indicated by a block under its name.  If data is currently being transmitted

or received, a blinking "X" will appear under the signal name (TxD or RxD,

respectively). Finally, the status of the concentrator appears at the right

-hand end of the box (AC, 16em, 8em or 8emp) if the connection to the host

adapter is good, or DN if the communication to the host adapter is down).

The display for the parallel port of a PORTS/8emp module will appear as shown:

+------------------------------------------------------------------------------+

|                     Digi Port Authority  version 2.3.16                      |

|                 Copyright (c) 1994, Digi International Inc.                  |

+------------------------------------------------------------------------------+

+----------------------------------------------------------------------------+

| Adapter Config:                                                            |

|       Number of EBI Modules = 3                                            |

|                                                                            |

|                                                                            |

|                         EBI - 3, Channel  9                                |

|      +--------------------------------------------------------------+      |

|      |                                                              |      |

|      |                                                              |      |

|      |     TxD  RxD            IP1  IP2  OP   IP3 OFC  IFC      8emp|      |

|      |      _    X             [ ]  [ ]   _   [ ]  _    _           |      |

|      |                                                              |      |

|      +--------------------------------------------------------------+      |

|                       Signal Active = [ ]   Inactive = _                   |

|  Input Modes :                                                             |

| Output Modes : XCASE:ONLCR:ONLRET                                          |

|Control Modes : 1 Stop Bits:No Parity:Fast Baud                             |

+----------------------------------------------------------------------------+

'?'=Help    <ESC>=Quit    h/j/k/l=Move    'T'=LoopBack    ^P=Print Screen

IP1, IP2 and IP3 reflect the state of Input Ports 1-3 (usually ERROR, SELECT

and FAULT, respectively); OP reflects the state of Output Port 2 (usually

SELECTIN); OFC or IFC reflect BUSY status (depending on whether the port is in

output or input mode, respectively).  An inactive signal is indicated by a dash

(-) under its name.  An active signal is indicated by a block under its name.

If data is currently being transmitted or received, a blinking "X" will appear

under the signal name (TxD or RxD, respectively).

Input, Output and Control Modes

Beneath the line status box are the input, output and control modes in effect

for that channel. These are UNIX style flags, and have the following meanings:

Input Modes        (* Not a standard UNIX flag )

IGNBRK	Ignore Break

BRKINT	Interrupt on Break

IGNPAR	Ignore parity errors

PARMRK	Mark parity errors

INPCK	Input parity check

ISTRIP	Strip input characters

ITOSS*	Toss IXANY characters

IXON	Enable start/stop output

IXANY	Restart output on any character

IXOFF	Enable start/stop input

IXONA*	Enable start/stop output Auxiliary

Output Modes        (* Not a standard UNIX flag )

XCASE*	Canonical upper/lower display

OLCUC	Map lower case to upper

ONCLR	Map NL to CR/NL

OCRNL	Map CR to NL

ONOCR	No CR output at column 0

ONLRET	NL performs CR function

OFILL	Use fill characters for delay

OFDEL	Fill character is DEL; otherwise NUL

NLDLY	NL delay is selected

CR1	Carriage Return delay type 1

CR2	Carriage Return delay type 2

CR3	Carriage Return delay type 3

TAB1	Tab delay type 1

TAB2	Tab delay type 2

TAB3	Tab delay type 3

BS1	Backspace delay

VT1	Vertical tab delay

FF1	Form feed delay

Control Modes

Baud Rate	50, 75, 110, 134, 150, 200, 300, 600, 1200, 1800, 2400,

4800, 9600, 19200, 38400.  If Fast Baud is set (see below),

baud rates of 50, 75 and 110 are translated to 57600,

75600 and 115200, respectively.  For parallel ports, the

baud rate defines the pulse width of the data strobe.

Char Bits	5, 6, 7, 8 or None.

Stop Bits	2, 1 or None.

Parity		Enabled, Odd or None.

Fast Baud*	Use fast baud rates  *Not a standard UNIX flag

From the channel monitoring screen, channels on any concentrator or PORTS

module may be monitored without returning to the previous screen. The follow-

ing single keystroke commands are available:

*, K, k

Increment concentrator number.  If the currently selected

concentrator is the last one on the synchronous line, then the

concentrator number wraps around to #1.

*, J, j

Decrement concentrator number.  If the currently selected

concentrator is the first one on the synchronous line, then the

concentrator number wraps around to the last one on the line.

*, L, l

Increment channel number.  If the currently selected channel is

the last one on the concentrator, then the channel number wraps

around to #1.  Note that an EPC/CON-16 concentrator with

attached PORTS (EBI) modules is treated as a single

concentrator with 32, 48 or 64 channels.

*, H, h

Decrement channel number.  If the currently selected channel is

#1, then the channel number wraps around to the last one on the

concentrator.  Note that an EPC/CON-16 concentrator with

attached PORTS (EBI) modules is treated as a single

concentrator with 32, 48 or 64 channels.

N

Switch between host adapter line 1 and line 2.

T

Execute loop back test on the selected channel.  The loop back

test transmits 128 bytes and attempts to read them back via the

same port.  A loop back plug is required.  See the following

section for information on the loopback test.

Print Screen

Dumps the current screen contents to the log file.  The default

log filename is /tmp/dpalog.  A different file may be

specified by using the -f logfile command line option.  If the

Print key does not map correctly, use ^P (Ctrl-P) instead.

<ESC>

Return to the previous (Host Adapter Configuration) screen.

Loop Back Test

A loop back diagnostic test can be run against any selected channel by press-

ing the T (or t) key from the channel monitoring screen. A loop back plug

(DigiBoard part number 60000401 for RJ-45, or 60000442 for DB-25) must be

installed in the channel to be tested.

The loop back test consists of five phases:

Phase 1	Verify that the channel is not currently in use. All signals must be

low. If the port is busy, a message similar to the following will appear:

**** Port is Busy :DTR:CD:DSR:RTS

Probable cause of failure:

- Port is open by some process (e.g. getty).

- Incorrect loop back connector or cable.

Phase 2	128 bytes of test data (the letter A is used) are put into the

transmit buffer. If the write operation fails, a message similar to the

following will appear:

**** Loop Back Test Failure #1

After the write operation is complete, the FEP/OS is notified that data is

available for transmission.  At this time, the 128 bytes of data is

transmitted.  If the transmission fails, the following message will appear:

**** Loop Back Test Failure #2

Probable cause of failure:

- Hardware failure.

- FEP/OS or driver failure.

Phase 3	At this point, the FEP/OS should have received 128 bytes of data. If

not, the following message will appear:

**** Loop Back Test Failure #3

**** Data Not Transferred to RX.

Probable cause of failure:

- Hardware failure.

- Loop back connector not installed, or incorrectly wired.

- FEP/OS or driver failure.

Phase 4	128 bytes of data are read from the receive buffer. If the read opera-

tion fails, the following message will appear:

**** Loop Back Test Failure #4

Probable cause of failure:

- Hardware failure.

- FEP/OS or driver failure.

Phase 5	The data that was transmitted is compared with the data that has been

read from the receive buffer. If the comparison fails, the following message

will appear:

**** Loop Back Test Failure #5

Probable cause of failure:

- Hardware failure.

- FEP/OS or driver failure.

If all test phases pass, the following message appears: Loop Back Test Passed.

After test completion, the input and output data can be viewed. Press the I

key to see the last 128 bytes of data that were put into the receive (input)

buffer.  Press the O key to see the last 128 bytes of data that were put into

the transmit (output) buffer. The display shows the hexadecimal value of each

character above the character itself. If the character is a non printing char-

acter, "|" is displayed in its place. NOTE: For security reasons, the receive

buffer contents will only be displayed if the loop back test passes.

Press the Q key to return to the channel monitoring screen.

Loop Back Plug Specification

The loopback plug for RJ-45 versions consists of a single 10-pin RJ-45 plug

wired as follows:

Pin 3 connected to pins 1 and 8 (RTS to RI and CTS);

Pin 5 connected to pin 6 (TxD to RxD);

Pin 9 connected to pins 2 and 10 (DTR to DSR and DCD).

For DB-25 versions, the loopback plug consists of a female DB-25 connector

wired as follows:

Pin 4 connected to pins 22 and 5 (RTS to RI and CTS);

Pin 2 connected to pin 3 (TxD to RxD);

Pin 20 connected to pins 6 and 8 (DTR to DSR and DCD).

Host Status (C/X and EPC/X Systems Only)

The DPA program provides several status and error counters to aid you in

monitoring activity between the host adapter and concentrators, and diagnosing

possible communication problems. Each status counter is described on the

following pages. Several of the counters increment steadily under normal

conditions.  An incrementing counter does not necessarily indicate a problem.

For example, if a concentrator is turned off, several counters will increment.

When the concentrator is turned back on, those counters will stop incrementing.

If any counters that are usually idle begin to increment steadily, perform the

recommended tests described for that counter.  Call DigiBoard Technical Support

if the counters continue to increment steadily after performing the recommended

tests. From the adapter configuration screen, press "s".  You will see the

following:

+------------------------------------------------------------------------------+

|                     Digi Port Authority  version 2.3.16                      |

|                 Copyright (c) 1994, Digi International Inc.                  |

+------------------------------------------------------------------------------+

+----------------------------------------------------------------------------+

|                                HOST STATUS                                 |

|----------------------------------------------------------------------------|

|Activity Counters: RECEIVE1  RECEIVE2  RECEIVE3  RECEIVE4 RECEIVE5  TRANSMIT|

|                     243820         0         0         0        0    243990|

|                                                                            |

|Sync Counters: ERR_UFRAM  ERR_ABSTAT   ERR_INC   ERR_CRC  ERR_DATA  ERR_NACK|

|                       0           0         0         0         0         0|

|                                                                            |

|Saturation Counters: ERR_TBUF   ERR_FIFO   ERR_ORUN   ERR_TBUSY    ERR_RITEM|

|                            0          0          0           0            0|

|                                                                            |

|FEP/OS: ERR_REC0  ERR_ADDR  ERR_MTYPE  ERR_MSIZE  ERR_RID  ERR_TID  ERR_PING|

|               0         0          0          0        0        0         0|

|                                                                            |

|Hardware Counters: ERR_ABREC    ERR_URUN    ERR_CONC    ERR_CHECK           |

|                           0           0           0            0           |

+----------------------------------------------------------------------------+

'?'=Help       <ESC>=Quit       'Z'=Zero Counters       ^P=Print Screen

The counters are reset during system initialization (boot-up).

Activity Counters

RECEIVE1

The number of times the host adapter processed a request from a concentrator,

and one packet was received. Under idle conditions, RECEIVE1 will increment,

representing the "heartbeat" of the concentrators communicating with the host.

RECEIVE2

through

RECEIVE4

The number of times the host adapter processed a request from a concentrator,

and two, three or four packets were received.

RECEIVE5

The host adapter is 100% busy if RECEIVE5 is incrementing. Users may

experience slower response time. Monitor EC (Error Count) on the concentrator

front panel. The DigiBoard product in use is being maximized. Consider adding

another host adapter, or advance to the EPC/X system if you are currently

using the standard C/X product.

TRANSMIT

The number of data packets transmitted from the host. Under idle conditions,

the TRANSMIT counter will increment. This is the heartbeat of the host checking

the status of communications to the concentrators. Under normal operating

conditions, the TRANSMIT counter will increment.

Sync Counters

ERR_UFRAM

The host adapter received an incomplete packet. Check the physical hardware

configuration to ensure a stable environment. Check for other incrementing

counters.

ERR_ABSTAT

The host adapter received a bad data packet, and discarded it. Check for

concentrator "down" [DN] status (see the previous screen). Check the physical

hardware configuration. If the ERR_ABSTAT counter is constantly incrementing,

run the concentrator diagnostics (see the concentrator Installation Guide).

Call DigiBoard Technical Support.

ERR_INC

The host adapter received an incomplete data packet. Check the physical hard-

ware configuration. If this counter is constantly incrementing, call DigiBoard

Technical Support.

ERR_CRC

The host adapter received a packet with a CRC error. Run the concentrator sync

line diagnostic tests (see the concentrator Installation Guide). If this

counter is constantly incrementing, call DigiBoard Technical Support.

ERR_DATA

The host adapter received a packet with an error found in the data. This will

cause other counters to increment. Check sync line connections. Run the concen-

trator sync line diagnostic tests (see the concentrator Installation Guide).

If this counter is constantly incrementing, call DigiBoard Technical Support.

ERR_NACK

The host adapter received a negative acknowledgement from the concentrator for

one of its messages. Check sync line connections. Run C/X host adapter diag-

nostics. If this counter is constantly incrementing, call DigiBoard Technical

Support.

Saturation Counters

ERR_TBUF

The transmit buffers on the host adapter are full.  The data is not getting out

to the ports fast enough. Also monitor RECEIVE1 through RECEIVE5 and TRANSMIT

counters.

ERR_FIFO

The host adapter is receiving more data than it can handle.

Also monitor RECEIVE5.

ERR_ORUN

The host adapter is receiving too much data; the host is 100% busy. The

DigiBoard product in use is being maximized.  Consider adding another host

adapter, or advance to the EPC/X system if you are currently using the standard

C/X product.

ERR_TBUSY

The number of times the host adapter tried to transmit data when the

transmitter was busy.  Also monitor RECEIVE counters.

ERR_RITEM

The receive buffer on the host adapter is full; the host adapter

is 100% busy.  Also monitor the ERR_ORUN counter.

FEP Counters

ERR_REC0

The number of times the host adapter received a request from a concentrator,

but no packets were received.  If the environment is stable and no other

counters are incrementing, call DigiBoard Technical Support.

ERR_ADDR

The host adapter received a message that contains an invalid node number. This

is not a physical node number error, but a software node number error. If this

counter is constantly incrementing, call DigiBoard Technical Support.

ERR_MTYPE

The host adapter received a message with a bad message type. If this counter

is constantly incrementing, call DigiBoard Technical Support.

ERR_MSIZE

The host adapter received a message with a bad length. If this counter is

constantly incrementing, call DigiBoard Technical Support.

ERR_RID

The host adapter received a message with an out-of-sequence message ID. This is

not a physical node number error, but a software node number error. If this

counter is constantly incrementing, call DigiBoard Technical Support.

ERR_TID

The host adapter sync line transmit sequence number is out of sequence. If this

counter is constantly incrementing, call DigiBoard Technical Support.

ERR_PING

The host adapter received a sync line ping message when not in ping mode. If

this counter is constantly incrementing, call DigiBoard Technical Support.

Hardware Counters

ERR_ABREC

The host adapter aborted receiving a packet.  Run C/X host adapter diagnostics.

If a hardware problem is detected, try a different host adapter.

ERR_URUN

The host adapter transmitted an incomplete packet. Run C/X host adapter diag-

nostics. If a hardware problem is detected, try a different host adapter.

ERR_CONC

A message sent to the concentrators was sent back to the host adapter. A

concentrator is down. Back up one screen in DPA to check for "DN" status on

a concentrator. Check the power on the concentrator and reset the concentrator.

If the counter is still incrementing, try a different concentrator.

ERR_CHECK

The host adapter received an incomplete status packet. Run concentrator

diagnostics.  If a hardware problem is detected, try a different concentrator.