Installation Guide for

DigiBoard Intelligent Asynchronous Communications Boards

Novell NetWare Access Server Driver

DBI 92000109D

Hardware Installation

Install your board according to the instructions in the Installation Guide

that came with the board, and the following notes:

PC/Xi Boards and Older PC/Xe Boards

Older PC/Xe boards have 64K of dual ported memory, and have two banks of DIP

switches on the top edge of the board.  Newer PC/Xe boards (called ISA 2Port,

4Port and 8Port boards) have 8K of memory, and one bank of DIP switches on

the end of the board.

Memory Size

PC/Xi boards are available with 128K, 256K or 512K of dual ported memory.

Only the 128K version is supported, and jumper J3 must be set for 64K

operation (place jumper J3 on pins 2 and 3_the bottom two pins; see the

PC/Xi board Installation Guide for more information).

Memory Start Address

The starting address of the PC/Xi and PC/Xe boards' dual ported memory is

set by DIP switches on the board (see the PC/Xi and PC/Xe board Installation

Guide for switch setting information).  This device driver supports eight

starting addresses for the dual ported memory: 80000h, 90000h, A0000h, B0000h,

C0000h, D0000h and  E0000h.  Of these, D0000h is the most likely to be avail-

able. If you are installing two boards (for a maximum of sixteen channels),

set both boards to the same starting address.  Remember which memory start

address you chose; you will need it later when configuring the NASGEN software.

I/O Port Address

The I/O port address for PC/Xi and PC/Xe boards is also set by DIP switches

on the board. The I/O port address choices are: 100h, 110h, 120h, 200h, 220h,

300h or 320h.  Choose any address that is not already in use in your Access

Server. If you are installing two boards, each board must have its own unique

I/O port address.

Interrupt (IRQ) Selection

This device driver does not use interrupts.  Disable interrupts on the PC/Xi

or PC/Xe board by turning all eight switches on DS2 to the OFF position (away

from the board).

Be sure to write down the I/O address and Memory Start address for each board

installed; the NASGEN program will need this information when you configure

the software.

ISA 2Port, 4Port and 8Port Boards

There are two different versions of the PC/Xe board:  64K Boards

Older PC/Xe boards have 64K of dual ported memory, and have two banks

of DIP switches on the top edge of the board, for setting the memory

start address, I/O port address and interrupt (IRQ) selection. 8K Boards

Newer PC/Xe boards have 8K of dual ported memory, and have a single bank

of DIP switches in the endplate of the board, for setting the I/O port

address (memory start address and interrupt selection are handled by the

device driver on ISA 2Port, 4Port and 8Port boards).

Memory Start Address

The dual ported memory starting address for ISA 2Port, 4Port and 8Port

boards is set by the device driver software.  The address you choose

when configuring the software (with NASGEN) will automatically be

programmed into the board whenever you start your server. This device

driver supports 56 starting addresses for the dual ported memory on ISA

2Port, 4Port and 8Port boards.  Of these, the most likely to be available

are the addresses beginning with "D" (D0000h, D2000h, D4000h, etc.).

I/O Port Address

The I/O port address for ISA 2Port, 4Port and 8Port boards is set by DIP

switches on the end of the board. I/O port address choices are: 100h, 110h,

120h, 200h, 220h, 300h or 320h.  Choose any address that is not already in use

in your Access Server.  If you are installing two boards, each board must

have its own unique I/O port address.

Interrupt (IRQ) Selection

This device driver does not use interrupts. The interrupts will be disabled

automatically by the software.

Be sure to write down the I/O address and Memory Start address for each

board installed; the NASGEN program will need this information when you

configure the software.

MC/Xi Boards

Because the dual ported memory must be set for 32K operation, only newer

MC/Xi boards (part number 30001224) are supported by this device driver

software.

Configuring the MC/Xi Board for 32K Operation To set the MC/Xi board's

dual ported memory for 32K operation, locate the jumper, J1, near the

edge connector of the MC/Xi processor board.  Place the jumper on pins 2

and 3 (the right two pins).

Installing the MC/Xi Board

Install the MC/Xi board according to the instructions in the Installation

Guide that came with the board.  Once the board is installed, run the

configuration program on the IBM Reference Diskette to set the I/O and

memory address parameters.

Be sure to write down the I/O address and Memory Start address for each

board installed; the NASGEN program will need this information when you

configure the software.

MCA 2Port, 4Port and 8Port Boards

Install the MCA 2Port, 4Port or 8Port board according to the instructions

in the Installation Guide that came with the board.  Once the board is

installed, run the configuration program on the IBM Reference Diskette

to set the I/O and memory address parameters.

Be sure to write down the I/O address and Memory Start address for each

board installed; the NASGEN program will need this information when you

configure the software.

Copying the DigiBoard Software

Before you can proceed with the software installation and configuration,

the DigiBoard software must be copied from the distribution diskette onto

the Access Server hard drive.  The installation program INSTALL.EXE, on the

distribution disk, will do this for you, as well as copy the DigiBoard

configuration information into the appropriate NASGEN menu files. Place

the DigiBoard distribution diskette in the Access Server's diskette drive

A, and enter:

A:INSTALL

Follow the instructions on your screen to copy the files from the diskette

to the hard drive.  You will be asked for the diskette drive on which the

NAS software resides (typically drive C), and the subdirectory in which the

NASGEN software is located (typically \NASGEN\). When asked for your system's

architecture, select PC/AT-Bus Architecture if your server is an ISA or EISA

bus machine, or Micro Channel Architecture if your server is a Micro Channel

machine. Once the files have been installed, you will be returned to the DOS

prompt. Proceed to the following section to configure the system to use the

board(s).

Using NASGEN to Configure the NAS Software

The following procedure assumes that the DigiBoard serial communications

board has been installed in your machine, and that the DigiBoard program

INSTALL.EXE has been run to copy the files from the distribution disk, as

described in the previous pages.  It is also assumed that NASGEN has been

previously run (to install the NetWare Access Services).

Because software is constantly being revised and updated, some of the

screens and prompts may not appear exactly as shown here. Please refer

to your Novell NetWare Access Server documentation for complete

configuration information.

Installing and Configuring the Device Driver

1. Change directories to your NASGEN directory

(typically C:\NASGEN), and enter the following

command: NASGEN

You will see the NASGEN welcome screen.  Press

<Esc> or <Enter> to continue.

2. You will see the NASGEN Main Menu:

--------------------------------------------------------------------

|                       NASGEN Main Menu                           |

--------------------------------------------------------------------

|   |                                                              |

|   | Install NetWare Access Services                              |

|   | Modify NetWare Access Services                               |

|   | Upgrade NetWare Access Services                              |

|   | Exit NASGEN Program                                          |

|   |                                                              |

--------------------------------------------------------------------

Use the arrow keys to highlight Modify NetWare

Access Services, and press <Enter>.

3.  You will be asked to enter your Access Server boot drive letter.

This is typically C, but may be different on some servers. Enter the

appropriate drive letter for your server.

4.  You will now see the Modify Access Services Options Menu:

--------------------------------------------------------------------

|               Modify Access Services Options Menu                |

--------------------------------------------------------------------

|   |                                                              |

|   |  Hardware Configuration                                      |

|   |  Port Configuration                                          |

|   |  Security Configuration                                      |

|   |  User Profile Configuration                                  |

|   |  General Configuration                                       |

|   |  Administrator Actions                                       |

|   |  Exit Menu                                                   |

|   |                                                              |

--------------------------------------------------------------------

Use the arrow keys to highlight Hardware Configuration, and press <Enter>.

5.  You will now see the Hardware Configuration menu:

--------------------------------------------------------------------

|                   Hardware Configuration                         |

--------------------------------------------------------------------

|                                                                  |

|  Display Adapter Type: VGA  - Video Graphics Array               |

|  Network Adapter: SMC EtherCard PLUS V4.00EC                     |

|  Remote Communications Adapter Type(s)                           |

|   |                                                              |

|   |                                                              |

|  Communications Ports Enabled  0                                 |

|                                                                  |

--------------------------------------------------------------------

Use the arrow keys to move the highlighted bar to the Remote Communications

Adapter Type(s) field, and press <Insert> to add your DigiBoard adapter(s).

6.  You will see a list of known drivers similar to the one of the examples

shown below:

PC/AT Architecture

--------------------------------------------------------------------

|                       Known Driver(s)                            |

--------------------------------------------------------------------

|   |                                                              |

|   | WNIM+ Board(s)                                               |

|   | NASI Connections                                             |

|   | DigiBoard PC/4*  (64k) Multiport Board(s)                    |

|   | DigiBoard PC/8*  (64k) Multiport Board(s)                    |

|   | DigiBoard PC/16* (64k) Multiport Board                       |

|   | DigiBoard 2Port  (ISA) Multiport Board(s)                    |

|   | DigiBoard 4Port  (ISA) Multiport Board(s)                    |

|   | DigiBoard 8Port  (ISA) Multiport Board(s)                    |

|   |                                                              |

--------------------------------------------------------------------

Note: PC/X* indicates either a PC/Xi board, or an older (64K) PC/Xe

board.  2Port, 4Port and 8Port refer to newer (8K) PC/Xe boards.

Micro Channel Architecture

--------------------------------------------------------------------

|                       Known Driver(s)                            |

--------------------------------------------------------------------

|   |                                                              |

|   | WNIM+ Board(s)                                               |

|   | NASI Connections                                             |

|   | DigiBoard MC/4i Multiport  Board(s)                          |

|   | DigiBoard MC/8i Multiport  Board(s)                          |

|   | DigiBoard MC/16i Multiport Board                             |

|   | DigiBoard 2Port (MCA) Multiport  Board(s)                    |

|   | DigiBoard 4Port (MCA) Multiport  Board(s)                    |

|   | DigiBoard 8Port (MCA) Multiport  Board(s)                    |

|   | DigiBoard MC/4* Multiport  Board(s)                          |

|   | DigiBoard MC/8* Multiport  Board(s)                          |

|   |                                                              |

--------------------------------------------------------------------

Use the arrow keys to highlight the board you wish to install, and press

<Enter>. When installing two boards, you must choose one driver for both

boards. Use the following criteria when selecting a driver: If both boards

do not have the same number of ports, choose a driver which supports the

board with the greater number of ports. If both boards are not of the same

type (i. e. PC/AT style boards with different memory sizes or an MC/Xe

board and an MC/Xi board), select a driver with a * in its name.

Examples:

Boards                       Driver

8Port (ISA) and older PC/8e (64K)        PC/8*

Older PC/8e (2 boards)                   PC/8*

8Port (ISA; 2 boards)                    8Port

8Port (ISA) and 4Port (ISA)              8Port

Older PC/4e and 8Port (ISA)              PC/8*

PC/8i and 2Port (ISA)                    PC/8*

2Port (ISA) and 8Port (ISA)              8Port

2Port (MCA) and MC/8i                    MC/8*

MC/4i and 8Port (MCA)                    MC/8*

MC/4i (2 boards)                         MC/4i

MC/4i and MC/8i                          MC/8i

2Port (MCA) and 8Port (MCA)              8Port

7.  You will now see the board configuration menu. Depending on your

system's architecture (PC/AT or Micro Channel), it will look similar

to one of the following:

PC/AT Architecture

--------------------------------------------------------------------

|         DigiBoard Multiport (64k PC/8*) Configuration            |

--------------------------------------------------------------------

|                                                                  |

|  Memory Address (64KB.  All Boards.)     E000                    |

|   Board #1  I/O Address   0320   Installed       AltPin Disabled |

|   Board #2  I/O Address   0120   Not Installed   AltPin Disabled |

|                                                                  |

--------------------------------------------------------------------

Micro Channel Architecture

--------------------------------------------------------------------

|           DigiBoard Multiport (MC/8i) Configuration              |

--------------------------------------------------------------------

|                                                                  |

|   Board #1   Installed         I/O Address         F8F0          |

|              AltPin Enabled    H/W Memory Address  D000          |

|   Board #2   Installed         I/O Address         F8F0          |

|              AltPin Disabled   H/W Memory Address  D000          |

|                                                                  |

--------------------------------------------------------------------

7a.	Memory Address

Use the arrow keys to highlight the Memory Address of Board #1 (if you

are installing ISA (PC/Xe or PC/Xi) boards, there is only one memory

address for both boards) and press <Enter>.  You will see a list of possible

memory addresses for that board. Choose the address you wrote down when you

installed the board. If you are installing two Micro Channel boards, repeat

this step for Board #2.

7b. I/O Address

Use the arrow keys to highlight the I/O Address of Board #1 and press

<Enter>.  You will see a list of possible I/O addresses for that board.

Choose the address you wrote down when you installed the board. If you

are installing two boards, repeat this step for Board #2.

7c. AltPin Enabled/Disabled (RJ-45 Connectors Only)

The standard wiring for DigiBoard products equipped with RJ-45 connectors

places the modem control lines RI (Ring Indicator) and DCD (Data Carrier

Detect) on pins 1 and 10, respectively, which makes them inaccessible to

an 8-pin RJ-45 plug.  While RI is seldom used, DCD is necessary for most

modem applications.  To facilitate the use of 8-pin RJ-45 plugs, a software

feature called "AltPin" has been included with this device driver. When

AltPin is enabled, DCD is swapped with DSR (Data Set Ready), which is on

pin 1 of an 8-pin RJ-45 plug. AltPin should always be disabled if your board

is equipped with DB-25 or DB-9 connectors. To enable or disable AltPin, use

the arrow keys to highlight the AltPin field for the desired board, and press

<Enter>.  Then select Enabled or Disabled from the list that appears and

press <Enter> again.

7d.  Installed/Not Installed

Finally, you need to make the board visible to the device driver. Use the

arrow keys to highlight the Installed/Not Installed field for the desired

board and press <Enter>.  Choose "Installed" from the list that appears and

press <Enter> again. To disable a board, select "Not Installed".  All other

board setup parameters will remain unaffected, but the board will be

invisible to the system.

7e.  Board Configuration Complete

When all board parameters have been set to your satisfaction, press <Esc>.

Answer "Y" when asked if you want to update the configuration.  You will

then be returned to the main Hardware Configuration menu.

8.  Now use the arrow keys to highlight the Communications Ports Enabled

field and press <Enter>.  Enter the combined number of ports for both

boards.  For example, if Board #1 is a PC/8i board and Board #2 is a

4Port board, enter "12".

9.  Press <Esc> to exit the Hardware Configuration menu and return to

the Modify Access Services Options Menu.

This completes the installation of the DigiBoard device driver.

You may now set up the individual port parameters (baud rates,

parity, etc.) by selecting the Port Configuration item from the

Modify Access Services Options Menu.  Instructions for configuring

the ports are in the Novell NetWare Access Server Administration

Guide.

When you quit the NASGEN program, be sure to save the changes you

made! You will be prompted at the appropriate time during the exit procedure.

Command Line Configuration

The DigiBoard Novell NAS device driver can also be installed from the

DOS command line.  The format for the command is:

DIGIXA /M1:hhhh /B1:hhh [/M2:hhhh /B2:hhh]

DIGIXA /I

DIGIXA /?

The parameters are:

/M1:hhhh   Starting address for dual ported memory of Board #1 (or

both boards, if the system is PC/AT architecture). This is

a segment address, and not an absolute address. If the

board's dual ported memory start address is set to D0000h,

use /M1:D000.

/B1:hhh	   I/O port address for Board #1.

/M2:hhhh   Starting address for dual ported memory of Board #2.

This parameter is valid only on Micro Channel installations.

/B2:hhh	   I/O port address for Board #2.

/I	   Driver information/configuration

/?	   Help display.

Error Messages

The following error messages may occur while the driver is being loaded.

Warning: Driver Already Installed

The driver is already in memory, and doesn't need to be loaded again.

Error: No DigiBoard intelligent ports were found Either the boards are

not in the computer, or they are not configured as expected (i.e. as

set up via the NASGEN program or command line parameters). Check the

DIP switch settings on the boards, and run NASGEN again to verify

the settings. Remember that PC/Xe and PC/Xi boards must have the same

memory start address.

Error: EMS page frame at hhhh overlaps with board RAM segment *

The memory address you have chosen conflicts with your system's EMS

page frame address.  Choose a new address.

Warning: XX extra channels on the above board are unused *

This is not an error, but indicates that not all of the channels on

the board are being used by the software.

Unused board found at I/O Address: hhh

A board was found at I/O address hhh that is not referenced in the driver

configuration.  Rerun NASGEN.

Unknown board found at I/O Address: hhh

Board at I/O address hhh is not a supported board type.

Not enough boards found in system

Didn't find all of the boards for which the driver was configured.

Either an attempt was made to reference an unsupported board, or

not enough boards have been installed in the system.

System configuration does not match the installed configuration

POS configuration does not match the installed configuration.

Indicates an I/O port or memory segment mismatch. Check the POS

settings (use the IBM Reference Diskette) and verify them with

the NASGEN (or command line) parameters.

DigiBoard NAS driver is installed

Success message.  Driver loaded, and found all expected boards.

Board reset failed

The driver could not reset the board. This usually indicates a

hardware failure, but could also result from an I/O port address

conflict with some other device in the system.

Memory test failed at address: hhhh

No memory was found at the address specified during driver configuration.

If the board's memory start address is set via DIP switches, verify the

switch settings.  If the board is an MC/Xi or MC/Xe board, check the POS

settings (use the IBM Reference Diskette). The board's memory address

setting must match the NASGEN memory address configuration.

BIOS startup failed

BIOS move of FEP/OS failed

FEP/OS execution failed

These messages indicate that the initialization process failed while

the driver was attempting to load or use on-board code.  Failures of

this kind may be caused by a board hardware malfunction, or by memory

address conflicts with another device in your system. Try rebooting

the system. If the message persists, check your system for possible

address conflicts.

Bad parameter/switch in command line, use /? to list switches.

Indicates an error in DIGIXA command line syntax.