..............................................................

.   DigiCHANNEL PC/X Installation Procedure  for             .

.   OpenVMS   Alpha based 2100 server                        .

.                                                            .

.  Installation is broken down into two separate functions:  .

.                                                            .

.  I.  Hardware installation procedure [Field Service  ]     .

.  II. Software installation procedure [F.S. & Sys.Mgr.]     .

..............................................................

*** CAVEATS:

1.  PC/8 has only been tested with more than one module

installed (3) in a limited modified configuration.

One module is all that is currently supported.

2.  Known EISA IRQ conflicts in 2100 server running OpenVMS:

o= IRQ's that worked in PC/8 3-module test; but

noted configuration changes had to be made.

IRQ 0 - Assigned to FLOPPY               !DEC2A01.CFG

IRQ 1 - Assigned to COMM2                !DEC2A01.CFG

IRQ 2 - Assigned to PARALLEL PORT        !DEC2A01.CFG

o IRQ 3 - Free for PC/X               use  !VMSB905.CFG

IRQ 4 - Does not work with PC/X  ??

o IRQ 5 - Default for DE422 if installed.  !DEC4220.CFG

IRQ 6 - Does not work with PC/X  ??

o IRQ 7 - Available but requires edit of   !DEC2A01.CFG

IRQ 8 - Assigned to COMM1                !DEC2A01.CFG

IRQ 9 - Assigned to QVISION if installed !ACPQ3nnn.CFG

SYSNOPSIS:

1. Configure the physical module by running the ECU and setting

the switches, and jumpers according to:

>>>RUN ECU

Step 2: ADD or Remove Boards

Select file to ADD               { !VMSB905.CFG }

Step 3: View or edit                 { Optional     }

Step 4: Examine required switches

View   { Set switches and jumpers as indicated. }

Step 5: Save and Exit

2. Extract and run  DIGI.COM  which in turn creates:

LAST_VERSION.TXT

DIGI_CONNECT_TTn.COM

3. Extract and edit  DIGI_LAST_VERSION.COM according to

'DIGI_CONNECT_TTn' values generated by  DIGI.COM .

( Will execute on subsequent system reboots.)

@SYS$SYSTEM:DIGI_CONNECT_TTn.COM

4. Edit SYSTARTUP_VMS.COM to include line:

@SYS$SYSTEM:DIGI_LAST_VERSION.COM

I. HARDWARE INSTALLATION

------------------------

REFERENCE:  Installation Guide and Reference Manual

for

DigiCHANNEL PC/8(tm)

DigiCHANNEL PC/4(tm)

DBI 90021400G

1.  PRIOR TO INSTALLING the module.

A.  !VMSB905.CFG  - modification of Digi AISAB905.CFG which

by default selected base addresses as 100h-3ffh.

[ Use  REFERENCE - APPENDIX C  ]

Extract the !VMSB905.CFG file from this document, create

a FAT formatted file on a PC and call it  !VMSB905.CFG

and put it onto your OpenVMS/OSF ECU diskette.  Note that

this file is identical to the AISAB905.CFG file that

is already on the disk, but has the default base address

values of 138h-3ffh; and IRQ 2,4,6 removed.

B.  Run the OpenVMS/OSF ECU (EISA Configuration Utility )

Note: Loading the ECU information from the floppy takes

several minutes, especially if it contains many

.CFG files.  It may be useful to make a copy and

delete those .CFG files which are not pertinent to

your system configuration. (Label =  SYSTEMCFG )

1. Use the 'ADD or remove boards' option in ECU and ADD the

!VMSB905.CFG for a selected slot (the one you will place

the module in).

>>>RUN ECU      (OpenVMS ECU floppy)

[ Step 2: Add or remove boards ]

2. EXAMINE - the switch and jumper settings for PC/X module as

calculated by ECU from base address file information;

the switches on the module are not interpreted by the ECU.

[ Step 3: View or edit  ( optional )]

[ Step 4: Examine required switches ]

VIEW

Set the physical switches and the jumpers on the PC/X module

according to the VIEW information generated by the ECU.

DS1             = CSR

DS2-DS9         = 8 port address

J1-J8 + J85-J90 = IRQ settings

HINT:  Switch values and address interpretations

-----------------

|8 switch dip   |

|1 2.3 4 5 6.7 8|

----.--------.---

O F.O O F F.F O    where  F=Off O=ON

2 1.8 4 2 1.8 NA   hex wt. value of each sw. pos

1.      3.8      138h Alphanumeric interpretation

The F values are interpreted and the hexidecimal wt. values

for the position relative to the '.' are added to translate

the switch position into a hexidecimal address.

SW 1-2  hex. range   = 0-3

SW 3-6  hex. range   = 0-F

SW 7    hex. range   = 0 or 8

3. Exit the ECU

[ Step 5: SAVE AND EXIT ]

4. Shut the system off.

5. Install the PC/X, which should now have the correct switch

and jumpers settings to match the ECU information, into

the EISA slot you selected.

6. Restart the system and boot OpenVMS

7. NOTE: The PC/X will not configure the adapter automatically.

**************

At this point the module has been physically installed

in the machine with the appropriate:

IRQ, CSR address, and I/O port addresses.

The ECU has been run and the hardware knows about the PC/X

module. The adapter module has not yet been connected to any

OpenVMS device driver, and no logical connections have been

defined for individual terminal lines.

OpenVMS should be up and running.

***************

You should now go to -  II: Software installation procedure.

II.     SOFTWARE INSTALLATION

-----------------------------

You can manually connect each PC/X line to an adapter using a

privileged account and SYSMAN utility IO commands to ascertain

the additional parameters ( CSR, vector, node, adaptor)

necessary to format an IO command, and then create duplicate

DCL scripts.

DCL scripts will be necessary if you do not wish to have to manually

connect all the terminal lines after each reboot. The scripts should

be  run at STARTUP time to logically connect the adapter to the

device lines automatically.

OR      [ Recommended method ]

You can use/create a set of .COM, DCL script files using some of

the template files appended to the end of this document.

IE:

DIGI.COM -                  [  REFERENCE: APPENDIX  A      ]

generic template initially used specifically for

2100 server platform, may can be modified for other

hardware platform use.

*** CAVEAT

NOTE:   The file 'DIGI.COM' creates 'DIGI_CONNECT_TTn.COM'

which must be executed each time the system boots (it should

be put it into the DIGI_LAST_VERSION.COM file). These .COM files

are potentially obsoleted by changes in hardware configuration

and software installation and upgrades. IE: If the base CSR

virtual address for the FLOPPY changes the offsets that are

calculated in formulating the .COM file will be incorrect

on subsequent reboots. The DIGI.COM file will have to

be rerun.

DIGI_CONNECT_TTn.COM -      [  REFERENCE: APPENDIX  B      ]

.COM files which are generated as result of running

previously listed 'DIGI.COM' file. Used to create

the initial adapter to logical terminal connections and

provide auto-method for subsequent reboots. Invoked  during

SYSTARTUP_VMS.COM routine by means of command inserted for

DIGI_LAST_VERSION.COM file .  Connects ports to OpenVMS

SYS$YSDRIVER.

DIGI_LAST_VERSION.COM -     [  REFERENCE: APPENDIX  D      ]

.COM file which will check file SYS$MANAGER:LAST_VERSION.TXT

created by running DIGI.COM for a change in OpenVMS version;

if there is a change the previous versions of DIGI_CONNECT_TTn.

COM are assumed to be obsolete  because they are dependent on

the virtual address of the FLOPPY. The files will be renamed

and you must rerun the DIGI.COM file.

A P P E N D I X         A

-------------------------

********START EXTRACTING - DIGI.COM - ON THE NEXT LINE  *******

$!  DIGI.COM -      .COM file used to partially automate the

$!                PC/X Digi adapater connection for on an OpenVMS

$!                ALPHA 2100 server platform.

$!  ------------------------------------------------------------

$!  NOTE: This command procedure is provided as an example only.

$!      Use entirely at your own risk, no support is available.

$! ................................................................

$! .                                                            .

$! .  File as originally written is specific to the 2100 server   .

$! .  platform, therefore, it will be necessary to edit areas of  .

$! .  this file to conform to future platform release nuances.          .

$! .                                                              .

$! ................................................................

$ VERSION = "1.3"

$!

$ SAY = "WRITE SYS$OUTPUT"

$ SAY " "

$!-------------------------------------------------------------------!

$!     Use this lexical to isolate the system identification value   !

$!     in order to create an instruction branch to code specific to  !

$!     the OpenVMS hardware platform. Example: value 9 = 2100 server !

$!-------------------------------------------------------------------!

$ CPU = F$INTEGER(F$EXTRACT(16,8,F$GETSYI("NODE_HWVERS")))

$ IF CPU .EQS. 9

$    THEN GOTO CPU9           ! Sable

$     ELSE

$ SAY "DigiCHANNEL PC/X is not supported on this CPU type.  Installation aborted."

$   EXIT

$ ENDIF

$CPU9:

$!-------------------------------------------------------------------!

$!                                                                   !

$! Check for current devices and create a new device string          !

$! This is system is a 2100 (9) so go get corresponding slot number. !

$!                                                                   !

$!-------------------------------------------------------------------!

$     devices := "-,A,B,C,D,E,F,G,H,I,J,K,L,M,N,O,P"

$     count = 0

$             MAKE_DEVICE:

$             count = count + 1

$             if count .lt. 8 then goto nextdev

$                     say "There are too many devices of type TT in the system.  Can not continue."

$                     exit

$                     nextdev:

$                     dev_name = "TT"+f$element('count,",",devices)

$!                    show sym dev_name

$                     if f$getdvi("''dev_name'","exists") then goto make_device

$

$ SAY "DIGI.COM Version ''version' building DIGI_CONNECT_''dev_name'.COM"

$!

$ SLOT_TYPE = "E"

$!----------------------------------------------------------------------------!

$! Following command only executes on an Alpha based machine - requires PRIVS !

$!----------------------------------------------------------------------------!

$ spawn/nolog/out=get_csr.tmp mc sysman io show bus

$ open infile get_csr.tmp

$ find_eisa:

$     read infile line

$     line = f$edit(line,"upcase,compress,trim")

$     bus = f$element(0," ",line)

$!            show sym bus

$     if bus .nes. "XBUS" then goto find_eisa

$ test = ""

$ count = 0

$ find_csr:

$       count = count + 1

$     digi_csr = test

$     test = f$element('count," ",line)

$!    show sym test

$     if test .eqs. " " then goto got_csr

$     goto find_csr

$ got_csr:

$! show sym digi_csr

$ close infile

$ pur get_csr.tmp

$

$!-------------------------------------------------------------------!

$!    Query for type of PC/X module - is it a PC/4 or PC/8         !

$!-------------------------------------------------------------------!

$GET_TYPE:

$ PROMPT = F$FAO("!52AS","Enter the module type [PC4 or PC8] ")

$ INQUIRE/NOPUNC DIGI_TYPE "''PROMPT'"

$! show sym digi_type

$ DIGI_TYPE = F$EDIT(DIGI_TYPE,"UPCASE,COMPRESS,TRIM")

$! show sym digi_type

$ IF DIGI_TYPE .EQS. "" THEN GOTO GET_TYPE

$     IF DIGI_TYPE .EQS. "PC4" THEN GOTO GOT_TYPE

$     IF DIGI_TYPE .EQS. "PC8" THEN GOTO GOT_TYPE

$

$ SAY "VALID MODULE TYPES ARE PC4 OR PC8 PLEASE RE-ENTER MODULE TYPE"

$! loop 'til we get a correct module type

$ GOTO GET_TYPE

$!

$ GOT_TYPE:

$!-------------------------------------------------------------------!

$!    Slot configuration information for 2100 server               !

$!-------------------------------------------------------------------!

$GET_SLOT:

$ PROMPT = F$FAO("!52AS","Enter the slot number for the Digi controller: ")

$ INQUIRE/NOPUNC SLOT "''PROMPT'"

$ IF SLOT .EQS. "" THEN GOTO GET_SLOT

$     IF SLOT_TYPE .EQS. "E" THEN MAX_SLOT = 8

$     IF SLOT_TYPE .EQS. "P" THEN MAX_SLOT = 2

$!

$     IF (SLOT .GT. 0) .AND. (SLOT .LE. MAX_SLOT) THEN GOTO GET_IRQ

$ SAY "VALID SLOT NUMBERS ARE:"

$ SAY "                                                                       "

$ SAY "               EISA   8                                                "

$ SAY "                      7                                                "

$ SAY "                      6                                                "

$ SAY "                      5                                                "

$ SAY "                      4                                                "

$ SAY "                      3                                                "

$ SAY "                      2                                                "

$ SAY "                      1                                                "

$ SAY "                      STANDARD I/O MODULE                              "

$ SAY "                PCI   0                                                "

$ SAY "                      1                                                "

$ SAY "                      2                                                "

$ SAY "                                                                       "

$! loop 'til we get a slot number!

$ GOTO GET_SLOT

$!

$! we have slot number, now get IRQ

$!

$!-------------------------------------------------------------------!

$!    Request IRQ information for PC/X - MUST MATCH MODULE JUMPER  !

$!-------------------------------------------------------------------!

$GET_IRQ:

$ PROMPT = F$FAO("!52AS","Enter the IRQ for the Digi controller [3]: ")

$ INQUIRE/NOPUNC IRQ "''PROMPT'"

$! loop 'til we get an IRQ...

$ IF IRQ .EQS. "" THEN IRQ = 3

$ IF IRQ .EQS. 2 THEN GOTO IRQERR

$ IF IRQ .EQS. 3 THEN GOTO COMPUTE_SYSMAN

$ IF IRQ .EQS. 4 THEN GOTO IRQERR

$ IF IRQ .EQS. 5 THEN GOTO COMPUTE_SYSMAN

$ IF IRQ .EQS. 6 THEN GOTO IRQERR

$ IF IRQ .EQS. 7 THEN GOTO COMPUTE_SYSMAN

$ IRQERR:

$ SAY "IRQ is not valid.  It must be 3,5, or 7."  !Even IRQ's don't work ??

$ GOTO GET_IRQ

$!

$! got a valid IRQ

$!

$!--------------------------------------------------------------------!

$!  We have module TYPE, SLOT number, IRQ value. Calculate the VECTOR.!

$!--------------------------------------------------------------------!

$COMPUTE_SYSMAN:

$ IF CPU .EQS. 9        ! if SABLE

$!

$ THEN

$!

$     EISA_SLOT:

$!

$!  FOR SABLE EISA  VECTOR    =       [(IRQ+7)*4]     DECIMAL

$!                 ADAPTER    =       3               DECIMAL

$!          NODE HIGH WORD      =     IRQ+7           HEX

$!           NODE LOW WORD      =     SLOT_NUMBER     HEX

$!

$             IF SLOT_TYPE .NES. "E" THEN GOTO PCI_SLOT

$!                    SAY "CALCULATING VALUES FOR EISA SLOT"

$

$                             NODE_IRQ = IRQ + 7

$                              ADAPTER = 3

$ VECTOR=NODE_IRQ*4

$ NODE_IRQ = F$FAO("!XW", F$INTEGER(NODE_IRQ))

$ NODE_SLOT = F$FAO("!XW", F$INTEGER(SLOT))

$ NODESPEC := "%X''NODE_IRQ'''NODE_SLOT'"

$!

$                     GOTO CALC_VECTOR

$!

$     PCI_SLOT:

$!

$!  FOR SABLE PCI   VECTOR    =       0,16,20 FOR SLOTS 0,1,2 DECIMAL

$!                 ADAPTER    =       2                       DECIMAL

$!                  NODE      =       48,56,64    "     0,1,2 DECIMAL

$!

$                     SAY "CALCULATING VALUES FOR PCI SLOT"

$!

$     ADAPTER = 2

$!

$     IF SLOT .EQS. 0 THEN NODE_IRQ = 0

$     IF SLOT .EQS. 1 THEN NODE_IRQ = 4

$     IF SLOT .EQS. 2 THEN NODE_IRQ = 5

$     SABLE_SLOT = SLOT + 6

$ VECTOR=NODE_IRQ*4

$ NODE_IRQ = 8 * SABLE_SLOT

$ NODESPEC := "''NODE_IRQ'"

$!

$ CALC_VECTOR:

$ ELSE

$   NODE_IRQ = IRQ

$   ADAPTER = 2

$ VECTOR=NODE_IRQ*4

$ VECTOR = F$FAO("!XB", F$INTEGER(VECTOR))

$ NODE_IRQ = F$FAO("!XW", F$INTEGER(NODE_IRQ))

$ NODE_SLOT = F$FAO("!XW", F$INTEGER(SLOT))

$ NODESPEC := "%X''NODE_IRQ'''NODE_SLOT'"

$ ENDIF

$!

$!

$! Now convert to hex and pad out to 16 bits.

$!

$!

$! Define some logicals so other procedures (e.g. PCSI) can grab some

$! important info.

$!

$ define/job/nolog qvi$vector 'vector'

$ define/job/nolog qvi$node 'nodespec'

$ define/job/nolog qvi$slot 'slot'

$ define/job/nolog qvi$adapter 'adapter'

$ outfilename = "DIGI_CONNECT_''dev_name'.com"

$! show sym outfilename

$!-------------------------------------------------------------------!

$!  Create .COM files based on module TYPE, IRQ, VECTOR, device_name.!

$!-------------------------------------------------------------------!

$ open/write outfile 'outfilename

$!

$     if digi_type .eqs. "PC4" then numports = 4

$     if digi_type .eqs. "PC8" then numports = 8

$! show sym numports

$       count = 0

$ digi_type_loop:

$     write outfile "$ mc sysman io con /csr=%x''digi_csr'/vector=''vector'/node=''nodespec'/adapt=''adapter' ''dev_name'''count': /driver=sys$ysdriver"

$     count = count +1

$     if count .lt. numports then goto digi_type_loop

$!

$ close outfile

$ purge 'oufilename

$! TYPE 'outfilename

$ say " "

$ say "**************************************************************** "

$ say "*                You will need to put the command              *       "

$ say "*                                                              * "

$ say "*          @SYS$MANAGER:DIGI_LAST_VERSION.COM                  * "

$ say "*                                                              * "

$ say "*                in your SYSTARTUP_VMS.COM file                * "

$ say "**************************************************************** "

$ say " "

$ say "Executing SYS$MANAGER:DIGI_CONNECT_''dev_name'.COM to connect the PC/X"

$ say " "

$ @SYS$MANAGER:DIGI_CONNECT_'dev_name.COM

$! show dev 'dev_name

$ say " "

$ say "If your system contains more than one Digi PC/4 or PC/8 "

$ say "run this command procedure once for each module"

$ say " "

$ say " "

$! Create a file to check OpenVMS version changes against.

$ CURRENT_VERSION = F$GETSYI("VERSION")

$ outfilename = "SYS$MANAGER:LAST_VERSION.TXT"

$ open/write outfile 'outfilename

$     write outfile "''CURRENT_VERSION'"

$ close outfile

$ purge 'outfilename

$ exit

*****COMPLETE EXTRACTION OF - DIGI.COM - AT END OF PREVIOUS LINE  *******

A P P E N D I X         B

-------------------------

!------------------------------------------------------------------------!

!  DIGI_CONNECT_TTn.com  -                                               !

!   example .COM file generated as a result of running DIGI.COM during !

!   the installation of PC/X module. Facsimile of this file should be  !

!     found in the SYS$MANAGER directory and a command line should be    !

!     included in the DIGI_LAST_VERSION.COM which will be invoked by     !

!     SYSTARTUP_VMS.COM file for each subsequent reboot procedure.       !

!                                                                        !

!------------------------------------------------------------------------!

!------------------------------------------------------------------------!

!  The example .COM file listed is strictly informational. The actual    !

!  .COM file to be used depends on information that is unique to each    !

!  system configuration.                                               !

!------------------------------------------------------------------------!

**  DO NOT EXTRACT THIS FILE - IT IS UNIQUE TO THE SYSTEM CONFIGURATION **

$! DIGI_CONNECT_TTn.com

$!

$ mc sysman io con /csr=%xFFFFFFFF8206A000/vector=40/node=%X000A0006/adapt=3 TTB0: /driver=sys$ysdriver

$ mc sysman io con /csr=%xFFFFFFFF8206A000/vector=40/node=%X000A0006/adapt=3 TTB1: /driver=sys$ysdriver

$ mc sysman io con /csr=%xFFFFFFFF8206A000/vector=40/node=%X000A0006/adapt=3 TTB2: /driver=sys$ysdriver

$ mc sysman io con /csr=%xFFFFFFFF8206A000/vector=40/node=%X000A0006/adapt=3 TTB3: /driver=sys$ysdriver

$ mc sysman io con /csr=%xFFFFFFFF8206A000/vector=40/node=%X000A0006/adapt=3 TTB4: /driver=sys$ysdriver

$ mc sysman io con /csr=%xFFFFFFFF8206A000/vector=40/node=%X000A0006/adapt=3 TTB5: /driver=sys$ysdriver

$ mc sysman io con /csr=%xFFFFFFFF8206A000/vector=40/node=%X000A0006/adapt=3 TTB6: /driver=sys$ysdriver

$ mc sysman io con /csr=%xFFFFFFFF8206A000/vector=40/node=%X000A0006/adapt=3 TTB7: /driver=sys$ysdriver

A P P E N D I X         C

-------------------------

!---------------------------------------------------------------------------!

! Example of a modified version of AISAB905.CFG Digi provided configuration !

! file for PC/8.                                                            !

! The original version by default set base address values to 100h-3ffh which!

! conflicted with addresses 102-130 already being used by the 2100 server.  !

! New base address is 138h-3ffh. IRQ 2,4,6  removed as options.             !

!---------------------------------------------------------------------------!

!---------------------------------------------------------------------------!

! Recommended 2100 server default settings:                                 !

!                                                                           !

! DS1      DS2   DS3   DS4   DS5   DS6   DS7    DS8   DS9                   !

! 138        140   148   150   158   160   168    170   178                 !

! CSR         Port  1     2     3     4     5     6      7     8            !

!                                                                         !

! IRQ3  J85                                                               !

!---------------------------------------------------------------------------!

!---------------------------------------------------------------------------!

!**The following is the !VMSB905.CFG file to be extracted and copied to     !

!  create a FAT format file on a PC for use with OpenVMS ECU.               !

!---------------------------------------------------------------------------!

********BEGIN EXTRACTING - !VMSB905.CFG - AT FOLLOWING LINE  *******

BOARD                   ; Modified version of the AISAB905.CFG for 2100 VMS

ID="VMSB905"

NAME="DigiBoard DigiCHANNEL PC/8 VMS AXP 2100 server settings"

MFR="DigiBoard"

CATEGORY="COM"

SLOT=ISA8

COMMENTS="

\nThe DigiBoard DigiCHANNEL board requires the use of a device driver

appropriate to your operating system. Each of the supported

operating systems has recommended parameter values. Refer to

documentation for these values and for further information.

\n\nIn order for the PC/8 to run correctly in a 2100 server, it

must be configured so as not to conflict with the onboard device settings.

Base address range is changed to 138h-3ffh.  IRQ 's 2,4,6 are removed.

"

SWITCH(1)=10

NAME="DS1"

STYPE=DIP

VERTICAL=NO

REVERSE=YES

SWITCH(2)=8

NAME="DS2"

STYPE=DIP

VERTICAL=NO

REVERSE=YES

SWITCH(3)=8

NAME="DS3"

STYPE=DIP

VERTICAL=NO

REVERSE=YES

SWITCH(4)=8

NAME="DS4"

STYPE=DIP

VERTICAL=NO

REVERSE=YES

SWITCH(5)=8

NAME="DS5"

STYPE=DIP

VERTICAL=NO

REVERSE=YES

SWITCH(6)=8

NAME="DS6"

STYPE=DIP

VERTICAL=NO

REVERSE=YES

SWITCH(7)=8

NAME="DS7"

STYPE=DIP

VERTICAL=NO

REVERSE=YES

SWITCH(8)=8

NAME="DS8"

STYPE=DIP

VERTICAL=NO

REVERSE=YES

SWITCH(9)=8

NAME="DS9"

STYPE=DIP

VERTICAL=NO

REVERSE=YES

JUMPER(1)=1

NAME="J1"

JTYPE=TRIPOLE

VERTICAL=NO

REVERSE=NO

LABEL=LOC(1) "J1"

JUMPER(2)=1

NAME="J2"

JTYPE=TRIPOLE

VERTICAL=NO

REVERSE=NO

LABEL=LOC(1) "J2"

JUMPER(3)=1

NAME="J3"

JTYPE=TRIPOLE

VERTICAL=NO

REVERSE=NO

LABEL=LOC(1) "J3"

JUMPER(4)=1

NAME="J4"

JTYPE=TRIPOLE

VERTICAL=NO

REVERSE=NO

LABEL=LOC(1) "J4"

JUMPER(5)=1

NAME="J5"

JTYPE=TRIPOLE

VERTICAL=NO

REVERSE=NO

LABEL=LOC(1) "J5"

JUMPER(6)=1

NAME="J6"

JTYPE=TRIPOLE

VERTICAL=NO

REVERSE=NO

LABEL=LOC(1) "J6"

JUMPER(7)=1

NAME="J7"

JTYPE=TRIPOLE

VERTICAL=NO

REVERSE=NO

LABEL=LOC(1) "J7"

JUMPER(8)=1

NAME="J8"

JTYPE=TRIPOLE

VERTICAL=NO

REVERSE=NO

LABEL=LOC(1) "J8"

JUMPER(9)=1

NAME="J9"

JTYPE=TRIPOLE

VERTICAL=NO

REVERSE=NO

LABEL=LOC(1) "J9"

JUMPER(10)=1

NAME="J10"

JTYPE=TRIPOLE

VERTICAL=NO

REVERSE=NO

LABEL=LOC(1) "J10"

JUMPER(11)=1

NAME="J85"

JTYPE=paired

VERTICAL=NO

REVERSE=NO

LABEL=LOC(1) "J85"

JUMPER(12)=1

NAME="J86"

JTYPE=paired

VERTICAL=NO

REVERSE=NO

LABEL=LOC(1) "J86"

JUMPER(13)=1

NAME="J87"

JTYPE=paired

VERTICAL=NO

REVERSE=NO

LABEL=LOC(1) "J87"

JUMPER(14)=1

NAME="J88"

JTYPE=paired

VERTICAL=NO

REVERSE=NO

LABEL=LOC(1) "J88"

JUMPER(15)=1

NAME="J89"

JTYPE=paired

VERTICAL=NO

REVERSE=NO

LABEL=LOC(1) "J89"

JUMPER(16)=1

NAME="J90"

JTYPE=paired

VERTICAL=NO

REVERSE=NO

LABEL=LOC(1) "J90"

SOFTWARE(1) ="

The DigiBoard DigiCHANNEL board requires the installation of a device

driver appropriate to your operating system. Refer to the

DigiBoard documentation for details and configure using the following

parameters:\n"

FUNCTION="Multiport Communications"

TYPE="COM"

SUBFUNCTION="All Operating Systems"

CHOICE="Adapter Options"

LINK

IRQ = 3|5|7

INIT=JUMPER(11) LOC(1) 1|0|0

INIT=JUMPER(12) LOC(1) 0|1|0

INIT=JUMPER(13) LOC(1) 0|0|1

INIT=JUMPER(14) LOC(1) 0|0|0

INIT=JUMPER(15) LOC(1) 0|0|0

INIT=JUMPER(16) LOC(1) 0|0|0

INIT=JUMPER(1)  LOC(1) 1|1|1

INIT=JUMPER(2)  LOC(1) 1|1|1

INIT=JUMPER(3)  LOC(1) 1|1|1

INIT=JUMPER(4)  LOC(1) 1|1|1

INIT=JUMPER(5)  LOC(1) 1|1|1

INIT=JUMPER(6)  LOC(1) 1|1|1

INIT=JUMPER(7)  LOC(1) 1|1|1

INIT=JUMPER(8)  LOC(1) 1|1|1

SUBFUNCTION="Status Register Port"

CHOICE="Enabled"

LINK

PORT=0138h-03FFh Step=8

INIT=SWITCH(1) LOC(1-7) 1011000-0000000

FREE

INIT=SWITCH(1) LOC(8-10) 111

CHOICE="Disabled"

FREE

INIT=SWITCH(1) LOC (1-10) 0000000000

SUBFUNCTION="Port 1"

CHOICE="Enabled"

LINK

PORT=0138h-03FFh Step=8

INIT=SWITCH(2) LOC(1-7) 1011000-0000000

FREE

INIT=SWITCH(2) LOC(8) 1

CHOICE="Disabled"

FREE

INIT=SWITCH(2) LOC (8) 0

SUBFUNCTION="Port 2"

CHOICE="Enabled"

LINK

PORT=0138h-03FFh Step=8

INIT=SWITCH(3) LOC(1-7) 1011000-0000000

FREE

INIT=SWITCH(3) LOC(8) 1

CHOICE="Disabled"

FREE

INIT=SWITCH(3) LOC (8) 0

SUBFUNCTION="Port 3"

CHOICE="Enabled"

LINK

PORT=0138h-03FFh Step=8

INIT=SWITCH(4) LOC(1-7) 1011000-0000000

FREE

INIT=SWITCH(4) LOC(8) 1

CHOICE="Disabled"

FREE

INIT=SWITCH(4) LOC (8) 0

SUBFUNCTION="Port 4"

CHOICE="Enabled"

LINK

PORT=0138h-03FFh Step=8

INIT=SWITCH(5) LOC(1-7) 1011000-0000000

FREE

INIT=SWITCH(5) LOC(8) 1

CHOICE="Disabled"

FREE

INIT=SWITCH(5) LOC (8) 0

SUBFUNCTION="Port 5"

CHOICE="Enabled"

LINK

PORT=0138h-03FFh Step=8

INIT=SWITCH(6) LOC(1-7) 1011000-0000000

FREE

INIT=SWITCH(6) LOC(8) 1

CHOICE="Disabled"

FREE

INIT=SWITCH(6) LOC (8) 0

SUBFUNCTION="Port 6"

CHOICE="Enabled"

LINK

PORT=0138h-03FFh Step=8

INIT=SWITCH(7) LOC(1-7) 1011000-0000000

FREE

INIT=SWITCH(7) LOC(8) 1

CHOICE="Disabled"

FREE

INIT=SWITCH(7) LOC (8) 0

SUBFUNCTION="Port 7"

CHOICE="Enabled"

LINK

PORT=0138h-03FFh Step=8

INIT=SWITCH(8) LOC(1-7) 1011000-0000000

FREE

INIT=SWITCH(8) LOC(8) 1

CHOICE="Disabled"

FREE

INIT=SWITCH(8) LOC (8) 0

SUBFUNCTION="Port 8"

CHOICE="Enabled"

LINK

PORT=0138h-03FFh Step=8

INIT=SWITCH(9) LOC(1-7) 1011000-0000000

FREE

INIT=SWITCH(9) LOC(8) 1

CHOICE="Disabled"

FREE

INIT=SWITCH(9) LOC (8) 0

********COMPLETE EXTRACTING - !VMSB905.CFG - AT END THE PREVIOUS LINE  *******

A P P E N D I X         D

-------------------------

********START EXTRACTING - DIGI_LAST_VERSION.COM - ON THE NEXT LINE  *******

$!DIGI_LAST_VERSION.COM                                    5/19/95 Ver 1.0

$!--------------------------------------------------------------------------!

$! This .COM file is used to at a minimum check that the version of OpenVMS

$! has not changed since the last reboot of the system.  If it has we will

$! assume that the virtual address for the FLOPPY has changed and that

$! this requires regeneration of the 'DIGI_CONNECT_TTn.COM' files that

$! connect the YS$DRIVER to the module.

$!--------------------------------------------------------------------------!

$set noon

$ SAY =  "write sys$output"

$ open/error=openerr input_file  last_version.txt

$!

$ read_loop:

$   read/end_of_file=okay input_file line

$ goto read_loop

$ okay:

$ close input_file

$!       show sym line

$ PREVIOUS_VERSION = line

$       show sym previous_version

$ CURRENT_VERSION = F$GETSYI("VERSION")

$       show sym current_version

$ IF CURRENT_VERSION .NES. PREVIOUS_VERSION

$   THEN GOTO CHANGED

$    ELSE

$ say "  Versions are the same"

$!-----------------------------------------------------------

$! Uncomment and/or modify the following appropriate command

$! line for your configuration.

$!-----------------------------------------------------------

$!! @SYS$MANAGER:DIGI_CONNECT_TTB.COM

$!! @SYS$MANAGER:DIGI_CONNECT_TTC.COM

$ EXIT         ! Exit here if OpenVMS versions are the same.

$ ENDIF

$ OPENERR:

$ say  "  LAST_VERSION.TXT file  has not been created. "

$ CHANGED:

$!

$ say  " ************************************************ "

$ say  " *      You MUST rerun                          * "

$ say  " *                                              * "

$ say  " *          DIGI.COM                            * "

$ say  " *                                              * "

$ say  " *      the previous                            * "

$ say  " *                                              * "

$ say  " *          DIGI_CONNECT_TTn.COM                * "

$ say  " *                                              * "

$ say  " *      files are assumed obsolete, and will    * "

$ say  " *      be renamed 'DIGI_CONNECT_TTn.OBS '.     * "

$ say  " ************************************************ "

$ say  " "

$ purge sys$manager:digi_connect_ttn.obs

$ rename sys$manager:digi_connect_tt*.com.* sys$manager:digi_connect_ttn.obs

$exit

*****  COMPLETE EXTRACTING - DIGI_LAST_VERSION.COM - AT PREVIOUS LINE  ******