#

| README file for

|

| ELSA tool PATCH928.COM, ver. 1.0

|

| Copyright (c) 1993-97 ELSA GmbH, Aachen (Germany)

|

|                                     Subsidary:

| ELSA GmbH                           ELSA Inc.

| Sonnenweg 11                        2150 Trade Zone Blvd.

| D-52070 Aachen                      Suite 101

| Germany                             San Jose, CA 95131

|                                     USA

|

| Phone      : +49-241-606-0          Phone: +1-408-935-0350

| Support Fax: +49-241-606-6399       Phone:  1-800-272-ELSA

| BBS (modem): +49-241-606-9800       Fax  : +1-408-935-0370

| BBS (ISDN) : +49-241-606-9820       BBS  : +1-408-935-0380

| CompuServe : GO ELSA                CIS  : GO ELSA

| WWW        : http://www.elsa.de     WWW  : http://www.elsa.com

|

| 04/23/97, gu, MGroegor

------------------------------------------------------------------------

Summary

This file contains a brief description of the setup utility

PATCH928.COM.

Some versions of the S3 VGA accelerator chip 86C928 are

influenced by a chip bug that affects the video graphics mode

Eh, Dh, 10h, 12h (4bpp, 16 colors), the bug produces fuzzy

horizontal lines in the graphics picture. Mode Eh, Dh, 10h and

12h works very well with most of the software we know, but

there is a small number of computer games (Comanche, Humans,

Lemmings) in which the effect is seen. To get a perfect

picture, type 'PATCH928.COM' before starting one of those

programs. The S3 chip then switches to its 0.5 MB mode, that

means the chip is told that there is only 0.5 MB of video ram

available. In this mode the 86C928 always works troublefree.

If this 0.5 MB mode is no longer necessary, 'PATCH928.COM' to

switch automatic over to the former video mode, since otherwise

all graphics software will produce defective pictures.