# Chapter 6 - FAX API Reference

This chapter provides a complete reference of the Mwave FAX Application Program Interface (API).

## MCI Telephone Event Handler

Communication of real-time status information from the FAX driver to the application is performed through an application event handler. The handler should be able to service messages posted by the FAX driver through the MCI device, which contain real-time status information about the device. The message, MM_MCIEVENT, is not a standard MCI message under Microsoft Windows, thus a Microsoft Windows application must call the RegisterWindowMessage function with the string "MM_MCIEVENT", to obtain the numeric value of the notification message.

MM_MCIEVENT

In addition to the message itself, wParam and lParam are used to pass information to the application.

WPARAM   wParam

Contains a device specific event message wEvent.

LPMCI_EVENT_PARMS    lParam

Specifies a far pointer to the following MCI_EVENT_PARMS structure:

typedef struct {

DWORD	dwDataParam1;

DWORD	dwEventData;

} MCI_EVENT_PARMS;

The data parameters are defined as follows:

DWORD   dwDataParam1

The low-order word specifies the device specific event message wEvent (same as wParam). The high-order word specifies the device ID of the device initiating the message.

DWORD   dwEventData

Contains a data parameter, which is dependent on the message type. The actual parameters passed are listed in Table 1 below, and detailed in the event message descriptions.

MM_MCIEVENT - OS/2

In addition to the message itself, wParam and lParam are used to pass information to the application.

DWORD    MsgParam1

Contains a device-specific event message and device ID.

WORD    wEvent

The low-order word of MsgParam1 specifies the device- specific event message 	(same as usEventCode or wParam)

WORD wDeviceID

The high-order word of MsgParam1 specifies the device ID of the device initiating 	the message.

LPMCI_EVENT_PARMS    MsgParam2

LPMCI_EVENT_PARMS    EventData

Specifies a pointer to the following structure:

typedef struct {

DWORD dwDataParam1;

DWORD dwEventData;

} MCI_EVENT_PARMS;

Note: The low-order word of dwDataParam1 contains the event code (same as 	wEvent).  The high-order word is not defined.

### FAX Event Message Descriptions

This section describes the Event Messages generated by the FAX API. The following table provides a summary of all the Event Messages (wEvent), along with a short description of the data parameter associated with each:

| Table 6-1: FAX Driver Event Messages1 Event Message (wEvent) | Data Parm. (dwEventData) |
| --- | --- |
| PHONE_EVENT_CALL_FAX | undefined |
| PHONE_EVENT_CALL_PROGRESS | New call state |
| PHONE_EVENT_CALL_TERMINATED | Call termination status |
| PHONE_EVENT_CALLER_ID | Caller ID status |
| PHONE_EVENT_DISTINCTIVE_RING | Ring Identifier |
| PHONE_EVENT_FAX_CONNECT | DCS frame information |
| PHONE_EVENT_FAX_HEADER | Pointer to fax header |
| PHONE_EVENT_FAX_PAGE_COMPLETE | Document completion status |
| PHONE_EVENT_FAX_PAGE_STATUS | Page completion status |
| PHONE_EVENT_FAX_POLL | undefined |
| PHONE_EVENT_HANDSET | Handset Status |
| PHONE_EVENT_HANDSET_KEY | Keypress character |
| PHONE_EVENT_LINE | Telephone line status |
| PHONE_EVENT_LINE_KEY | Keypress character |
| PHONE_EVENT_ADVANCED_RING | undefined, use lParam |
| PHONE_EVENT_RING | Telephone ring status |

For all messages posted to the event handler routine, the message value is MM_MCIEVENT. The value of wEvent and dwEventData vary according to the specific message posted. Below is a more detailed description of the event messages and their parameters.

Arguments	wEvent:	PHONE_EVENT_CALL_FAX

dwEventData:	undefined

Description	This message is posted when a call has been answered by the device, and has been determined to have originated from a fax device. At this time, the application that is not doing fax polling should immediately make a call to MCI_RECEIVE to receive any incoming fax data. At this point, the application should expect any of four  additional messages to be posted by the device:

·	PHONE_EVENT_CALLER_ID (If Discriminator running)

·	PHONE_EVENT_FAX_CONNECT

·	PHONE_EVENT_FAX_HEADER

·	PHONE_EVENT_FAX_POLL

These additional messages are documented below.

Note:  If this message is posted then you are guaranteed to get a PHONE_EVENT_CALL_TERMINATED.   At which time, you must do a MCI_FAX_SET_HOOK (ONHOOK).

Arguments	wEvent:	PHONE_EVENT_CALL_PROGRESS

dwEventData:	dwCallProgress

Description	This message is posted when there has been a change in the current call state (or status). The new state of the call is supplied in dwCallProgress, and can be any of the following:

·	DIALTONE

·	ANSWERTONE

·	SLOWBUSY

·	FASTBUSY

·	RINGTONE

·	UNIDENTIFIEDTONE

·	QUIET

·	BUSY

Arguments	wEvent:	PHONE_EVENT_CALL_TERMINATED

dwEventData:	dwTermination

Description	This message is posted when a call has been terminated either by the caller, by the owning application, or because of an error condition. The reason for call termination is given in dwTermination, which may be any of the following values:

·	TERMINATION_ERROR_RECV

·	TERMINATION_ERROR_XMIT

·	TERMINATION_NORMAL

·	TERMINATION_REQUESTED

·	TERMINATION_UNEXPECTED

·	TERMINATION_DISK_FULL

Note:  At this time the application MUST perform a MCI_FAX_SET_HOOK (ONHOOK).

Arguments	wEvent:	PHONE_EVENT_CALLER_ID

dwEventData:	dwCallerId

Description	This message is posted when a caller ID string has been decoded off a ringing line. It is posted only if a caller ID signal is present. dwCallerID indicates the completion status.

·	MCI_VALID_CALLER_ID_RECEIVED

·	MCI_CALLER_ID_FRAME_ERROR

The application must issue an MCI_INFO message to retrieve the id (for MCI_VALID_CALLER_ID_RECEIVED) or the error code (for MCI_CALLER_ID_FRAME_ERROR).

PHONE_EVENT_CALLER_ID is only supported if Discriminator is loaded.

Arguments	wEvent:	PHONE_EVENT_DISTINCTIVE_RING	PL

dwEventData:	dwRingIdentifier

Description	This message is posted when a distinctive ring has been decoded off a ringing line. It is posted only if distinctive ring support is installed. dwRingIdentifier indicates which distinctive ring has been decoded.  The ring identifier is a number between 1 and 20.  This support is added with Ver 3.2.  For FAX it is available only when running the discriminator.

Arguments	wEvent:	PHONE_EVENT_FAX_CONNECT

dwEventData:	dwConnect

Description	This message is posted after a fax call has been answered by the device, and has finished the negotiation period and established the Digital Command Signal (DCS) connection parameters. The dwConnect specifies a far pointer to MCI_FAX_CONNECT_PARMS data structure containing these connection parameters:

typedef struct {

DWORD	dwSignalRate;

DWORD	dwCompression;

DWORD	dwErrorCorrection;

DWORD	dwResolution;

DWORD	dwWidth;

DWORD	dwMinScanLineTime;

}  MCI_FAX_CONNECT_PARMS;

The signal rate is passed in dwSignalRate, and can be any of the following:

·	MCI_FAX_MODEM_V27TER_2400

·	MCI_FAX_MODEM_V27TER_4800

·	MCI_FAX_MODEM_V29_7200

·	MCI_FAX_MODEM_V29_9600

·	MCI_FAX_MODEM_V17_7200

·	MCI_FAX_MODEM_V17_9600

·	MCI_FAX_MODEM_V17_12000

·	MCI_FAX_MODEM_V17_14400

·	MCI_FAX_MODEM_ANY

The following compression types are passed in dwCompression. This message is especially useful if the compression type is BFT (binary file transfer), because in this case, the file resulting from an MCI_RECEIVE is an unencoded binary file.

·	MCI_FAX_COMPRESSION_1D

·	MCI_FAX_COMPRESSION_2D

·	MCI_FAX_COMPRESSION_BFT

The error correction is passed in dwErrorCorrection, and can be either TRUE or FALSE.

The resolution is passed in dwResolution, and can be any of the following:

·	MCI_FAX_RESOLUTION_NORMAL

·	MCI_FAX_RESOLUTION_FINE

The document width in pels is passed in dwWidth.

The device specific minimum milliseconds to scan a line is passed in dwMinScanLineTime.

Arguments	wEvent:	PHONE_EVENT_FAX_HEADER

dwEventData:	lpstrFaxHeader

Description	This message is posted when a fax header string has been decoded off a fax call. It is posted only if a header string is present. An application can use this string to identify the fax sender/receiver. A pointer to the null terminated ASCII string is pointed to by lpstrFaxHeader.

Arguments	wEvent:	PHONE_EVENT_FAX_PAGE_COMPLETE

dwEventData:	dwCompletionStatus

Description	This message is posted when the device has completed either sending or receiving a fax document page. In the event that the device is in the middle of a MCI_SEND, the completion status (measured in percent) is supplied in dwCompletionStatus.

Arguments	wEvent:	PHONE_EVENT_FAX_PAGE_STATUS

dwEventData:	dwPageStatus

Description	This message is posted several times per page during either sending or receiving a fax document. The completion status (measured in percent) is supplied in dwPageStatus for MCI_SEND, and is not supplied for MCI_RECEIVE except for 0% when incoming page is known.

Note:  It is expected behavior to only get a the 0% and 100% on the first page of the outgoing fax.  The reason is due to the low priority of timer messages in Windows.  Subsequent pages should give a % every second.

Arguments	wEvent:	PHONE_EVENT_FAX_POLL

dwEventData:	undefined

Description	This message is posted after a call has been answered by the device, and has been determined to have originated from a fax device and a poll command is received. At this time, the application should immediately make a call to MCI_SEND to send the requested fax data.

Arguments	wEvent:	PHONE_EVENT_HANDSET

dwEventData:	dwHandsetStatus

Description	This message is posted when the status of the telephone handset changes, due to the user either picking up or replacing the telephone handset. The value of dwHandsetStatus is as follows:

dwHandsetStatus = 0	Handset is on-hook

dwHandsetStatus = 1	Handset is off-hook (in use)

The Discriminator must be running to enable receipt of this message.

Arguments	wEvent:	PHONE_EVENT_HANDSET_KEY

dwEventData:	dwKeypress

Description	This message is posted when a key has been pressed on the handset device. The index of the pressed key (0 to 11, 10 for '*' and 11 for '#') is supplied in dwKeypress.

PHONE_EVENT_HANDSET_KEY Is not supported in current FAX driver.

Arguments	wEvent:	PHONE_EVENT_LINE

dwEventData:	dwLineStatus

Description	This message is posted when the status of the telephone line changes, due to another application in the system making use of the telephone line. When an application takes the telephone line off hook, or is called to service an incoming call, it remains in possession of the telephone line for the duration of the call. Applications which require use of the telephone line and find it busy, can simply wait for this message to signal that the telephone line can be used. The value of dwLineStatus is as follows:

dwLineStatus = 0	Telephone line is free

dwLineStatus = 1	Telephone line is in use

The Discriminator must be running to enable receipt of this message.

.

Arguments	wEvent:	PHONE_EVENT_LINE_KEY

dwEventData:	dwKeypress

Description	This message is posted when a key has been pressed on the incoming telephone line. An ASCII character representing the pressed key ('0' - '9', 'a' - 'd', '#' or '*'), is supplied in dwKeypress.

PHONE_EVENT_LINE_KEY is not supported in current FAX driver.

Arguments	wEvent:	PHONE_EVENT_RING

dwEventData:	dwRingStatus

Description	This message is posted when a ring signal change is detected by the device. This message can be used by the application to count ring cycles, or determine ring length. The value of dwRingStatus is as follows:

dwRingStatus = 0	Telephone ring signal end (not ringing)

dwRingStatus = 1	Telephone ring signal start (ringing)

Arguments	wEvent:	PHONE_EVENT_ADVANCED_RING

dwEventData:	not used, actual ring count is in lParam

Description	If the application has requested ‘advanced format ring notifications’ by setting advanced ring notify to TRUE, PHONE_EVENT_ADVANCED_RING is sent to the application instead of PHONE_EVENT_RING.  In this case, lParam is not a pointer to a structure.  Instead, the low word of lParam contains the ring count, and the high word of lParam contains the device ID.

LOWORD(lParam) = 0	Telephone ring signal end (not ringing)

LOWORD(lParam) = ‘n’	Telephone ring count (where ‘n’ is the ring number)

The Discriminator must be running to enable receipt of this message.

## FAX Driver API Messages and Flags

This section describes the MCI compliant FAX API messages and flags. The following table provides a summary of the MCI command messages used in the FAX API, and a short description of each:

| MCI Message | Description |
| --- | --- |
| MCI_CLOSE | Close the device driver |
| MCI_CONVERT | Convert from/to device dependent file to/from device independent file. |
| MCI_DIAL | Dial the telephone |
| MCI_GETDEVCAPS | Get the capabilities of the device |
| MCI_INFO | Get device string identifier |
| MCI_OPEN | Open the device driver |
| MCI_RECEIVE | Receive a fax file |
| MCI_SEND | Send a fax file |
| MCI_SET | Configure the device |
| MCI_STATUS | Query device configuration |
| MCI_STOP | Stop sending or receiving a FAX |

Table 6-2:  FAX Driver API Messages1

MCI_CLOSE

This command message closes the FAX driver.

Parameters	DWORD    lParam1

The following flags apply to the FAX device:

MCI_NOTIFY

Specifies that MCI should post the MM_MCINOTIFY message when this command completes. The window to receive this message is specified in the dwCallback field of the data structure identified by lParam2.

MCI_WAIT

Specifies that the operation should finish before MCI returns control to the application.  The event handler window must be specified in the dwCallback field regardless of whether MCI_NOTIFY or MCI_WAIT is selected.

LPMCI_GENERIC_PARMS   lParam2

Specifies a far pointer to the following MCI_GENERIC_PARMS data structure:

typedef struct {

DWORD		dwCallback;

} MCI_GENERIC_PARMS;

Note: Be sure to assign the handle of the window procedure responsible for processing MM_MCINOTIFY messages to dwCallback prior to calling MCI_CLOSE regardless of whether MCI_WAIT or MCI_NOTIFY is specified.  Failure to do so results in erratic behavior of the Fax device driver when using versions earlier than Ver 2.1 of the Fax device driver

Return Value	Returns zero if successful. Otherwise, it returns an MCI error code.

MCI_CONVERT

This command message is used to convert data files between a MCI device dependent format, and a standard device independent format. The call is used to convert to and from FAX multi-page documents.

Parameters	DWORD    lParam1

The following flags apply to the FAX device:

MCI_NOTIFY

Specifies that MCI should post the MM_MCINOTIFY message when this command completes. The window to receive this message is specified in the dwCallback field of the data structure identified by lParam2.

MCI_WAIT

Specifies that the operation should finish before MCI returns control to the application.

MCI_CONVERT_CREATE

Indicates that the destination file is a new file which should be created. This  overwrites any existing file.

MCI_CONVERT_DESTINATION_FILE

Indicates the lpstrDestFilename field of the data structure identified by lParam2 contains a pointer to a buffer containing the destination file name.

MCI_CONVERT_DESTINATION_FORMAT

Indicates the dwDestFormat field of the data structure identified by lParam2 contains the desired format of the destination file. These include:

·	MCI_CONVERT_FMT_DIB_BMP (from source of type DEVFAX)

·	MCI_CONVERT_FMT_DIB_RLE (from source of type DEVFAX..not supported in current FAX driver)

·	MCI_FAX_CONVERT_FMT_DEVFAX (from DIB_BMP or DIB_RLE.  DIB_RLE conversion not supported in current FAX driver)

MCI_CONVERT_DESTINATION_FROM

Specifies that a media starting position is included in the dwDestFrom field of the data structure identified by lParam2. This specifies the document page (starting at zero) at which the converted data is written to the destination file.

MCI_CONVERT_INFO

Indicates that no conversion operation is to take place, but rather, the dwLength field of the data structure identified by lParam2 should be set to the length of the media of the supplied source device dependent filename. For a device dependent FAX file, the value returned is the document page count. If a device dependent file is not specified, this call returns an error.

MCI_CONVERT_OVERWRITE

Indicates that newly converted information should overwrite any existing data. If this flag is not specified, the new data is inserted into the file.

MCI_CONVERT_SOURCE_FILE

Indicates the lpstrSrcFilename field of the data structure identified by lParam2 contains a pointer to a buffer containing the file name.

MCI_CONVERT_SOURCE_FROM

Specifies that a media starting position is included in the dwSrcFrom field of the data structure identified by lParam2. This specifies the document page (starting at zero) at which the data to be converted is read from the source file.

LPMCI_CONVERT_PARMS   lParam2

Specifies a far pointer to the following MCI_CONVERT_PARMS data structure:

typedef struct {

DWORD		dwCallback;

LPCSTR	lpstrDestFilename;

DWORD		dwDestFormat;

DWORD		dwDestFrom;

DWORD		dwLength;

LPCSTR	lpstrSrcFilename;

DWORD		dwSrcFrom;

} MCI_CONVERT_PARMS;

Return Value	Returns zero if successful. Otherwise, it returns an MCI error code.

MCI_DIAL

This command message takes the phone off-hook, and dials the supplied number. If the telephone is owned by another application at the time of this call, the command will fail.

Parameters	DWORD    lParam1

The following flags apply to the telephone device:

MCI_NOTIFY

Specifies that MCI should post the MM_MCINOTIFY message when this command completes. The window to receive this message is specified in the dwCallback field of the data structure identified by lParam2.

MCI_WAIT

Specifies that the operation should finish before MCI returns control to the application.

MCI_DIAL_DIALMODE

Specifies that the dwDialMode field of the data structure identified by lParam2 contains a constant specifying the phone dialing mode. Two modes are defined:

·	MCI_DIAL_MODE_PULSE

·	MCI_DIAL_MODE_TONE

MCI_DIAL_FLASH

Indicates that the telephone should be flashed before dialing the supplied number (if any).

MCI_DIAL_MONITOR

Specifies that the audio speaker device should be enabled during the calling process.

MCI_DIAL_MONITOR_HANDSHAKING_ONLY

Specifies that the audio speaker device should be enabled only during the negotiation period of the calling process.

MCI_DIAL_STRING

Specifies that the lpstrDialString field of the data structure identified by lParam2 contains a pointer to a null terminated dialing string. Numeric characters '0' to '9' correspond to phone digits.  The '*' and '#' characters, the alpha characters 'a' to 'd'  and the '-' are also supported ('-' is ignored).  The 'w' character in the string specifies that the device should wait for a second dial tone before proceeding, and a ',' character indicates a pause in the dialing sequence. The time-out limit for the wait command (default 30 seconds) and the delay time for the pause command (default 2 seconds) are configurable using MCI_SET. The ‘@’ character in the string specifies wait for quiet.  The ‘p’ character in the string specifies switch to pulse dialing.  The ‘t’ character in the string specifies switch to tone dialing.  The ‘!’ character in the string specifies flash the line

The maximum size string that can be dialed is specified by MAX_DIAL_STRING.

MCI_DIAL_VERIFY

Specifies that the call is to be verified. The phone is verified to be off-hook, and that a dial tone is present before dialing. The correct line type format is also verified.

LPMCI_DIAL_PARMS   lParam2

Specifies a far pointer to the following MCI_DIAL_PARMS data structure:

typedef struct {

DWORD		dwCallback;

DWORD		dwDialMode;

LPCSTR	lpstrDialString;

} MCI_DIAL_PARMS;

Return Value	Returns zero if successful. Otherwise, it returns an MCI error code.

MCI_GETDEVCAPS

This command is used to obtain static information about a device.

Parameters	DWORD    lParam1

The following flags apply to the FAX device:

MCI_NOTIFY

Specifies that MCI should post the MM_MCINOTIFY message when this command completes. The window to receive this message is specified in the dwCallback field of the data structure identified by lParam2.

MCI_WAIT

Specifies that the operation should finish before MCI returns control to the application.

MCI_GETDEVCAPS_ITEM

Specifies that the dwItem field of the data structure identified by lParam2 contains a constant specifying which device capability to obtain. The following constants are defined:

MCI_GETDEVCAPS_CAN_EJECT

The dwReturn field is set to FALSE.

MCI_GETDEVCAPS_CAN_PLAY

The dwReturn field is set to FALSE.

MCI_GETDEVCAPS_CAN_RECORD

The dwReturn field is set to FALSE.

MCI_GETDEVCAPS_CAN_SAVE

The dwReturn field is set to FALSE.

MCI_GETDEVCAPS_COMPOUND_DEVICE

The dwReturn field is set to FALSE.

MCI_GETDEVCAPS_DEVICE_TYPE

The dwReturn field is set to MCI_DEVTYPE_OTHER.

MCI_GETDEVCAPS_HAS_AUDIO

The dwReturn field is set to FALSE.

MCI_GETDEVCAPS_HAS_VIDEO

The dwReturn field is set to FALSE.

MCI_GETDEVCAPS_USES_FILES

The dwReturn field is set to TRUE.

MCI_FAX_GETDEVCAPS_COMPRESSION_TYPES

The dwReturn field is set to the logical ORing of the following supported compression types:

·	MCI_FAX_COMPRESSION_MH

·	MCI_FAX_COMPRESSION_MR

·	MCI_FAX_COMPRESSION_MMR

·	MCI_FAX_COMPRESSION_NONE

·	MCI_FAX_COMPRESSION_BFT

Only MCI_FAX_COMPRESSION_MH supported in current FAX driver.

MCI_FAX_GETDEVCAPS_CAN_RECEIVE

The dwReturn field is set to TRUE if the device supports receiving FAX file data from the telephone line. Otherwise, it is set to FALSE.

MCI_FAX_GETDEVCAPS_CAN_SEND

The dwReturn field is set to TRUE if the device supports sending FAX file data to the telephone line. Otherwise, it is set to FALSE.

MCI_FAX_GETDEVCAPS_HAS_HANDSET

The dwReturn field is set to TRUE if the device supports call monitoring through an external handset; otherwise, it returns FALSE.

MCI_FAX_GETDEVCAPS_HAS_HANDSET not supported in current FAX driver.

MCI_FAX_GETDEVCAPS_MODEM_TYPES

The dwReturn field is set to the logical ORing of the following supported modem types:

·	MCI_FAX_MODEM_V27TER_2400

·	MCI_FAX_MODEM_V27TER_4800

·	MCI_FAX_MODEM_V29_7200

·	MCI_FAX_MODEM_V29_9600

·	MCI_FAX_MODEM_V17_7200

·	MCI_FAX_MODEM_V17_9600

·	MCI_FAX_MODEM_V17_12000

·	MCI_FAX_MODEM_V17_14400

MCI_FAX_GETDEVCAPS_POLLING

The dwReturn field is set to TRUE if FAX polling is supported, and FALSE if not.

MCI_FAX_GETDEVCAPS_RESOLUTION

The dwReturn field is set to the resolution of the device.

·	MCI_FAX_RESOLUTION_FINE        200x200 PIXELS/INCH

·	MCI_FAX_RESOLUTION_NORMAL 100x200 PIXELS/INCH

MCI_FAX_GETDEVCAPS_SUPPORTS_ECM

The dwReturn field is set to TRUE if FAX ECM is supported, and FALSE if not.

MCI_FAX_GETDEVCAPS_FILE_FORMATS

File formats supported for fax send/receive. The dwReturn field is set to logical ORing of the following file formats:

·	TIFF_CLASS_F

·	DCX

·	RIFF

·	TIFF_6.0

MCI_FAX_GETDEVCAPS_WIDTH

The dwReturn field is set to the width in pels of the device.

MCI_FAX_GETDEVCAPS_WIDTH not supported in current FAX driver.

LPMCI_GETDEVCAPS_PARMS   lParam2

Specifies a far pointer to the following MCI_GETDEVCAPS_PARMS data structure:

typedef struct {

DWORD		dwCallback;

DWORD		dwReturn;

DWORD		dwItem;

} MCI_GETDEVCAPS_PARMS;

Return Value	Returns zero if successful. Otherwise, it returns an MCI error code.

MCI_INFO

This command message obtains string information from the device.

Parameters	DWORD    lParam1

The following flags apply to the FAX device:

MCI_NOTIFY

Specifies that MCI should post the MM_MCINOTIFY message when this command completes. The window to receive this message is specified in the dwCallback field of the data structure identified by lParam2.

MCI_WAIT

Specifies that the operation should finish before MCI returns control to the application.

MCI_INFO_PRODUCT

Obtains a description of the hardware associated with a device. The description identifies both the driver and the hardware used. The string is copied to the buffer pointer to by the lpstrReturn field of the structure identified by lParam2. The size of this buffer is specified by the dwRetSize field of the same structure, and if the buffer is of insufficient size to contain the string, the string is truncated to fit the buffer.  The string contains a version number (i.e, “Ver 3.0”).  Driver enhancements will be denoted in this document with the “Ver x.y”. that corresponds with the first release that the feature shows up in.  The version number will always increase in future releases, so a program can parse the string, looking for “Ver “, convert the characters that follow “Ver “ to a number, and do a numeric greater-than-or-equal compare to determine if the function is available in the release the application is running with.

Note:  Unless otherwise noted, all functions are available as of Ver 2.2

MCI_INFO_CALLER_ID

Obtains a caller ID string.  (See PHONE_EVENT_CALLER_ID).  The caller ID data is copied into the buffer pointed to by the lpstrReturn field of the structure identified by lParam2. The size of this buffer is specified by the dwRetSize field of the same structure (maximum size = MCI_MAX_CALLER_ID_SIZE).  If the buffer is of insufficient size to contain the data, the data is truncated to fit the buffer, the return code is set to MCIERR_INVALID_BUFFER, and the dwRetSize is set to the size needed to retrieve the entire caller ID buffer.

Note: The caller ID data is in the format defined by Bellcore’s technical reference bulletin TR-TSY-000031 and TR-NWT-001188.  Also note that a checksum is included at the end of the caller ID data.

MCI_INFO_CALLER_ID is only supported when the discriminator is loaded.

MCI_INFO_CALLER_ID_ERROR

Obtains the caller ID error code. (See PHONE_EVENT_CALLER_ID). The code is copied into the buffer pointed to by the lpstrReturn field of the structure identified by lParam2. The size of this buffer is specified by the dwRetSize field of the same structure.  The error code is either MCI_CHECKSUM_ERROR or MCI_FRAME_ERROR.

MCI_INFO_CALLER_ID_ERROR is only supported when the discriminator is loaded.

MCI_INFO_CALLER_PARSED_CALLER_ID

Obtains a caller an already-parsed Caller ID string.  (See PHONE_EVENT_CALLER_ID).  The information is copied into the structure pointed to by the lpstrReturn (Windows) or dwReturn (OS/2) field of the structure identified by lParam2. The structure is:

typedef struct

{

char szDateTime[DATE_TIME_LEN+1];

char szNumber[MCI_MAX_CALLER_ID_SIZE]; /* callers number */

char szName[MCI_MAX_CALLER_ID_SIZE];    /* callers name (may      	                                                                                        be null) */

} CIDINFO;

This function is implemented in “Ver 3.0” of the TAM driver.

MCI_INFO_CALLER_PARSED_CALLER_ID is only supported when the discriminator is loaded.

LPMCI_INFO_PARMS   lParam2

Specifies a far pointer to the following MCI_INFO_PARMS data structure:

typedef struct {

DWORD		dwCallback;

LPSTR		lpstrReturn;

DWORD		dwRetSize;

} MCI_INFO_PARMS;

typedef struct {

DWORD		dwCallback;

LPSTR		lpstrReturn;

DWORD		dwRetSize;

} MCI_INFO_PARMS;              /*OS/2*/

Return Value	Returns zero if successful. Otherwise, it returns an MCI error code.

MCI_OPEN

This command message initializes the telephony driver and hardware.

Parameters	DWORD    lParam1

The following flags apply to the FAX device:

MCI_NOTIFY

Specifies that MCI should post the MM_MCINOTIFY message when this command completes. The window to receive this message is specified in the dwCallback field of the data structure identified by lParam2.

MCI_WAIT

Specifies that the operation should finish before MCI returns control to the application.  In older versions of the driver, the event handler window must be specified in the dwCallback field regardless of whether MCI_NOTIFY or MCI_WAIT is selected.

MCI_OPEN_ALIAS

Specifies that an alias is included in the lpstrAlias field of the data structure identified by lParam2. This command is handled by MCI.

MCI_OPEN_SHAREABLE

Specifies that the device should be opened as shareable.

MCI_OPEN_SHAREABLE is not supported in current FAX driver.

MCI_OPEN_TYPE

Specifies that a device type name or constant is included in the lpstrDeviceType field of the data structure identified by lParam2. To open the fax driver, specify “Mwavefax” in the lpstrDeviceType. This command is handled by MCI.

MCI_OPEN_TYPE_ID

Specifies that the low-order word of the lpstrDeviceType field of the associated data structure contains a standard MCI device type ID and the high-order word optionally contains the ordinal index for the device. This command is handled by MCI.

LPMCI_OPEN_PARMS   lParam2

Specifies a far pointer to the following MCI_OPEN_PARMS data structure:

typedef struct {

DWORD		dwCallback;

WORD		wDeviceID;

WORD		wReserved0;

LPCSTR	lpstrDeviceType;

LPCSTR	lpstrElementName;

LPCSTR	lpstrAlias;

} MCI_OPEN_PARMS;

Note:	With Microsoft Windows, be sure to assign the handle of the window procedure responsible for processing MM_MCINOTIFY messages to dwCallback prior to calling MCI_OPEN regardless of whether MCI_WAIT or MCI_NOTIFY is specified.  Failure to do so results in erratic behavior when using versions earlier than Ver 2.1 of the Fax device driver.

Return Value	Returns zero if successful. Otherwise, it returns an MCI error code.

Remarks

·	Case is ignored in the device name, but there must not be any leading or trailing blanks.

·	Note that the device type is the pszDeviceType field of the MCI_OPEN_PARMS data structure, but it does not have a corresponding flag because it is required and does not have a command-string parameter.

For the Mwave Fax and TAM drivers, the device types are:

Mwavetpl

Mwavetps

Mwavefax

·	OS/2 only: If automatic type selection is desired (through the extensions or EA section or INI), the file name (including the extension) must be passed in the pszElementName parameter, the pszDeviceType is left null, and the MCI_OPEN_ELEMENT flag is set.

MCI_RECEIVE

This command message receives a file. In the case of FAX, this file is an OEM dependent FAX file consisting of one or more image pages. The number of pages actually received is available in MCI_STATUS.

Parameters	DWORD    lParam1

The following flags apply to the FAX device:

MCI_NOTIFY

Specifies that MCI should post the MM_MCINOTIFY message when this command completes. The window to receive this message is specified in the dwCallback field of the data structure identified by lParam2.

MCI_WAIT

Specifies that the operation should finish before MCI returns control to the application.

MCI_RECEIVE_FILE

Indicates the lpstrFilename field of the data structure identified by lParam2 contains a pointer to a buffer containing the file name where the received fax data is to be stored.

MCI_ALREADY_DIALED

Indicates the document is to be received immediately because the application has already connected to the partner fax machine.  In FAX vernacular, this is often referred to as Manual Receive.

LPMCI_RECEIVE_PARMS   lParam2

Specifies a far pointer to the following MCI_RECEIVE_PARMS data structure:

typedef struct {

DWORD		dwCallback;

LPCSTR	lpstrFilename;

} MCI_RECEIVE_PARMS;

Return Value	Returns zero if successful. Otherwise, it returns an MCI error code.

MCI_SEND

This command message sets up a document or documents for sending, which then takes place during a following MCI_DIAL command message. In the case of FAX, this file is an OEM dependent FAX file or files consisting of one or more image pages. The number of pages sent can be obtained via MCI_STATUS.

Parameters	DWORD    lParam1

The following flags apply to the FAX device:

MCI_NOTIFY

Specifies that MCI should post the MM_MCINOTIFY message when this command completes. The window to receive this message is specified in the dwCallback field of the data structure identified by lParam2.

MCI_WAIT

Specifies that the operation should finish before MCI returns control to the application.

MCI_SEND_FILE

Indicates the lpstrFilename field of the data structure identified by lParam2 contains an array of pointers to pointers to strings identifying the file name of each FAX file to send. The lpstrFilename array is terminated with a NULL string pointer to indicate the end of the file name list.

MCI_FAX_SEND_SINGLE_FILE

Indicates the lpstrFilename field of the data structure identified by lParam2 contains a string identifying the file name of the FAX file to send.

MCI_ALREADY_DIALED

Indicates the document is to be sent immediately because the application has already connected to the partner fax machine. In FAX vernacular, this is often referred to as Manual Send.

MCI_SEND_HEADING

Indicates the lpstrHeading field of the data structure identified by lParam2 contains a string identifying the full path and file name of the heading file.  The heading file must be in Tiff Class F format, single strip.  Each heading should be a tiff page.

Note:  The heading file:  should contain a heading for every page to be sent, must have the same fill order and resolution as the page being sent with it, and must be less than 24K.

This function is implemented in “Ver 3.0” of the FAX driver

LPMCI_SEND_PARMS   lParam2

Specifies a far pointer to the following MCI_SEND_PARMS data structure:

typedef struct {

DWORD		dwCallback;

LPCSTR	lpstrFilename[];

LPSTR       lpstrHeading;

} MCI_SEND_PARMS;

Return Value	Returns zero if successful. Otherwise, it returns an MCI error code.

MCI_SET

This command is used to set the FAX device configuration. This configuration determines the environment used to send Fax Document Files. The item to set is specified by dwItem field of the MCI_FAX_SET_PARMS structure, pointed to by lParam2, and set data information is passed in dwSetData.

Parameters	DWORD    lParam1

The following flags apply to the FAX device:

MCI_NOTIFY

Specifies that MCI should post the MM_MCINOTIFY message when this command completes. The window to receive this message is specified in the dwCallback field of the data structure identified by lParam2.

MCI_WAIT

Specifies that the operation should finish before MCI returns control to the application.

MCI_SET_ITEM

Specifies that the dwItem field of the data structure identified by lParam2 contains a constant specifying which item to set. The following constants are defined:

MCI_FAX_SET_ADVANCED_RING_NOTIFY

The dwSetData field is set to the indicate the type of message that is sent to the application when the phone rings. When set to FALSE (the default), a PHONE_EVENT_RING is sent to the application. When the flag is set to TRUE, a PHONE_EVENT_ADVANCED_RING is sent to the application.  With advanced format ring events, lParam does not contain a pointer to dwSetData.  Instead, lParam contains the device ID and the actual ring count (not a pointer to it).  A ring count of zero signifies the end of a ring.

MCI_FAX_SET_ADVANCED_RING_NOTIFY is not supported in current driver.

MCI_FAX_SET_API_STYLE

Specifies that the dwSetData field of the data structure identified by lParam2 contains the API style of the FAX device.  The possible values are:

·	MCI_FAXTAM_STYLE_MMPM

·	MCI_FAXTAM_STYLE_WINDOWS

The default style under Windows is WINDOWS. The default style under OS/2 is MMPM. The API style  affects return codes for MCI_STATUS, MCI_GETDEVCAPS, and MCI_INFO.  The style also affects return codes and return    information for MM_MCINOTIFY.  See Microsoft Windows Multimedia Programmer’s Reference and IBM’s Programming Reference for Multimedia Presentation Manager Toolkit/2 for details of the MCI interface as specified for Windows and OS/2.

MCI_FAX_SET_AUDIO_VOLUME

Specifies that the dwSetData field of the data structure identified by lParam2 contains a constant specifying the volume level of the speaker device. The volume level is specified from 0x0 (silence) to 0xFFFF (maximum volume) and is interpreted logarithmically. This means the perceived volume increase is the same when increasing the volume level from 0x5000 to 0x6000 as it is from 0x4000 to 0x5000.

MCI_FAX_SET_CALL_FILTER

Specifies that the dwSetData field of the data structure identified by lParam2 is set to TRUE if the device is to receive fax calls; otherwise it is set to FALSE. If another application has this filer enabled, attempting to enable the filter causes an error return.

MCI_FAX_SET_COMPRESSION_TYPES

Specifies that the dwSetData field of the data structure identified by lParam2 contains the allowable FAX compression type(s) for files to be received. The following type flags are defined:

·	MCI_FAX_COMPRESSION_MH

·	MCI_FAX_COMPRESSION_MR

·	MCI_FAX_COMPRESSION_MMR

·	MCI_FAX_COMPRESSION_NONE

·	MCI_FAX_COMPRESSION_BFT

·	MCI_FAX_COMPRESSION_ANY

Only MH compression type supported in current FAX driver

MCI_FAX_SET_DIAL_FLASH_TIME

The dwSetData field is set to the desired flash time (in milliseconds) of the telephone flash option in the MCI_DIAL command. The default value is 500 (one half second).

MCI_FAX_SET_DIAL_PAUSE_TIME

The dwSetData field is set to the desired pause time (in milliseconds) that an embedded ',' character produces in the dial string. The default value is 2000 (2 seconds).

MCI_FAX_SET_DIAL_WAIT_TIME

The dwSetData field is set to the desired time-out limit (in milliseconds) that an embedded 'w' character in the dial string allows, waiting for a second dial tone. The default value is 30000 (30 seconds).

MCI_FAX_SET_ECM_LEVEL

Specifies that the dwSetData field of the data structure identified by lParam2 contains the current Error Correction Mode (ECM) quality level. The higher quality telephone lines require less rigorous ECM checking. The following line quality levels are defined:

·	MCI_FAX_ECM_POOR_LINE

·	MCI_FAX_ECM_AVERAGE_LINE

·	MCI_FAX_ECM_QUALITY_LINE

·	MCI_FAX_ECM_NONE

MCI_FAX_SET_ECM_LEVEL not supported in current FAX driver

.

MCI_FAX_SET_EVENT_HANDLER

Specifies that the dwSetData field of the data structure identified by lParam2 contains the handle of the application event handler. The MCI driver posts MM_MCIEVENT messages when an event occurs which changes the status of the driver. Setting this value to zero disables event posting. See the event handler section of the document for more details.

MCI_FAX_SET_HOOK

The dwSetData field is set to the desired hook status of the telephone line. It is set to TRUE to take the handset off-hook, and FALSE to place the handset on-hook. If another application owns the phone line, this call will fail. When an application sets dwSetData to FALSE, it relinquishes ownership of the line.

MCI_FAX_SET_MODEM_TYPES

Specifies that the dwSetData field of the data structure identified by lParam2 contains the desired maximum and minimum FAX modem types ORed together for calls received and transmitted.  If the actual negotiation speed is lower than the selected minimum modem type, the call is terminated. The following modem type flags are defined (in fall back order; from highest speed to lowest):

·	MCI_FAX_MODEM_ANY

·	MCI_FAX_MODEM_V17_14400

·	MCI_FAX_MODEM_V17_12000

·	MCI_FAX_MODEM_V17_9600

·	MCI_FAX_MODEM_V17_7200

·	MCI_FAX_MODEM_V29_9600

·	MCI_FAX_MODEM_V29_7200

·	MCI_FAX_MODEM_V27TER_4800

·	MCI_FAX_MODEM_V27TER_2400

MCI_FAX_MODEM_ANY not supported in current FAX driver

MCI_FAX_SET_PASS_CALL

Specifies that the dwSetData field of the data structure identified by lParam2 contains a constant specifying the device to which the phone line should be passed.  The line can only be passed from the fax driver when the MODE is OPEN (see MCI_STATUS_MODE). If the mode is not open, the application must do a MCI_STOP to reset the fax out of send or receive mode.

The possible values of dwSetData are:

·	MCI_FAXTAM_PASS_VOICE

·	MCI_FAXTAM_PASS_MODEM

MCI_FAX_SET_PASS_CALL is only supported when the discriminator is loaded.

MCI_FAXTAM_PASS_MODEM not supported in current  driver.

MCI_FAX_SET_POLLING

Specifies that the dwSetData field of the data structure identified by lParam2 is set to TRUE if the device is to be set to receive a FAX poll for this application. Both calling and called applications must issue this command followed by an MCI_RECEIVE to set up for polling.

MCI_FAX_SET_RESOLUTION

Specifies that the dwSetData field of the data structure identified by lParam2 contains the resolution mode of the FAX device. This setting is used to tell the calling party the fax device’s capabilities (DIS info) for negotiating the receive.  The possible values are:

·	MCI_FAX_RESOLUTION_NORMAL

·	MCI_FAX_RESOLUTION_FINE

MCI_FAX_SET_RING_COUNT

Specifies that the dwSetData field of the data structure identified by lParam2 contains a constant specifying the ring count at which the device should answer the telephone. The default ring count for FAX is 1.

If the discriminator is loaded, it will answer the telephone on the shortest ring count request of all registered applications (Windows), but never on less than two rings (OS/2).  Caller ID can arrive between rings 1 and 2.

The maximum ring count that can be set is specified by MAX_RING_COUNT.

Note:  If application is providing homologation support see MCI_STATUS for more information on the min and max ring count allowable.

MCI_FAX_SET_STATION_ID

Specifies that the dwSetData field of the data structure identified by lParam2 contains a pointer to a null terminated character string which gives the station identifier that is sent by the device during negotiation.

LPMCI_FAX_SET_PARMS   lParam2

Specifies a far pointer to the following MCI_FAX_SET_PARMS data structure:

typedef struct {

DWORD		dwCallback;

DWORD		dwSetData;

DWORD		dwItem;

} MCI_FAX_SET_PARMS;

Return Value	Returns zero if successful. Otherwise, it returns an MCI error code.

MCI_STATUS

This command is used to obtain information about the FAX device configuration. Information is returned in the dwReturn field of the MCI_STATUS_PARMS structure, pointed to by lParam2.

Parameters	DWORD    lParam1

The following flags apply to the FAX device:

MCI_NOTIFY

Specifies that MCI should post the MM_MCINOTIFY message when this command completes. The window to receive this message is specified in the dwCallback field of the data structure identified by lParam2.

MCI_WAIT

Specifies that the operation should finish before MCI returns control to the application.

MCI_STATUS_ITEM

Specifies that the dwItem field of the data structure identified by lParam2 contains a constant specifying which status item to obtain. The following constants are defined:

MCI_STATUS_LENGTH

The dwReturn field is set to the number of pages of the last Fax sent or received.

MCI_STATUS_MODE

The dwReturn field is set to the current mode of the device. The following modes are defined:

·	MCI_MODE_NOT_READY

·	MCI_MODE_OPEN

·	MCI_MODE_RECEIVE

·	MCI_MODE_SEND

MCI_STATUS_POSITION

The dwReturn field is set to the current number of pages received or sent.

MCI_STATUS_READY

The dwReturn field is set to TRUE if the device is ready; otherwise, it is set to FALSE. If another telephony application has ownership of the telephone line, this status command  returns FALSE.

MCI_STATUS_TIME_FORMAT

The dwReturn field is set to the time format of the play/record media. This always returns MCI_FAX_FORMAT_PAGES.

MCI_FAX_STATUS_AUDIO_VOLUME

The dwReturn field of the data structure identified by lParam2 returns a constant specifying the volume level of the speaker device. The volume level is specified from 0x0 (silence) to 0xFFFF (maximum volume) and is interpreted logarithmically. This means the perceived volume increase is the same when increasing the volume level from 0x5000 to 0x6000 as it is from 0x4000 to 0x5000.

MCI_FAX_STATUS_CALL_FILTER

The dwReturn field of the data structure identified by lParam2 is set to TRUE if the device is currently set to receive fax calls; otherwise it is set to FALSE.

MCI_FAX_STATUS_COMPRESSION_TYPES

Specifies that the dwReturn field of the data structure identified by lParam2 is set to the allowable FAX compression type(s) for calls received and sent. The following type flags are defined:

·	MCI_FAX_COMPRESSION_MH

·	MCI_FAX_COMPRESSION_MR

·	MCI_FAX_COMPRESSION_MMR

·	MCI_FAX_COMPRESSION_NONE

·	MCI_FAX_COMPRESSION_BFT

·	MCI_FAX_COMPRESSION_ANY

Only MH compression type supported in current FAX driver.

MCI_FAX_STATUS_DIAL_FLASH_TIME

The dwReturn field is set to the current flash time (in milliseconds) of the telephone flash option in the MCI_DIAL command.

MCI_FAX_STATUS_DIAL_PAUSE_TIME

The dwReturn field is set to the current pause time (in milliseconds) that an embedded ',' character produces in the dial string.

MCI_FAX_STATUS_DIAL_WAIT_TIME

The dwReturn field is set to the current time-out limit (in milliseconds) that an embedded 'w' character in the dial string allows, waiting for a second dial tone.

MCI_FAX_STATUS_ECM_LEVEL

Specifies that the dwReturn field of the data structure identified by lParam2 is set to the current ECM quality level. The higher quality telephone lines require less rigorous ECM checking. The following line quality levels are defined:

·	MCI_FAX_ECM_POOR_LINE

·	MCI_FAX_ECM_AVERAGE_LINE

·	MCI_FAX_ECM_QUALITY_LINE

·	MCI_FAX_ECM_NONE

MCI_FAX_STATUS_ECM_LEVEL not supported in current FAX driver.

MCI_FAX_STATUS_HANDSET

The dwReturn field is set to the current status of the telephone handset.  It is set to TRUE if the handset is off-hook; otherwise, it is set to FALSE.

MCI_FAX_STATUS_HANDSET not supported in current FAX driver.

MCI_FAX_STATUS_HANDSET_VOLUME

The dwReturn field of the data structure identified by lParam2 returns a constant specifying the volume level of the speaker device.  The volume level is specified from 0x0 (silence) to 0xFFFF (maximum volume).  The perceived volume increase is the same when increasing the volume level from 0x5000 to 0x6000 as it is from 0x4000 to 0x5000.

MCI_FAX_STATUS_HANDSET_VOLUME not supported in current FAX driver.

MCI_FAX_STATUS_HOOK

The dwReturn field is set to the current hook status of the telephone line. It is set to TRUE if the phone is off-hook; otherwise, it is set to FALSE.

MCI_FAX_STATUS_LINE

The dwReturn field is set to the current phone line status. The following status modes are defined:

·	MCI_FAX_LINE_ONHOOK

·	MCI_FAX_LINE_DIALTONE

·	MCI_FAX_LINE_BUSY

·	MCI_FAX_LINE_RINGTONE

·	MCI_FAX_LINE_FAX_CARRIER

·	MCI_FAX_LINE_UNKNOWN

MCI_FAX_STATUS_MAX_MODEM_SPEED

Specifies that the dwReturn field of the data structure identified by lParam2 is set to the highest speed FAX modem type desired for calls received and sent. The following modem type flags are defined (in order from highest speed to lowest):

·	MCI_FAX_MODEM_ANY

·	MCI_FAX_MODEM_V17_14400

·	MCI_FAX_MODEM_V17_12000

·	MCI_FAX_MODEM_V17_9600

·	MCI_FAX_MODEM_V17_7200

·	MCI_FAX_MODEM_V29_9600

·	MCI_FAX_MODEM_V29_7200

·	MCI_FAX_MODEM_V27TER_4800

·	MCI_FAX_MODEM_V27TER_2400

MCI_FAX_MODEM_ANY not supported in current FAX driver

MCI_FAX_STATUS_MIN_MODEM_SPEED

Specifies that the dwReturn field of the data structure identified by lParam2 is set to the lowest speed FAX modem type desired for calls received and sent. The following modem type flags are defined (in order from lowest speed to highest):

·	MCI_FAX_MODEM_ANY

·	MCI_FAX_MODEM_V17_14400

·	MCI_FAX_MODEM_V17_12000

·	MCI_FAX_MODEM_V17_9600

·	MCI_FAX_MODEM_V17_7200

·	MCI_FAX_MODEM_V29_9600

·	MCI_FAX_MODEM_V29_7200

·	MCI_FAX_MODEM_V27TER_4800

·	MCI_FAX_MODEM_V27TER_2400

MCI_FAX_MODEM_ANY not supported in current FAX driver

MCI_FAX_STATUS_POLLING

Specifies that the dwReturn field of the data structure identified by lParam2 is set to TRUE if the device is set for fax polling for this application. Otherwise this value is FALSE.

MCI_FAX_STATUS_RESOLUTION

Specifies that the dwReturn field of the data structure identified by lParam2 is set to the resolution mode of the FAX device. The possible return values are:

·	MCI_FAX_RESOLUTION_NORMAL

·	MCI_FAX_RESOLUTION_FINE

MCI_FAX_STATUS_RING_COUNT

The dwReturn field is set to a constant specifying the ring count at which the device answers the telephone. The driver answers on the shortest ring count request of all active applications, so this value might not match the value specified in MCI_SET.

MCI_FAX_STATUS_STATION_ID

The dwReturn field of the data structure identified by lParam2 contains a pointer to a null terminated character string containing the station identifier.

MCI_FAX_STATUS_WORLDTRADE_SUPPORT

The dwReturn field is set to a binary encoded set of values indicating restrictions that are in effect for the current country.  Some of the bit settings require the application to make a subsequent MCI_STATUS call to determine a maximum value. This support is added with driver version 3.4. The defined bits include:

·  PULSE_DIAL_NOT_ALLOWED is set TRUE if pulse

dialing is not supported.

·  DTMF_DIAL_NOT_ALLOWED is set TRUE if DTMF

dialing is not supported.

·  BUSYTONE_DETECT_NOT_VALID is set TRUE if

busy tone detection is not available in the country.

·  BUSYTONE_DETECT_REQUIRED is set TRUE if

busy tone detection is required in country.

·  DIALTONE_DETECT_NOT_VALID is set TRUE if dial

tone detection is not available in the country.

·  DIALTONE_DETECT_REQUIRED is set TRUE if dial

tone detection is required in country.

MCI_FAX_STATUS_COUNTRY_CODE

The dwReturn field is set to the current country code.    This can be used by applications that must change the looks of the user interface for different countries like a French keypad in France. This support is added with driver version 3.4.  The following table shows the codes assigned to each country:

| COUNTRY | CODE |  | COUNTRY | CODE |  | COUNTRY | CODE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| USA/Canada | 1 |  | Australia | 14 |  | Norway | 27 |
| Belgium | 2 |  | Austria | 15 |  | Denmark | 28 |
| Hong Kong | 3 |  | Mexico | 16 |  | France | 29 |
| Singapore | 4 |  | South Africa | 17 |  | Netherlands | 30 |
| New Zealand | 5 |  | Chile | 18 |  | U. K. | 31 |
| Japan | 6 |  | Switzerland | 19 |  | Sweden | 32 |
| Portugal | 7 |  | Germany | 20 |  | Italy | 33 |
| Ireland | 8 |  | Brazil | 21 |  | Finland | 34 |
| Generic | 9 |  | Russia | 22 |  | Thailand | 35 |
| Spain | 10 |  | Yugoslavia | 23 |  | Korea | 36 |
| Greece | 11 |  | Hungary | 24 |  | Malaysia | 37 |
| Israel | 12 |  | Czechrepublic | 25 |  | PRC | 38 |
| Taiwan | 13 |  | Luxembourg | 26 |  | Slovakia | 39 |
|  |  | TABLE 6-3: Country Codes |  |  |  |  |  |

MCI_FAX_STATUS_AUTO_ANSWER_MIN_RINGS

The dwReturn field contains the minimum number of rings that can be set in MCI_FAX_SET_RING_COUNT. This support is added with driver version 3.4.

MCI_FAX_STATUS_AUTO_ANSWER_MAX_RINGS

The dwReturn field contains the maximum number of rings can be set in MCI_FAX_SET_RING_COUNT.  If the value is '7FFF'x then there is no limit in that country. This support is added with driver version 3.4.

MCI_FAX_STATUS_MAX_CALL_RETRIES

The dwReturn field contains the maximum number of unsuccessful retries allowed.  If the value is '7FFF'x there is no max in that country. This support is added with driver version 3.4.

MCI_FAX_STATUS_MIN_CALL_RETRY_TIME

The dwReturn field contains the minimum time allowed between retries. This support is added with driver version 3.4.

LPMCI_STATUS_PARMS   lParam2

Specifies a far pointer to the following MCI_STATUS_PARMS data structure:

typedef struct {

DWORD		dwCallback;

DWORD		dwReturn;

DWORD		dwItem;

} MCI_STATUS_PARMS;

Return Value	Returns zero if successful. Otherwise, it returns an MCI error code.

MCI_STOP

This command is used to cancel a fax send or a fax receive.

Parameters	DWORD    lParam1

The following flags apply to the FAX device:

MCI_NOTIFY

Specifies that MCI should post the MM_MCINOTIFY message when this command completes. The window to receive this message is specified in the dwCallback field of the data structure identified by lParam2.

MCI_WAIT

Specifies that the operation should finish before MCI returns control to the application.

LPMCI_GENERIC_PARMS   lParam2

Specifies a far pointer to the following MCI_GENERIC_PARMS data structure:

typedef struct {

DWORD		dwCallback;

} MCI_GENERIC_PARMS;

Note: It is necessary to wait for PHONE_EVENT_CALL_TERMINATED before hanging up the phone.

Return Value	Returns zero if successful. Otherwise, it returns an MCI error code.