# Chapter 1 - Introduction

This chapter provides an overview of the Mwave multimedia system and software environment, and briefly describes how to get started integrating Mwave multimedia capabilities into your OS/2 or Microsoft Windows 3.1 application.

Most of the Application Programmer’s Guide is applicable to both OS/2 and to Windows.  In those cases where a difference exists, the text will say so explicitly or the OS/2-specific text will be shaded.

## Mwave System Overview

The Mwave system is a programmable DSP (Digital Signal Processor) based hardware and software platform designed specifically to handle the demands of multimedia in the desktop PC environment. A single Mwave system can integrate a variety of multimedia capabilities such as audio, speech, FAX, modem, and Telephone Answering Machine (TAM).

OS/2 and Microsoft Windows provide high-level and low-level services which enable an application developer to take advantage of the extended capabilities of a multimedia PC.  By providing Mwave Application Programming Interfaces (APIs) which are compatible with OS/2 MMPM and  Microsoft Windows multimedia services, the application developer is able to develop powerful, portable applications which utilize the multimedia capabilities offered by a wide range of Mwave products.

This manual describes the Mwave APIs and how to use them to develop Mwave multimedia applications for OS/2 and Microsoft Windows 3.1.

Notice

This material is being made available to enable software developers to produce digital signal processing applications, device drivers, and tasks of MwaveÔ Technology Platforms.  It is not intended to enable others to provide the services of the applications interfaces described herein, rather to enable others to interface to these services.

The following figure illustrates the Mwave runtime software environment:

Application

Programming

Interface

Figure 1-1:  Mwave Runtime Software Environment

Mwave Virtual

Hardware Tasks

Mwave Operating

System

Mwave DSP

DSP Code

PC Host Code

Mwave Manager

Mwave Device Driver

Mwave Application Program

The following functional blocks comprise the Mwave runtime software environment.

| Mwave Application Program | The application program communicates to a device driver through standardized APIs, thus performing a variety of multimedia tasks on the Mwave platform. |
| --- | --- |
| Mwave Device Drivers | These MCI compliant drivers provide the software interface between the application program and the Mwave manager, enabling a single Mwave application to run on a variety of Mwave platforms. |
| Mwave Manager | This host-based software manages DSP resources and provides a hardware-independent interface layer between the Mwave Device Drivers and the underlying Mwave hardware. |
| Mwave Operating System | Real-time, multitasking DSP kernel that allows concurrent processing of virtual hardware tasks. |
| Mwave DSP | High-performance DSP optimized for the demands of multimedia applications. |
| Mwave Virtual Hardware Tasks | DSP-optimized software library that emulates multimedia hardware components such as audio, speech, FAX, and communications. |

## Developing an Mwave Application

Before writing an Mwave multimedia application, you should have a working knowledge of the following:

·	Programming in the Microsoft Windows environment

·	Understand the high-level and low-level multimedia services provided in OS/2 Multimedia Presentation Manager/2 (MMPM/2) or Microsoft Windows 3.1.

·	Be able to develop programs which use the C-language interface to MMPM’s or Microsoft's Media Control Interface (MCI)

The Multimedia Presentation Manager Toolkit/2 and Microsoft Windows 3.1 Software Development Kit provide documentation and program examples to help you understand these concepts.

If you are planning on adding audio multimedia capabilities to your application, consult Chapter 2, "Audio Services" in this manual for additional information.

If you are developing an application which will utilize the FAX and/or TAM capabilities of the Mwave system, then start out by reviewing Chapter 3, "Telephony Services". This chapter provides an overview of Mwave telephony features and how to access these capabilities from your application program. Next, consult one of the following chapters for specific code examples illustrating how to call the FAX and TAM APIs from your application program:

·	For FAX application program examples, see Chapter 4, "FAX Services". The complete FAX API reference is located in Chapter 6, "FAX API Reference".

·	For TAM application program examples, see Chapter 5, "TAM Services". The complete TAM API reference is located in Chapter 7, "TAM API Reference".