RENDITION II/XE, RELEASE 3.03                       README.DOC

April 28, 1992

This file, README.DOC, contains information that is more

recent than that contained in the Rendition II/XE User Guide

and the Software Driver Guide.  Check this file for information

you may need to set-up or use your Rendition II/XE.

If you should need more assistance, the Appian Technical Support

staff will gladly assist you Monday through Thursday between the

hours of 8:00 AM and 5:00 PM Pacific Time and Friday from

8:00 AM to 4:00 PM Pacific Time.  You may reach them at

(206) 649-8086 or send your concerns to:

Appian Technology Inc.

Technical Support

2405 140th Ave. NE

Bellevue, WA 98005

FAX (206) 649-5360

Rendition II/XE Set-up Notes

----------------------------

*** Memory Conflicts with Rev H RVGA Daughterboard

The Rev H RVGA Daughterboard uses 4K of address space at

CA00 to CAFF.  This is done to insure compatibility with

the IBM VGA adapter.  Some hard disk controllers default

to an address space that includes CA00, creating a conflict.

If you have this conflict, it will probably cause your computer

to hang during boot-up with the daughterboard installed.  To

resolve the conflict, you must change the address space your

hard disk controller uses.  Consult the documentation which came

with your hard disk controller for information on how to do this.

*** Dell Model 220

The VGA built into the Dell Model 220 does not have an

IBM standard feature connector, consequently it cannot

be used for VGA pass-through to the Rendition II/XE.  There

are two possible solutions to this:

1)  Connect a separate monitor to the 220's VGA and do

not pass through the VGA to the Rendition II/XE.

2)  Disable the 220's built-in VGA and use the Rendition

II/XE RVGA for VGA compatibility.

*** Hewlett-Packard Vectra RS/20 PC

The HP Vectra RS/20 386 PC will not boot if an RVGA

daughterboard is installed on the Rendition II/XE.  There

is no problem if the Rendition II/XE RVGA daughterboard is

NOT installed.   Consequently, for VGA compatibility, a

separate VGA (either passed-through to the Rendition II/XE

or to its own monitor) is required in the system.

Memory Upgrade Kit

------------------

Due to the shortage of Toshiba TC528128AZ-10 VRAM chips (as specified

in the "Adding Memory" section of your user guide), a special Memory

Upgrade Board and corresponding sockets have been designed for your

Rendition II/XE.  To upgrade to 1MB of VRAM, you MUST purchase the

Memory Upgrade Kit.  The sockets for the Memory Upgrade Board are

located in the same place where the video RAM sockets are pictured in

your user guide and are already in place on your Rendition II/XE.

Installation instructions for the Memory Upgrade Board will be

shipped with the Memory Upgrade Kit.  To obtain the Memory Upgrade

Kit please contact your dealer.

Board Diagnostic Utility

------------------------

It is always a good idea to check your Rendition II/XE the first time

you install it with the Board Diagnostic Utility, UDIAG.  The

Rendition II/XE User Guide has detailed instructions for running this

utility.  If you have a monochrome monitor connected to your board, the

"video color bars" test will not display colors which correspond to

the text shown (it will display shades of gray, instead).  If you are

using Single Test Mode, simply press "Y" to continue to the next

test.  All other diagnostics are valid for monochrome monitors.  In

addition, if you have a monitor which is not VGA compatible, the Color

Bars Test may not sync properly.

====End README.DOC==========================================================