Installing network adapters for MS Windows for Workgroups

================================================================

Network adapters are supplied with a driver disk which includes the

NDIS driver.

The NDIS driver programs for DOS are developed following Microsoft

NDIS specification version 2.0.1.  This document is a detailed step-by-step

installation procedure for using PE-EPP network adapters with MS Windows

for Workgroups operating system.

Naming Conventions

==================

1.  Starting at this point, we will refer to Windows for Workgroups as

WFW for simplicity.  Any appearance of WFW within this text will refer

to the Microsoft Windows for Workgroups operating system.

Installation Procedures

=======================

1.  Preinstall WFW using the included Novell NE2000 compatible driver. WFW

will generate a default PROTOCOL.INI and CONFIG.SYS file using the

selected NE2000 network driver.

2.  You must modify the generated PROTOCOL.INI and CONFIG.SYS files using

any text editor. (i.e. MSDOS EDIT)

--------- modify PROTOCOL.INI ----------

3.  The PROTOCOL.INI file will have multiple references to the "ms$ne2clone"

name.  Change all occurances of this name to "PE-EPP".  In a standard

PROTOCOL.INI, there should be a total of 7 replacements.

There would be 4 replacements in the [network.setup] section.  Please

take note of the following lines in [network.setup] section,

netcard=...

lana0=...

lana1=...

There will be a [ms$ne2clone] section.  This should also be changed to

[PE-EPP].

In both [MS$NETBEUI] and [MS$IPX] sections, there will be a BINDINGS=

command line.  The BINDINGS command must be bound to "PE-EPP".

4.  There will also be a reference to "ms2000$" in the [PE-EPP] section

which needs to be changed to the PE-EPP NIC type name.

5.  In the [PE-EPP] section, the "IOBASE" keyword must be changed to

"IOADDRESS".

The succeeding pages include working sample of CONFIG.SYS and PROTOCOL.

INI files for WFW using PE-EPP network adapter.

--------- modify CONFIG.SYS ----------

6.  Your CONFIG.SYS will have the following statement for loading the

"ms2000.DOS" device driver.

DEVICE=C:\WINDOWS\MS2000.DOS

Change this line to load the appropriate PE-EPP NDIS device driver

of your network adapter.	Using PE-EPP adapter, the line would

appear as,

DEVICE=C:\WINDOWS\EPP.DOS

Note: You would have to copy the appropriate NDIS driver to

your WFW directory. (i.e. C:\WINDOWS)

PE-EPP NDIS PROTOCOL.INI User Guide

===================================

This guide lists the available PROTOCOL.INI keywords for all

PE-EPP NDIS device drivers.  Most of the keywords are optional

and have default values.  The asterisk symbol (*) indicates the

list of PE-EPP adapters where the keyword is applicable.

IOADDRESS - This keyword specifies the base I/O port address used

by the PE-EPP Network Interface Card. This value must

match the base I/O address of printer port on your PC.

INTERRUPT - This keyword specifies the interrupt line used by the

PE-EPP NIC. This value must match the IRQ of printer

port on PC.

DRIVERNAME- This keyword specifies the NIC driver's name.  The NIC

driver name follows the NIC adapter type.

NETADDRESS- This keyword specifies the node address (hex) used by

your network interface card.  This option is used only

if you want to override the node address burned in your

NIC.  In this case, the specified address will be used

instead of the NIC's actual node address.

Working samples of WFW CONFIG.SYS and PROTOCOL.INI

==================================================

CONFIG.SYS (using PE-EPP NIC)

------------------------------------

:

device=c:\windows\himem.sys

device=c:\windows\protman.dos /i:c:\windows

device=c:\windows\workgrp.sys

device=c:\windows\msipx.sys

device=c:\windows\epp.dos	     <������� PE-EPP NDIS driver

:

AUTOEXEC.BAT

------------

:

c:\windows\net start

c:\windows\msipx

c:\windows\netx

:

C:\WINDOWS\PROTOCOL.INI (using PE-EPP NIC)

------------------------------------------

[network.setup]

version=0x3100

netcard=PE-EPP,1,PE-EPP 	       <������Ŀ

transport=ms$netbeui,MS$NETBEUI 	       |  Use PE-EPP NIC

lana0=PE-EPP,1,ms$netbeui	       <��������� for Netbios and

transport=ms$ipx,MS$IPX 		       |  IPX

lana1=PE-EPP,ms$ipx		       <��������

[protman]

DriverName=PROTMAN$

PRIORITY=MS$NETBEUI

[PE-EPP]			       <�����Ŀ

DriverName=EPP$ 			      �� This section for

;INTERRUPT=7				      �  PE-EPP NIC

;IOADDRESS=0x378		       <�������

[MS$NETBEUI]

DriverName=netbeui$

SESSIONS=5

NCBS=10

BINDINGS=PE-EPP 		       <������Ŀ

LANABASE=0				       |

|

[MS$IPX]				       �� Bind to [PE-EPP]

DriverName=IPX$ 			       |

MediaType=Novell/Ethernet		       |

BINDINGS=PE-EPP 		       <��������