S.T.B. Systems Compatability Report

1651 North Glenville

Richardson ,TX  75085             Product Tested:Nitro ISA 2MEG REV.d

MONITOR MAKE & SIZE:NEC XV15	DATE OF REPORT:

C.P.U. MAKE & SPEED:		VIDEO  BIOS  REV:Nitro 1.0

SYSTEM  &  BIOS   VER:		WIN & UTIL DRV. REV:

MONITOR MAKE & SIZE:	 	DATE OF REPORT:

C.P.U. MAKE & SPEED:	 	VIDEO  BIOS  REV:

SYSTEM  &  BIOS   VER:		WIN & UTIL DRV. REV:

TECHNICIANS NAME:

-----SPECIFIC HARDWARE OR APPLICATION ERRORS -----

Win95 detected the chip as Cirrus Logic,and selected the correst driver set.All applications seemed to run smoothly,but in 256 color modes the text in graphic applications were skewed,and broken.The cards in solitair were smeared and broken in 256 color modes.

Drivers run without problems:

640x480@16,256,16-bit,24-bit,and 32-bit color modes@72Hz

800x600@16,256,and16-bit color modes@75Hz

1024x768@256,and16-bit color modes@60Hz

Note:The 256 color driver in any resolution exibits skewed text and smeared

graphics,but otherwise works fine.