| Field Note: 122 |  | Distribute freely |
| --- | --- | --- |

Subject:  		Chicago beta

Date Created:	July 31, 2026

Product: 		928Movie

Microsoft Chicago (next generation of Windows) is currently in beta phase but many 928Movie users want to use it as well as Windows 3.1.  The best way to do this is to use the 928Movie display drivers supplied by VideoLogic rather than the S3 928 drivers supplied with Chicago.  If you use the S3 drivers, then the pointer will be invisible which makes Chicago unusable.  This is because the generic S3 drivers don't account for the hardware cursor used by 928Movie.

To install the 928Movie drivers, follow this procedure:-

i)	Set the display driver as VGA when installing Chicago.

ii)	Run Setup on 928Movie v1.1 disk1, from Chicago

iii)	When Setup asks if you have the audio option, then answer 'NO'.

iv)	The 'Restart Windows ' button in the 928Movie Display applet (called at the

end of Setup) will not work with Chicago, so quit the application at this stage.

v)	You can select resolution, no. of colours and refresh rate with the 928Movie

display applet but do not try 'Restart Windows '.  Use 'Return to Windows'

instead and then restart Chicago with Start - Shutdown..

An alternative is to change between different display drivers as follows:-

i)	Select My computer - Control Panel - Display - Settings - Change Display type

ii)	Select Adapter type - Change (show all devices) - Windows 3.x drivers - Have

disk and put 928Movie disk1 in the drive.

If 928Movie has the audio option, then this will NOT be functional.  The VFW scaling capabilities of 928Movie will not be functional.

Steve Everitt

July, 26