| Field Note: 191 |  | Distribute freely |
| --- | --- | --- |

Subject:	PowerPlay32 software and Midget 90

Date Created:	July 28, 2026

Product:	928Movie

PowerPlay32 is the movie acceleration and VMC interface chip fitted to 928Movie and PCIMovie graphics cards.

PowerPlay32 has 2 revisions; Rev A which is labelled as VL i110A and Rev B, also known as Midget 90, which is labelled as VL i110B.  Rev B is fitted to the lastest 928Movie / MPEG Player combinations and some PCIMovie cards.  The minimum base level of software required to support Rev B is shown in the table below:

| Product | 928Movie | PCIMovie | MPEG Player | Captivator Pro/TV |
| --- | --- | --- | --- | --- |
| Base software | 1.1.1 | 1.1.2 & patch 87 | 1.1 | 1.0 |
| Latest Patch | 86 | none | none | 2.0 |

If a previous version is used, the graphics card will give an error message saying "cannot recognise PowerPlay device" when Windows starts.  The Rev A chip is still supported in these new versions.

The MPEG Player v1.1 or above software contains PowerPlay drivers for 928Movie (and PCIMovie) that are a later version than those available with the graphics card base software shown above.  This is the same as patch 86.

The Captivator Pro/TV v2.0 or above software contains PowerPlay drivers for 928Movie (and PCIMovie) that are a later version than those available with the graphics card base software shown above.  This is the same as patch 86.

Full screen video should be achievable in 1024x768 65K colour display mode, for both MPEG Player and Captivator Pro, when Rev B is fitted.

Steve Everitt

July, 26