| Field Note: 50 |  | Distribute freely |
| --- | --- | --- |

Subject:  Recommended IRQ settings when using sound cards with the DVA 4000

Date Created: March 19, 1993

Product: DVA 4000

Recent testing has highlighted a potential problem involving sound cards such as the SoundBlaster Pro.

When installed in a system, the sound card must occupy a high priority interrupt.  If any peripheral devices, such as the DVA-4000 or a serial card, occupy a higher priority interrupt than the sound card the system may suddenly lock during recording or playback of an audio file.  Sometimes this is a permanent lock up and sometimes control will return to the system after a period of time.

The order or priority of interrupts in a PC is as follows:

HIGHEST PRIORITY	IRQ 2

IRQ8	}

IRQ9	}

IRQ10	}	(IRQ's 8 thru 15 are routed via

IRQ11	}	IRQ2 so take a higher priority

IRQ12	}	than IRQ 3 to 7)

IRQ13	}

IRQ14	}

IRQ15	}

IRQ 3

IRQ 4

IRQ 5

IRQ 6

LOWEST PRIORITY		IRQ 7

We therefore advise that the DVA is kept on its default interrupt of IRQ7 and the sound card on IRQ5.

Liz Parratt

March, 93