| Field Note: 54 |  | Distribute freely |
| --- | --- | --- |

Subject:  DVA-4000 no video - video shows as black

Date Created: July 28, 2026

Product: DVA-4000

Although the connectors on the back of the DVA-4000 are D-type plugs, the video portion of cable VL-62509 can very easilly be plugged in upside down. This is due to the fact that there are pins intentionally missing on the cable.

The symptom of this situation is black in place of where video should be. The easiest way to check is to look at the top cable coming off of the DVA-4000 (VL-62509) and determine if the short audio plug is coming out from the top or in-between the VGA cable and the video source cable (VL-62509). The correct position is with the audio cable protruding from in-between the two connectors.

Peiter Zatko

July, 26