| Field Note: 67 |  | Distribute freely |
| --- | --- | --- |

Subject:  Video sources with bad syncs (e.g. cameras)

Date Created: July 28, 2026

Product: DVA-4000

The DVA-4000 (all flavours) uses the sync pulses from the incoming video as the clock for a lot of its processing. If no video is attached, one of the on-board chips (the Philips 9051) simulates incoming sync pulses, but once a source is there, the DVA effectively drives itself from the syncs on the video.

The vast majority of video sources provide fair to good sync pulses, but a number of the cheaper devices (such as consumer-end Camcorders and video still cameras) produce very irregular syncs - especially if they are paused. In certain circumstances, this can cause the DVA-4000 to stop responding to the system - as it cannot operate correctly trying to run from (almost) random interrupts. One particularly sensitive time is during a MediaSpace calibrate where the system is worked particularly hard, and is more susceptible to bad sync signals.

There is no clean solution to the problem. Obviously one answer is to use equipment which produces sync signals within the video specification, but this is not always possible. Certainly, if tape players are being used, you should pause the player as little as possible, and particularly at difficult times (such as a MediaSpace calibrate).

In MIC System II, if the DVA should stop responding to the system for this reason, restarting Windows will reinitialise the DVA again. You may also have to reset your video source in some way (e.g. power off and on) to completely reset the system.

Colin Crawford

July, 26