|  | SOFTWARE RELEASE NOTE |  |
| --- | --- | --- |
| Product: S3REFRSH.EXE |  | Version: 2.21 |
| Used For Chip(s): ALL |  |  |
| Date: 2/23/96 |  |  |
| Changes Made From S3REFRSH.EXE Ver. 2.18 |  |  |
| CHANGES: Fixed message error which using 764 and earlier chips refresh rate table for 765 and late chips (56Hz was showed). Change CR41 and CR52 bit setting for Virge, VirgeVX, M65* and Telluride* according the new scheme below: 43/56Hz Int [000] 60Hz [001] 70Hz [002] 72Hz [002] 75Hz [003] 85Hz [004] * This new scheme not be implemented in current BIOS version, need use S3REFRSH Ver. 2.18 to set the refresh rate properly. |  |  |
| Comment: CR 41 CR 52 1280 1024 800 640 Comment (continue): The refresh rate value in the resolution fields of CR41 and CR52: Trio64V+ (765): 640x480 800x600 1024x768 1280x1024 60 0 1 2 4 70 x x 3 x 72 1 2 x 5 75 2 3 4 6 85 3 4 5 x int x x 0,1 0-3 Virge and VirgeVX: 640x480 800x600 1024x768 1280x1024 60 1 1 1 1 70 x x 2 x 72 2 2 x x 75 3 3 3 3 85 4 4 4 x int x x 0 0 |  |  |