***********************************

*                                 *

*   MICROTOUCH/MICROVITEC Patch   *

*                                 *

*          Version 4.0.6          *

*                                 *

***********************************

*************************  I M P O R T A N T  *************************

MICROTOUCH

1.  A bug has been found that prevents MicroTouch touch screens from

working in their default settings.  SCRN-CAL gives a comms error message

if the MicroTouch is used with any communication parameters other than

N81.

2.  The new MicroTouch driver will check the touch screen's communication

parameters first, and then it will set the PC's com port.

MICROVITEC

3.  A bug has been found that prevents MicroVitec touch screens from

working with faster PCs.  SCRN-CAL will give a comms error message when

this bug occurs.  This patch provides a replacement driver for the

MicroVitec that cures this bug.

INSTALLATION PROCEDURE

4.  This patch software is installed from within AMS in the usual way from

the "Install an AMS Application" option on the main menu.

The procedure renames the files

SCRN-MCT.SYS  ---> SCRN-MCT.SYN

SCRN-MVT.SYS  ---> SCRN-MVT.SYN

and then copies the SCRN-MCT.SYS and SCRN-MVT.SYS files from the supplied

diskette into the MICFILES directory. For that reason the destination drive

specified must be the one on which MIC is installed.

5.  After installation you must re-load the MIC system to bring the patch into

effect.