|  | Compaq Armada 4150 |  | Compaq Armada 4160T | Compaq Armada 4210T | Compaq Armada 4220T |
| --- | --- | --- | --- | --- | --- |
| System Unit |  |  |  |  |  |
| Microprocessor |  |  |  |  |  |
| Standard | Intel Pentium 150-MHz with MMX technology |  | Intel Pentium 166-MHz with MMX technology | Intel Pentium 233-MHz with MMX technology | Intel Pentium 266-MHz with MMX technology |
| Cache | 256 KB level 2 write-back |  | 256 KB level 2 write-back | 256 KB level 2 write-back | 512 KB level 2 write-back |
| Architecture | 64-bit CPU cache and memory bus; PCI local bus graphics |  | 64-bit CPU cache and memory bus; PCI local bus graphics | 64-bit CPU cache and memory bus; PCI local bus graphics | 64-bit CPU cache and memory bus; PCI local bus graphics |
| RAM |  |  |  |  |  |
| Standard | 16 MB |  | 16 MB | 32 MB | 32 MB |
| Maximum1 | 80 MB |  | 80 MB | 96 MB | 96 MB |
| Coprocessor | Integrated |  | Integrated | Integrated | Integrated |
| Advanced Power Management | Yes |  | Yes | Yes | Yes |
| Display |  |  |  |  |  |
| Type | 12.1-inch high-contrast color STN |  | 12.1-inch color TFT | 12.1-inch color TFT | 12.1-inch color TFT |
| Resolution | 800 x 600 with up to 256 colors |  | 800 x 600 with up to 64,000 colors | 800 x 600 with up to 16.7 million colors | 800 x 600 with up to 16.7 million colors |
| Extended Text | Yes |  | Yes | Yes | Yes |
| Graphics |  |  |  |  |  |
| 64-bit PCI Local Bus Video | No |  | No | Yes | Yes |
| 32-bit PCI Local Bus Video | Yes |  | Yes | No | No |
| 1024 x 768 External | Yes |  | Yes | Yes | Yes |
| Storage |  |  |  |  |  |
| 1.44-MB Diskette Drive |  | Standard | Standard | Standard | Standard |
| Upgradable SMART Hard Drives |  |  |  |  |  |
| Standard | 1.6-GB |  | 2.0-GB | 3.2-GB | 4-GB |
| Optional | 2.0-GB, 3.2-GB1, 4.0-GB1 |  | 3.2-GB1, 4.0-GB1 | 3.2-GB1, 4.0-GB1 | 3.2-GB1, 4.0-GB1 |
| Mobile CD Unit | Optional |  | Optional | Optional | Optional |
| Interfaces |  |  |  |  |  |
| PC Card | Two Type II/ One Type III slot(s) |  | Two Type II/ One Type III slot(s) | Two Type II/ One Type III slot(s) | Two Type II/ One Type III slot(s) |
| Infrared Communication Port | 1 |  | 1 | 1 | 1 |
| Enhanced Parallel | 1 |  | 1 | 1 | 1 |
| Serial | 1 |  | 1 | 1 | 1 |
| External Monitor | 1 |  | 1 | 1 | 1 |
| NTSC/PAL Video Output | 1 |  | 1 | 1 | 1 |
| External Keyboard/Pointing Device | 1 |  | 1 | 1 | 1 |
| Expansion Base Connector | 1 |  | 1 | 1 | 1 |
| AC Adapter/Automobile Adapter | 1 |  | 1 | 1 | 1 |

|  | Compaq Armada 4150 | Compaq Armada 4160T | Compaq Armada 4210T | Compaq Armada 4220T |
| --- | --- | --- | --- | --- |
| Interfaces (cont’d) |  |  |  |  |
| Microphone In | 1 | 1 | 1 | 1 |
| Headphone Out | 1 | 1 | 1 | 1 |
| Stereo Line In | 1 | 1 | 1 | 1 |
| Monitors |  |  |  |  |
| V50 Color | Optional | Optional | Optional | Optional |
| V70 Color | Optional | Optional | Optional | Optional |
| V90 Color | Optional | Optional | Optional | Optional |
| P50 Color | Optional | Optional | Optional | Optional |
| P70 Color | Optional | Optional | Optional | Optional |
| P110 color | Optional | Optional | Optional | Optional |
| TFT500 Flat Panel | Optional | Optional | Optional | Optional |
| Keyboard |  |  |  |  |
| 101-key compatible keyboard with Inverted “T” Cursor Keys, Hotkeys, Right Ctrl Key and four user-programmable keys | Yes | Yes | Yes | Yes |
| External Enhanced III Keyboard | Optional | Optional | Optional | Optional |
| Supported System Software |  |  |  |  |
| Software | Microsoft Windows 3.1, Windows 95 OSR2, Windows NT Workstation 3.51, 4.0, and IBM OS/2 | Microsoft Windows 3.1, Windows 95 OSR2, Windows NT Workstation 3.51, 4.0, and IBM OS/2 | Microsoft Windows 3.1, Windows 95 OSR2, Windows NT Workstation 4.0 | Microsoft Windows 3.1, Windows 95 OSR2, Windows NT Workstation 4.0 |
| Physical Specifications |  |  |  |  |
| Dimensions (HxWxD) | 1.49 x 11.6 x 10.3 in/3.78 x 29.64 x 26.16 cm | 1.49 x 11.6 x 10.3 in/3.78 x 29.64 x 26.16 cm | 1.49 x 11.4 x 8.8 in/ 3.78 x 28.96 x 22.35 cm | 1.49 x 11.4 x 8.8 in/ 3.78 x 28.96 x 22.35 cm |
| Weight (shipping) | 6.2 lb/2.83 kg | 6.2 lb/2.83 kg | 5.3 lb/2.42 kg | 5.3 lb/2.42 kg |

1	The 3.2- and 4.0-GB SMART Hard Drives will be available 2Q 1998.