| System Unit | Dimensions (HxWxD) | 1.5 x 12 x 9.27 in/3.91 x 30.5 x 23.55 cm |
| --- | --- | --- |
| . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .. | Weight (without AC Adapter) |  |
|  | Notebook only | 6 lb/2.72 kg |
|  | Notebook with AC Adapter and Power Cord | 6.85 lb/3.12 kg |
|  | Power Supply |  |
|  | Rated Input Voltage | 100-120/220-240 VAC |
|  | Rated Input Current | 1.0/0.6 A |
|  | Rated Frequency | 50-60 Hz |
|  | Rated Output Power (Max) | 44 W (18.85 Vdc/3.2 A) |
|  | Stand-Alone Power Requirements |  |
|  | Nominal Operating Voltage | 14.4 VDC |
|  | Average Operating Power | 15 W |
|  | Peak Operating Power | 25 W |
|  | Temperature |  |
|  | Operating | 50 to 104° F/10 to 40° C |
|  | Nonoperating | -4 to 140° F/-20 to 60° C |
|  | Relative Humidity (noncondensing) |  |
|  | Operating | 10 to 90% |
|  | Nonoperating | 5 to 95% |
|  | Shock |  |
|  | Operating | 10 G, 11 ms, half sine |
|  | Non operating | 240 G, 2 ms, half sine |
|  | Vibration |  |
|  | Operating | 0.5 G |
|  | Nonoperating | 1.5 G |
|  | Maximum Altitude (unpressurized) |  |
|  | Operating | 10,000 ft/3.05 km |
|  | Nonoperating | 30,000 ft/9.14 km |
| 12.1-Inch Color TFT 1024 x 768 Display | Dimensions (HxW) Diagonal Size | 7.3 x 9.7 in/18.45 x 24.60 cm 12.1 in/30.75 cm |
|  | Mounting | Internal |
|  | Number of Colors | Up to 64,000 |
|  | Contrast Ratio | 100:1 |
|  | Brightness | 85 CD/M2 @ 6.0 mA |
|  | Pixel Resolution |  |
|  | Pitch | 0.240 x 0.240 mm |
|  | Format | 1024 x 768 |
|  | Configuration | RGB Stripe |
|  | Backlight | Edge Lit |
|  | Character Display | 80 x 25 |
|  | Total Power Consumption (maximum) | 3.8 W |
| 12.1-Inch Color TFT 800 x 600 (SVGA) Display | Dimensions (HxW) Diagonal Size | 7.3 x 9.7 in/18.45 x 24.60 cm 12.1 in/30.75 cm |
|  | Mounting | Internal |

| 12.1-Inch Color TFT 800 x 600 (SVGA) Display (cont’d) | Number of Colors Contrast Ratio | Up to 16M 100:1 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Brightness . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .. | 110 CD/M2 @ 5.75 mA |  |  |  |
|  | Pixel Resolution |  |  |  |  |
|  | Pitch | 0.3075 x 0.3075 mm |  |  |  |
|  | Format | 800 x 600 |  |  |  |
|  | Configuration | RGB Stripe |  |  |  |
|  | Backlight | Edge Lit |  |  |  |
|  | Character Display | 80 x 25 |  |  |  |
|  | Total Power Consumption (maximum) | 3.8 W |  |  |  |
| Hard Drives1 |  |  | 2.1-GB | 3-GB | 4-GB |
|  | Capacity Per Drive |  | 2.1 GB | 3 GB | 4 GB |
|  | SMART, drive fault prediction capable |  | Yes | Yes | Yes |
|  | Drive Height |  | 12.7 mm | 12.7 mm | 12.7 mm |
|  | Transfer Rate |  |  |  |  |
|  | Media |  | 39.1 to 61.6 Mb/s | 39.6 to 71.5 Mb/s | 51.7 to 83.4 Mb/s |
|  | Interface |  | 16.6 MB/s | 16.6 MB/s | 16.6 MB/s |
|  | Sector Interleave |  | 1:1 | 1:1 | 1:1 |
|  | Seek Times (typical, including settling) |  |  |  |  |
|  | Track-to-track |  | 4 ms | 2.5 ms | 4.0 ms |
|  | Average |  | 13 ms | 13.0 ms | 13.0 ms |
|  | Full Stroke |  | 23 ms | 23.0 ms | 23.0 ms |
|  | Disk Rotational Speed |  | 4000 rpm | 4000 rpm | 4000 rpm |
|  | Physical Configuration |  |  |  |  |
|  | Cylinders |  | 4975 | 6371 | 6975 |
|  | Heads |  | 6 | 6 | 6 |
|  | Bytes per sector |  | 110 to 180 | 117 to 216 | 144 to 240 |
|  | Logical Configuration |  |  |  |  |
|  | Cylinders |  | 4200 | 6300 | 7944 |
|  | Heads |  | 16 | 16 | 16 |
|  | Sectors per track |  | 63 | 63 | 63 |
|  | Bytes per sector |  | 512 | 512 | 512 |
|  | Buffer Size |  | 96 KB | 128 KB | 512 KB |
| Diskette Drive | Diskette Size Light | 3.5-inch None |  |  |  |
|  | Height | .53 in /1.35 cm |  |  |  |
|  | Bytes per Sector | 512 |  |  |  |
|  | Sectors per Track |  |  |  |  |
|  | High density | 18 (1.44-MB)/15 (1.2-MB) |  |  |  |
|  | Low density | 9 |  |  |  |

| Diskette Drive (cont’d) | Tracks per side |  |
| --- | --- | --- |
|  | High density | 80 (1.44-MB)/80 (1.2-MB) |
|  | Low density | 80 |
|  | Read/Write heads | 2 |
|  | Access Times |  |
|  | Track-to-track (high/low) | 3 ms/6 ms |
| . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .. | Average (high/low) | 95 ms/173 ms |
|  | Settling time | 15 ms |
|  | Latency average | 100 ms |
| 20X Max2 CD-ROM Drive | Applicable Disc | CD-DA, CD-ROM Mode 1, Mode 2 CD-XA (Mode 2, Form 1, Form 2) CD-I (Mode 2, Form 1, Form 2) CD-I Ready, CD Extra, Video CD, CD-Bridge Photo CD (Single and Multisession) CD-WO (Fixed packets only) |
|  | Center Hole Diameter | .59 in/1.5 cm |
|  | Disc Diameter | 4.72 in, 3.15 in/12 cm, 8 cm |
|  | Track Pitch | 1.6 µm |
|  | Access Times (typical including settling) Random | Less than 170 ms |
|  | Full Stroke | Less than 600 ms |
|  | Audio Output Level |  |
|  | Line Out | .7 Vrms |
|  | Cache Buffer | 128 KB |
|  | Data Transfer Rates (typical, including settling) |  |
|  | Sustained (1X) | 150 KB/s |
|  | Sustained (20X) | 3.0 MB/s |
|  | Startup Time | Less than 10 seconds |
| Li-Ion Battery Pack | Dimensions (HxWxD) Weight | .79 x 3.46 x 5.43 in/2.0 x 8.8 x 13.8 cm .85 lb/.39 kg |
|  | Energy |  |
|  | Voltage | 14.4 V |
|  | Capacity | 2.7 Ah |
|  | Battery Life | Up to 3 hours3 |
|  | Temperature |  |
|  | Operating | 50° to 104° F/10° to 40° C |
|  | Recommended Storage | 32° to 113° F/0° to 45° C |
| ArmadaStation (optional) | Dimensions (HxWxD) Without Monitor Support Cover | 5 x 17.3 x 16 in/12.7 x 46.5 x 40.6 cm |
|  | With Monitor Support Cover | 5.6 x 17.3 x 16 in/14.2 x 46.5 x 40.6 cm |

| ArmadaStation (optional) | Weight |  |
| --- | --- | --- |
| (cont’d) | ArmadaStation | 22.7 lb/10.3 kg |
|  | Monitor Support Cover | 3.3 lb/1.5 kg |
|  | Power Supply (Input) Operating Voltage | 90-260 VAC |
|  | Operating Current | 5.0 Amp Maximum |
|  | Rated Voltage | 100-120 to 220-240 VAC |
|  | Rated Current | 2.5-5.0 Amp |
|  | Line Frequency | 50-60 Hz |
| . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .. | Temperature |  |
|  | Operating | 5 to 95° F/10 to 35° C |
|  | Nonoperating | -22 to 140° F/-30 to 60° C |
|  | Relative Humidity |  |
|  | Operating | 20 to 80% |
|  | Nonoperating | 5 to 90% |
|  | Maximum Altitude |  |
|  | Operating | 10,000 ft/3048 m |
|  | Nonoperating | 30,000 ft/9144 m |
|  | Shock Operating | 5 G, 11 ms, half sine |
|  | Nonoperating | 140 G, 2 ms, half sine |
|  | Vibration |  |
|  | Operating | .25 G, 10 to 500 Hz sine .25 octave/minute sweep |
|  | Nonoperating | .75 G, 10 to 500 Hz sine .5 octave/minute sweep |
|  | Monitor Support (maximum) | 55 lb/25 kg |
| Armada MiniStation E (optional) | Dimensions (HxWxD) Without Monitor Support Cover With Monitor Support Cover | 5.0 x 14.75 x 15.37 in/12.7 x 37.47 x 39.05 cm 5.6 x 14.75 x 15.37 in/14.2 x 37.47 x 39.05 cm |
|  | Weight Armada MiniStation E | 7.67 lb/3.48 kg |
|  | Monitor Support Cover Option | 3.3 lb/1.5 kg |
|  | Power Supply (Input) |  |
|  | Operating Voltage | 90-260 VAC |
|  | Operating Current | 3.0 Amp Maximum |
|  | Rated Voltage | 100-120 VAC to 220-240 VAC |
|  | Rated Current | 1.5-3.0 Amps Maximum |
|  | Line Frequency | 50-60 Hz |
|  | Temperature |  |
|  | Operating | 5 to 95° F/10 to 35° C |
|  | Nonoperating | -22 to 140° F/-30 to 60° C |
|  | Relative Humidity |  |
|  | Operating | 20 to 80% |
|  | Nonoperating | 5 to 90% |
|  | Maximum Altitude |  |
|  | Operating | 10,000 ft/3048 m |
|  | Nonoperating | 30,000 ft/9144 m |
|  | Shock |  |
|  | Operating | 5 G, 11 ms, half sine |
|  | Nonoperating | 140 G, 2 ms, half sine |

| Armada MiniStation E (optional) (cont’d) | Vibration Operating | .25 G, 10 to 500 Hz sine .25 octave/minute sweep |
| --- | --- | --- |
|  | Nonoperating | .75 G, 10 to 500 Hz sine .5 octave/minute sweep |
|  | Monitor Support (Maximum) | 55 lb/25 kg |
| . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .. | 1Hard drive specifications may vary slightly depending on vendor. 2Data transfer tates will vary from 1275 KB/s to 3000 KB/s. 3When tested in standard configuration using the Ziff-Davis BatteryMark test version 1.0, the battery life of the Armada 7300 family will range from 1 hour and 30 minutes to 3 hours and 00 minutes depending on individual usage. |  |
|  | Compaq, registered United States Patent and Trademark Office. Armada, ArmadaStation, MiniStation, MultiLock, QuickBlank, QuickLock, are trademarks and/or service marks of Compaq Computer Corporation. Other product names mentioned herein may be trademarks and/or registered trademarks of their respective companies. ©1998 Compaq Computer Corporation. All rights reserved. Printed in the U.S.A. |  |