| System Unit | Dimensions (HxWxD) |  | 5.00 x 18.25 x 15.69 in/12.70 x 46.36 x 39.85 cm |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Weight (approximate) |  | 28.3 lbs/12.85 kg |  |  |  |  |  |
| . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . |  |  |  |  |  |  |  |  |
| Power Supply |  |  |  |  |  |  |  |  |
|  | Operating Voltage Range |  | 90 - 132 VAC/180 - 264 VAC |  |  |  |  |  |
|  | Rated Voltage Range |  | 100 - 120 VAC/220 - 240 VAC |  |  |  |  |  |
|  | Rated Line Frequency |  | 50 - 60 Hz |  |  |  |  |  |
|  | Rated Input Current |  | 5A/3A |  |  |  |  |  |
|  | Maximum Rated Power |  | 200 W |  |  |  |  |  |
|  | Temperature |  |  |  |  |  |  |  |
|  | Operating |  | 50° to 95°F/10° to 35°C |  |  |  |  |  |
|  | Nonoperating |  | -4° to 140°F/-20° to 60°C |  |  |  |  |  |
|  | Humidity (noncondensing) |  |  |  |  |  |  |  |
|  | Operating |  | 8% to 90% |  |  |  |  |  |
|  | Nonoperating |  | 5% to 95% |  |  |  |  |  |
|  | Maximum Altitude (unpressurized) |  |  |  |  |  |  |  |
|  | Operating |  | 10,000 ft/3048 m |  |  |  |  |  |
|  | Nonoperating |  |  | 30,000 ft/9144 m |  |  |  |  |
| Enhanced Keyboard | Dimensions (HxWxD) Weight |  | 1.5 x 18.0 x 6.5 in/3.81 x 45.72 x 16.51 cm 3.15 lbs/1.59 kg |  |  |  |  |  |
| 1.44-MB Diskette Drive | Diskette Size |  | 3.5-in |  |  |  |  |  |
|  | LED Indicators |  | Green |  |  |  |  |  |
|  | Read/Write Capacity/Diskette (high/low density) |  | 1.44 MB/720 KB |  |  |  |  |  |
|  | Drive Height |  | 1 (in) |  |  |  |  |  |
|  | Drive Rotation |  | 300 rpm |  |  |  |  |  |
|  | Transfer Rate (high/low density) |  | 500/250 Kb/s |  |  |  |  |  |
|  | Bytes/Sector |  | 512 |  |  |  |  |  |
|  | Sectors/Track (high/low density) |  | 18/9 |  |  |  |  |  |
|  | Tracks/Side (high/low density) |  | 80/80 |  |  |  |  |  |
|  | Seek Time |  |  |  |  |  |  |  |
|  | Track-to-Track (high/low density) |  | 3/6 ms |  |  |  |  |  |
|  | Average (high/low) |  | 94/173 ms |  |  |  |  |  |
|  | Settling Time |  | 15 ms |  |  |  |  |  |
|  | Latency Average |  | 100 ms |  |  |  |  |  |
|  | Cylinders (high/low) |  | 80/80 |  |  |  |  |  |
|  | Read/Write Heads |  |  |  | 2 |  |  |  |
| UltraSCSI Hard Drives |  | 2.1 GB |  |  |  | 4.3 GB |  |  |
|  | Formatted Capacity | 2.16 GB |  |  |  | 4.29 GB |  |  |
|  | Transfer Rate |  |  |  |  |  |  |  |
|  | Media | 62.5-103.4 Mb/s |  |  |  | 83.2-139.3 Mb/s |  |  |

InterfaceUp to 20 MB/sUp to 20 MB/s

| UltraSCSI Hard Drives |  | 2.1 GB | 4.3 GB |
| --- | --- | --- | --- |
| (cont’d) | Disk Rotational Speed | 5400 rpm | 7200 rpm |
|  | Seek Time (typical including settling) |  |  |
|  | Single Track | 1.19 ms | 1.0 ms |
|  | Average | 8.5 ms | 8.0 ms |
|  | Full Stroke | 15 ms | 15 ms |
|  | Cylinders |  |  |
|  | Physical | 8209 | 6144 |
|  | Logical | — | — |
|  | Data Heads | 3 | 8 |
|  | Sectors per Track |  |  |
|  | Physical | 132-211 | 126-216 |
|  | Logical | — | — |
|  | Buffer Size | 512 K | 512 K |
| 24X Max1 CD-ROM Drive | Disk Applicable Disk | CD-ROM (Mode 1 and 2), CD-DA, CD-XA Ready, |  |
|  |  | Photo CD (single and multi session), Mixed Mode (audio and data combined), Cdi Ready |  |
|  | Capacity | 650 MB (Mode 2, 12 cm), 540 MB (Mode 1, 12 cm) |  |
|  | Block Size | 2048, bytes (Mode 1) |  |
|  |  | 2340, 2336 bytes (Mode 2) |  |
|  |  | 2352 bytes (CD-DA) |  |
|  |  | 2328 bytes (CD-XA) |  |
| . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . |  |  |  |
| Rotational Speed | 5200 rpm |  |  |
|  | Diameter | 12 cm, 8 cm |  |
|  | Center Hole | N/A |  |
|  | Thickness | 1.2 mm |  |
|  | Track Pitch | 1.6 µm |  |
|  | Interface | IDE |  |
|  | Access Time (typical) |  |  |
|  | Random | < 85 ms |  |
|  | Full Stroke | < 150 ms |  |
|  | Audio Output Level |  |  |
|  | Line Out | 700 MV (.7 Vrms) |  |
|  | Headphone | 0.6 Vrms |  |
|  | Data Transfer Rate |  |  |
|  | Sustained | 150 KB/s |  |
|  | Burst | 4.0 MB/s (variable) |  |
|  | Cache Buffer | 128 KB |  |
|  | Startup Time | Less than 7 seconds |  |
|  | Stop Time | Less than 4 seconds |  |
|  | Laser Parameters |  |  |
|  | Type | Semiconductor Laser GaAIAs |  |
|  | Wave Length | 780 +/- 25 nm |  |
|  | Divergence Angle | 53.5° +/- 1.5° |  |

Output Power0.14 mW (at Objective Lens)

| 24X Max1 CD-ROM Drive (cont’d) | Operating Conditions Temperature |  |  |  |  | 41 to 113° F/5 to 45° C |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Humidity |  |  |  |  | 10 to 80% |  |  |  |  |  |  |  |  |  |
|  | Dimensions (HxWxD, maximum) |  |  | 1.69 x 5.91 x 8.2 in/4.29 x 15.0 x 20.8 cm |  |  |  |  |  |  |  |  |  |  |  |
|  | Weight |  |  | 2.7 lbs/1.20 kg |  |  |  |  |  |  |  |  |  |  |  |
| Compaq Netelligent 10/100 TX PCI UTP Controller (embedded) | Network Interface Expansion Bus Architecture Network Transfer Rate |  |  |  |  | 10/100 Base TX 32-Bit PCI bus-master |  |  |  |  |  |  |  |  |  |
|  | 10Base-T (Half-Duplex) |  |  |  |  | 10 Mb/s |  |  |  |  |  |  |  |  |  |
|  | 10Base-T (Full-Duplex) |  |  |  |  | 20 Mb/s |  |  |  |  |  |  |  |  |  |
|  | 100Base-TX (Half-Duplex) |  |  |  |  | 100 Mb/s |  |  |  |  |  |  |  |  |  |
|  | 100Base-TX (Full-Duplex) |  |  |  |  | 200 Mb/s |  |  |  |  |  |  |  |  |  |
|  | Connectors |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Ethernet |  |  |  |  | RJ-45 |  |  |  |  |  |  |  |  |  |
|  | Standards Compliance |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Ethernet |  |  |  |  | IEEE 802.3 |  |  |  |  |  |  |  |  |  |
|  | 100 Mb/s Ethernet |  |  |  |  | IEEE 802.3 |  |  |  |  |  |  |  |  |  |
|  | I/O Address |  |  |  |  | Auto-configured |  |  |  |  |  |  |  |  |  |
|  | Environmental Requirements |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Operating |  |  |  |  | 10° to 35°C; 10% to 90% humidity |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | (noncondensing); 3048 m altitude |  |  |  |  |  |  |  |  |  |
|  | Nonoperating |  |  |  |  | 10° to 50°C; 5% to 90% humidity |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | (noncondensing); 9144 altitude |  |  |  |  |  |  |  |  |  |
|  | Shipping |  |  |  |  | 30° to 50°C; 20% to 80% humidity |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | (noncondensing); 3084 m altitude |  |  |  |  |  |  |  |  |  |
|  | Communication Processor |  |  |  |  | Texas Instruments ThunderLAN 3.1 |  |  |  |  |  |  |  |  |  |
|  | OS Driver Support |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | MS Windows NT Server and Workstation v. 3.51 |  |  |  |  | NDIS 3.0 Compliant |  |  |  |  |  |  |  |  |  |
|  | MS Windows NT Workstation 4.0 |  |  |  |  | NDIS 4.0 Compliant |  |  |  |  |  |  |  |  |  |
|  | MS Windows 95 (OSRO) |  |  |  |  | NDIS 3.0 Compliant |  |  |  |  |  |  |  |  |  |
|  | MS Windows 95 (OSR2) |  |  |  |  | NDIS 4.0 Compliant |  |  |  |  |  |  |  |  |  |
|  | WFW v.3.11 |  |  |  |  | NDIS 3.0 Compliant |  |  |  |  |  |  |  |  |  |
|  | MS LAN Manager |  |  |  |  | NDIS 2.0 Compliant |  |  |  |  |  |  |  |  |  |
|  | Novell NetWare Server v. 3.x, 4.x |  |  |  |  | ODI Compliant |  |  |  |  |  |  |  |  |  |
|  | Novell NetWare DOS |  |  |  |  | ODI Compliant |  |  |  |  |  |  |  |  |  |
|  | IBM OS/2 v. 3.x, 4.x, WARP, SMP |  |  |  |  | OS/2 NDIS 2.0 Compliant |  |  |  |  |  |  |  |  |  |
|  | IBM OS/2 NetWare Requestor v.2.x, 3.x |  |  |  |  | ODI compliant |  |  |  |  |  |  |  |  |  |
| Matrox Millennium II AGP Graphics Controller | Resolut . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . ion |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4-MB WRAM Colors Supported | 8-MB WRAM Colors Supported | 16-MB WRAM Colors Supported | Refresh Rate |  |  |  |  |  |  |  |  |  |  |  |  |

640 x 480 
800 x 600
1024 x 768
1280 x 1024
1600 x 1200
1920 x 1200
16.7M16.7M 
16.7M 
16.7M 
16.7M 
256
64K16.7M 
16.7M 
16.7M 
16.7M 
256
64K85 Hz
85 Hz
85 Hz

| Matrox Millennium II AGP Graphics Controller (cont’d) . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . |  |  |
| --- | --- | --- |
| Acceleration Memory | 3D GUI, 250 MHz RAMDAC 4-MB WRAM Standard (4 MB and 12 MB upgrade available to maximum 16 MB total) |  |
|  | 1Data transfer rate will vary from 150 KB/s to 3600 KB/s. |  |
|  | Compaq, Compaq Insight Manager, Deskpro, registered United States Patent and Trademark Office. Netelligent, SmartQ, MultiLock, QuickBlank, QuickLock, are trademarks and /or service marks of Compaq Computer Corporation. Novell and NetWare are registered trademarks of Novell, Inc. Microsoft, Windows, Windows NT and SQL Server are trademarks of Microsoft Corporation. IBM and OS/2 are registered trademarks of IBM Corporation. SCO is a registered trademark of the Santa Cruz Operation, Inc. UNIX is a registered trademark of the Open Group in the US and other countries, Inc. ORACLE is a registered trademark and ORACLE7 is a trademark of Oracle Corporation. Cheyenne and ARCserve are registered trademarks of Cheyenne Software, Inc. JETserve is a trademark of Cheyenne Software, Inc. Other product names mentioned herein may be trademarks and/or registered trademarks of their respective companies. ©1998 Compaq Computer Corporation. All rights reserved. Printed in the U.S.A. |  |

85 Hz
85 Hz
75 Hz

16.7M 
16.7M
16.7M
256 
256