A Tiny Little Symass Doc File

In most ways, this unofficial version of Symass (let's call it 3.9) is just

like Brad Templeton's (commercial) assembler Pal, so if you know Pal, you're

laughing. If you don't know Pal, here's a few things you should know, along

with some points particular to Symass. If you have questions, address them

to Karl Hildon (76703,4242) or Nick Sullivan (76703,4353) on CompuServe's

CBMPRG Forum.

1) Unlike Pal, Symass was written by Robert Huehn, with additional features

by Tim Buist. The program is copyright by Transactor Publishing Inc., and

may be freely redistributed but not sold commercially. If you want to include

it in your user group library, that's okay, but you should really wait for

the official version 4.0 release, coming soon on a Transactor disk, with

a few bug fixes and some further enhancements.

2) Known bugs in this version

a) the stack is not restored properly after assembly. This is not a major

problem, but if it bothers you, do a CLR in direct mode to clean the stack.

b) the .byte and .word pseudo-ops work properly only if there are no

embedded spaces in the data. Thus:

.byte $57,$4f,$52,$4b,$53      ;works

.byte $46, $41, $49, $4c, $53  ;fails

3) To start this version of Symass, load "boot",8 and run. The assembler

must be on the disk in device 8, drive 0, under the name Symass3.9. It loads

under the Basic ROM at $A000, consuming about 5K. A short interface routine

is located at address 700, where it is put by the boot program.

4) Write your source as a Basic program file with SYS 700 at the start of

your 6502 code, and .END at the end. Specify the start address with a line

like:

110 *= $c000

5) The .OPT pseudo-op specifies output options. The main one is .OPT OO,

which means to assemble to memory at the specified address. You can also

specify assembly to disk with .OPT #, where # is a logical file you opened

in a line before the SYS 700, like this:

100 OPEN 2,8,2,"0:MUTANT FEET,P,W"

110 SYS 700

120 .OPT O2

130 *= $1000

Symass closes the file for you after assembly - you don't have to do it

yourself.

6) Set up equates using the equals sign, not EQU or whatever. Like this:

150 CHROUT = $FFD2

160 SKIDOO = $23

7) Many of Pal's more rococo features are not supported by Symass, e.g.

the relocating object option, the .BAS and .FILE pseudo-ops and division in

expressions. At least .FILE will probably be added for the next version.

Nick Sullivan

Transactor

READING FILE FROM IMAGE