HSTPCBQM.DOC                Original Release 1.0                       06/22/87

USR Courier HST usage with PCBoard 11.8a/E3 and with Qmodem 3.X

by Dave Schubert, The Casino PCBoard BBS (609)652-6030 Atlantic City, NJ

Overview

This document provides step-by-step information to initialize your HST

modem to compatible values with other PCBoard System Operators (SYSOPs).

If you are not a PCBoard SYSOP, you can still use the Qmodem setup values

to ensure compatibility and optimum performance.

Objective

Upon completion of these procedures, your PCBoard will answer the phone

line at 19,200 bps and handle all baud rates 300 through 19,200.  Your

callers log and user logon response will also reflect their correct baud

rate even though the modem will be connected at a fixed rate of 19,200

bps.  This feature is part of PCBoard reading the "CONNECT XXXX' statement

from the modem upon connection.

Also upon completion, your Qmodem 3.X (3.0 or 3.1) will be able to

call out to any and all BBS systems of any baud rate, and work correctly.

Using This Procedure

To use this procedure, first load QINSTALL.COM, which is the install

program for Qmodem.  Once the values match the examples below, you will

then be able to initialize your modem correctly so that it will be

easily switchable between PCBoard and Qmodem.  Each section must be

followed in sequence.  ASSUME NOTHING.  All steps are explicit.  I'll

be happy to explain my reasons for any of the commands indicated.

HST Physical Switch Settings

It doesn't matter how your modem came set when you received it.  The

values indicated here guarantee compatibility.  Basically, all switches

are up ("on") except for switches number 3, 5, and 8.

+--------+   +-------------------+

ON    U  P       U U . U . U U . U U  ON

OFF               . . D . D . . D . .  OFF

+--------+   +-------------------+

1 2 3 4 5 6 7 8 9 1

0

HSTPCBQM Page 1

Section 1.0  Qmodem Initialization

The following is EXACTLY what you should have upon loading Qinstall, and

the subsequent commands will take through each required step.

Qinstall - Installation Program for Qmodem SST v3.1

1) Cosmetics     : Colors & Windows

2) Hardware      : Communication parameters

3) Directories   : File and Path Name parameters

4) RunTime Setup : Set Defaults for many features

S) Save & Exit   : Save Configuration and Exit to DOS

Q) Exit to DOS   : No Changes Saved

Option >2

Qinstall - Hardware : Modem & Port Setup

1) Modem Setup #1   Modem Initialization and Timing

2) Modem Setup #2   Modem Return Codes and Commands

3) Port  Setup      Comm Port Addresses and IRQs

X) Exit back to Main Menu

Option >1

Qinstall - Modem Communication Parameters

Baud       Data   Parity   StopEnter initial Modem settings  [19200]    [8]     [N]     [1]

String to send BEFORE changing Baud rate [              ]

String to send AFTER changing Baud rate  [{             ]

How many seconds should I wait between Redial attempts [10   ]

How many seconds should I wait before Cancelling a call [55  ]

How many milliseconds should be inserted between each character

when sending the "Dial" command to the modem [0    ]

What Comm port is your modem attached to [1-8] [1]

(Press F10 to accept changes and return to previous screen)

Qinstall - Hardware : Modem & Port Setup

1) Modem Setup #1   Modem Initialization and Timing

2) Modem Setup #2   Modem Return Codes and Commands

3) Port  Setup      Comm Port Addresses and IRQs

X) Exit back to Main Menu

Press F1 for Help, F10 to Accept changes, or Esc to Exit

(Press F10 to accept changes and return to previous screen)

Qinstall - Modem Dial and Redial Parameters

Message your modem returns when connected . . . . . . . [CONNECT       ]

Four messages your modem returns when Dialing was unsuccessful                   [NO CARRIER    ]    [BUSY          ]    [NO DIAL TONE  ]    [NO CONNECT    ]

Text sent to cancel a Redial attempt  . . . . . . . . . [{{{           ]

Text received that cancels a Redial attempt . . . . . . [OK            ]

Modem Initialization text . . [ATZ{                                    ]

Modem Dial Prefix [ATDT          ]    Modem Dial Suffix [{             ]

Text sent to Hang up the Modem  . . . . . . . . . . . . [~~+++~~ATH{   ]

Character (if any) to be sent twice in XMODEM transfers [  ]

Press F1 for Help, F10 to Accept changes, or Esc to Exit

(Press F10 to accept changes and return to previous screen)

Qinstall - Installation Program for Qmodem SST v3.1

1) Cosmetics     : Colors & Windows

2) Hardware      : Communication parameters

3) Directories   : File and Path Name parameters

4) RunTime Setup : Set Defaults for many features

S) Save & Exit   : Save Configuration and Exit to DOS

Q) Exit to DOS   : No Changes Saved

Option >S

(Now you have saved your new default values to Qmodem.cnf and you can call

Qmodem itself to get the modem ready.)

Section 2.0  Default Modem values.

The US Robotics Courier HST comes with factory defaults.  To get back to

these values you must enter AT&F to restore to these original values.  Even

if you have never done AT&W command, do so now to ensure proper baseline

values.

The following example graphically displays the procedure used while in

Qmodem.  You should be able to type ATI4 at any time to get the current

values for the modem.

AT&F

OK

ATI4

USRobotics Courier 9600 HST Settings....

C=1  E=1  F=1  M=1  Q=0  V=1  X=1  B=1

BAUD=19200  PARITY=N  WORDLEN=8

DIAL=PULSE  ON HOOK    TIMER

&A1  &B0  &G0  &H0  &I0  &M4

&N0  &P0  &R1  &S0  &Y1

S00=000  S01=000  S02=043  S03=013

S04=010  S05=008  S06=002  S07=030

S08=002  S09=006  S10=007  S11=070

S12=050  S13=000  S14=001  S15=000

S16=000  S17=000  S18=000  S19=003

S20=000  S21=010  S22=017  S23=019

OK

(Now make the following commands to get your modem to the new default values)

ATX4 &B1 &H1 &R2 &S1 &W

OK

ATI4

USRobotics Courier 9600 HST Settings....

C=1  E=1  F=1  M=1  Q=0  V=1  X=4  B=1

BAUD=19200  PARITY=N  WORDLEN=8

DIAL=TONE   ON HOOK    TIMER

&A1  &B1  &G0  &H1  &I0  &M4

&N0  &P0  &R2  &S1  &Y1

S00=000  S01=000  S02=043  S03=013

S04=010  S05=008  S06=002  S07=030

S08=002  S09=006  S10=007  S11=070

S12=050  S13=000  S14=001  S15=000

S16=000  S17=000  S18=000  S19=003

S20=000  S21=010  S22=017  S23=019

Reviewing the commands for this section:

AT&F                       Restores modem to factory default values

ATI4                       Displays current values (factory default)

ATX4 &B1 &H1 &R2 &S1 &W    Write new default values to modem NRAM

ATI4                       Displays current values (your default)

Section 3.0  PCBoard 11.x Modem Initialization

Below are the "Modem Data" screen from PCBoard Setup (PCBSETUP.EXE) which you

must set to match EXACTLY on the "Modem Initialization String" and the

"Modem Off-Hook String".  All other values are at your own discretion.  The

values shown are from my system here in Atlantic City.

�����������������������������������������������������������������������������ͻ

� 06-21-87 �                      Modem Data                       � 22:33:30 ������������������������������������������������������������������������������ͼ

Seconds to wait for Carrier: 15       (5 to 60 Seconds)

Modem Initialization String: ATM0H0S2=255

Modem Off-Hook String: ATM0H1

Communications Port: COM1:    (COM1:, COM2:, or NONE)

Baud Rate to Open Modem At: 19200    (300, 1200, 2400, 9600, 19200)

Allow 300 Baud Callers Access: Y        (Y/N)

Start Time for Above (hh:mm): 00:01    (00:01 to 23:59)

End Time for Above (hh:mm): 24:00    (00:01 to 24:00)

(Be sure to save these values to PCBOARD.DAT by saying "Y" as you exit

PCBSETUP.)

Note: The value above "Seconds to wait for Carrier" must be equal to or

shorter than the S7 value in the modem.  Since S7 defaults to 30 seconds

and we have not changed it, 15 is an acceptable value.

If you were to examine the modem while PCBoard is loaded and waiting for calls,

here is what you would see:

ATI4

USRobotics Courier 9600 HST Settings....

C=1  E=1  F=1  M=1  Q=0  V=1  X=4  B=1

BAUD=19200  PARITY=N  WORDLEN=8

DIAL=TONE   ON HOOK    TIMER

&A1  &B1  &G0  &H1  &I0  &M4

&N0  &P0  &R2  &S1  &Y1

S00=000  S01=000  S02=255  S03=013

S04=010  S05=008  S06=002  S07=030

S08=002  S09=006  S10=007  S11=070

S12=050  S13=000  S14=001  S15=000

S16=000  S17=000  S18=000  S19=003

S20=000  S21=010  S22=017  S23=019

OK

End of Section 3.0

Section 4.0  Enhanced Commands

You may make certain addition commands to your system to make life easier

for dialing out and for your BBS users.  Here are just a few.

Qmodem Enhancements:

Modem Initialization text . . [ATZ{~~ATS10=10S11=45{                   ]

The above example would allow your connection to stay established after a

1 second carrier loss.  This might help survive a  "Call Waiting" tone,

or perhaps a little longer like S10=15 would help better.  The S11=45

can be experimented with.  It is the speed of the tones sent to the phone

company to call a number.  I have mine set down to S11=37 here and it

works reliably.  The 45 milliseconds shown has worked just about everywhere.

PCBSetup Enhancements:

Modem Initialization String: ATM0H0S2=255S10=10