Last modified: March 6, 1998  ( Draft version )

LAN to LAN Connection Routing IPX (with One NetWare Server) over L2TP tunnel

Note: The IPX network numbers shown above are examples only!

This application note explains how to set up a workstation on a branch office network to use a Prestige to make a connection to corporate network Prestige to route IPX packets over the Internet.  In this application, the workstation in branch office network has NetWare client software, such as Windows 95, so that you will be able to log into the NetWare server on the corporate  network.  Of course, the two Prestiges above must support L2TP function. Note here, that your ISPs on both side of Internet connect do not need to provide L2TP function.  The L2TP tunnel will be created between 2 Prestiges directly, and this is what we call Direct Mode Tunneling.

Some special notes here is that your P128+(B) located in the corporate network is better to have a static IP address from the ISP. The reason for this is that when P128+(A) wants to create the tunnel, it needs to send a setup packet as an IP packets, and the destination will be P128+(B)’s IP address.  Therefore, when P128+(A) sends the packet, the ISPs must know how to route the packet.  If your P128+(B) on the corporate network is going to have a dynamic assigned IP address, then you have to configure the IP address into P128+(A) every time you are going to setup the tunnel to your corporate network.  On the other hand, the P128+(A) on the branch office network does not need a static IP address. This is because that P128+(B) will answer P128+(A)’s tunnel setup request to whatever IP address that P128+(A) is using.

There will be three items that you need to set up.  These are the workstation and the two Prestiges.

I.	Workstation Setup (NetWare Client)

To set up the workstation (W/S), you will need to set the following parameters:

- IPX Frame Type - First determine the frame type that Prestige is configured, and set the same frame type on the NetWare client workstation. ( For Windows 95, it’s better to set the frame type to ‘auto’ in Network Setup. )

- Server Name - In some cases, you may need the name that has been configured for the NetWare server you wish to login to.  In the previous example, that would be ‘Server_1’.

The procedure for configuring these parameters for your workstation may differ depending on the type of IPX client software you are using on your workstation.  If you are unfamiliar with how to set these parameters, you can refer to the technical notes corresponding to your software.

- DOS workstation w/ NetWare Client (VLM) software

- Windows 95/NT - see section ‘LAN-to-LAN’

II.	Prestige P128+(A) Setup

In order to configure the Prestige P128+(A) on the NetWare client side for this application, make sure you have the following menus configured correctly.

Note that the fields are indicated in bold type, and Menus such as ISDN setup will not be included in this application note.

Menu 1:

Menu 1 - General Setup

System Name= P128A

Location= branchOffice

Contact Person's Name= User

Route IP= Yes

Route IPX= Yes

Bridge= No

Press ENTER to Confirm or ESC to Cancel:

- In menu 1, make sure the Route IPX and Route IP fields are set to ‘Yes’.

( Menu 2 is skipped in this application note, and you need to make sure that you’ve configure Menu 2 for your Prestige to make outcall. )

Menu 3.3:

Menu 3.3 - Novell IPX Ethernet Setup

Seed Router= Yes

Frame Type 802.2= Yes

IPX Network #= 00002222

Frame Type 802.3= No

IPX Network #= N/A

Frame Type Ethernet II= No

IPX Network #= N/A

Frame Type SNAP= No

IPX Network #= N/A

Press ENTER to Confirm or ESC to Cancel:

- Set the Seed Router field to ‘Yes’, since there is no Novell Server connected to P128+(A) LAN.

- Make sure that you have set the correct  Frame Type(s) to ‘Yes’.  You should enable the frame type or types that the NetWare client is running.  Please note that it is possible to set more than one frame type to ‘Yes’. If you are using Windows 95 as the Netware Client, then the easiest way is to set the IPX frame type to ‘Auto’ in the Network setup.

- Set IPX Network # to any valid unique network number that is not being used on that network.  In our example, we chose to use ‘00002222’.

Menu 10:

Menu 10 - Tunnel Endpoint Setup

1. P_CORP

2. ________

3. ________

4. ________

Enter Node # to Edit:

- Menu 10 is a new screen introduced for L2TP tunneling. The purpose of this configuration is to specify where to create the tunnel to and what’s the password (secret) for this tunnel.

- Each tunnel has two endpoints, and in this application example, P128+(A) and P128+(B) are the two endpoints. Therefore, we need to configure the ‘Tunnel Endpoint Setup’ in both Prestiges.

Menu 10.1

Menu 10.1 - Tunnel Endpoint Profile

Endpoint Name= P_CORP

Active= Yes

My Host Name= p_remote

Peer Host Name= p_corp

shared Secret= ********

IP Address= 200.1.1.175

Press ENTER to Confirm or ESC to Cancel:

- Specify a name for identification of the tunnel. (You can configure Prestige to tunnel to 4 different places/network/Prestiges.)

- Specify the ‘Host Name’ information for identify which endpoint.  Specify the ‘Secret’ for the tunnel, and note that the secret must be the same configured in both P128+(A) and P128+(B).

- Specify the IP address that the tunnel is going to create to.  This IP address must be a legal IP address from the Internet.  Note: In this example that branch office network need to reach the network in corporate, so branch office Prestige P128+(A) will initiate the tunnel setup. Therefore, you must provide an IP address that your router and your ISP knows how to route the tunnel setup packet to.  ( See the special notes at the beginning of this application note. )

Menu 11:

Menu 11 - Remote Node Setup

1. ISP_A (ISP, SUA)

2. ToServer

3. ________

4. ________

Enter Node # to Edit:

- The ISP_A is the remote node that you need to connect to the ISP.

- The ToServer is the remote node that will go through the L2TP tunnel to connect to corporate IPX network.

Menu 11.1

Remote Node to ISP:

Menu 11.1 - Remote Node Profile

Rem Node Name= ISP_A

Active= Yes                          Route= IP

Call Direction= Outgoing             Bridge= No

Tunneling Mode= None

Endpoint Index= N/A                Edit PPP Options= No

Rem IP Addr= 0.0.0.0

Incoming:                            Edit IP/IPX/Bridge= No

Rem Login=                         Telco Option:

Rem Password= ********               Transfer Type= 64K

Rem CLID= N/A                        Allocated Budget(min)= 0

Call Back= N/A                       Period(hr)= 0

Outgoing:                            Session Options:

My Login= user                       Input Filter Sets=

My Password= ********                Output Filter Sets=

Authen= CHAP/PAP                     Call Filter Sets=

Pri Phone #= 20001                   Idle Timeout(sec)= 300

Sec Phone #=

Press ENTER to Confirm or ESC to Cancel:

- Normal IP remote needs to setup the Tunneling Mode to ‘None’.

Remote Node for IPX over L2TP tunnel:

Menu 11.1 - Remote Node Profile

Rem Node Name= ToServer

Active= Yes                          Route= IPX

Call Direction= Outgoing             Bridge= No

Tunneling Mode= Direct

Endpoint Index= 1                  Edit PPP Options= No

Rem IP Addr= N/A

Incoming:                            Edit IP/IPX/Bridge= No

Rem Login=                         Telco Option:

Rem Password= ********               Transfer Type= 64K

Rem CLID= N/A                        Allocated Budget(min)= 0

Call Back= N/A                       Period(hr)= 0

Outgoing:                            Session Options:

My Login= remote                     Input Filter Sets=

My Password= ********                Output Filter Sets=

Authen= CHAP/PAP                     Call Filter Sets=

Pri Phone #= N/A                     Idle Timeout(sec)= 300

Sec Phone #= N/A

Press ENTER to Confirm or ESC to Cancel:

- Setup the Tunneling Mode to Direct, and the Endpoint Index to the index in Menu 10  Tunnel Endpoint Setup.

- The Call Direction should be set to ‘Outgoing’.

- Make sure the remote node is enabled by setting Active to ‘Yes’.

- My Login and My Password is the login information that this Prestige will use to connect to the remote router.

·	Route should be set to ‘IPX’.

- Set Edit IP/IPX/AppleTalk/Bridge to ‘Yes’ and press ENTER.

Menu 11.3:

Menu 11.3 - Remote Node Network Layer Options

IP Options:                          IPX Options:

Rem IP Addr:                         Dial-On-Query= Yes

Rem Subnet Mask= N/A                 Rem LAN Net #= 00000000

My WAN Addr= N/A                     My WAN Net #= 00000000

Single User Account= N/A             Hop Count= 1

Server IP Addr= N/A                Tick Count= 2

Metric= N/A                          W/D Spoofing(min)= 3

Private= N/A                         SAP/RIP Timeout(min)= 3

RIP Direction= N/A

Version= N/A                     Bridge Options:

Dial-On-Broadcast= N/A

Ethernet Addr Timeout(min)= N/A

Enter here to CONFIRM or ESC to CANCEL:

- Setup the Dial-On-Query to Yes, so the IPX query packets to look for IPX servers will trigger the tunnel setup, and which will trigger the outcall to the ISP automatically.

- The Remote LAN Net # can be left as all 0s or the internal network number of the IPX server in the corporate network. This number can be obtained by that network’s system administrator.  In our example, this value is ‘00001111’.

- SAP/RIP Timeout can be set to a greater value, so if your ISDN connection drops for an extended period of time, the Prestige will still retain the proper SAP and RIP tables.  This value should be the same as the W/D Spoofing field in the server-side Prestige.

III.	Prestige P128+(B) Setup

In order to configure the Prestige P128+(B) on the NetWare server side, make sure you have the following menus configured correctly.

Note that the fields are indicated in bold type.

Menu 1:

Menu 1 - General Setup

System Name= P128B

Location= corpOffice

Contact Person's Name= Admin

Route IP= Yes

Route IPX= Yes

Bridge= No

Press ENTER to Confirm or ESC to Cancel:

- In menu 1, make sure the Route IPX and Route IP fields are set to ‘Yes’.

Menu 3.2:

Menu 3.2 - TCP/IP and DHCP Ethernet Setup

DHCP Setup:

DHCP= None

Client IP Pool Starting Address= N/A

Size of Client IP Pool= N/A

Primary DNS Server= N/A

Secondary DNS Server= N/A

TCP/IP Setup:

IP Address= 200.1.1.175

IP Subnet Mask= 255.255.255.0

RIP Direction= Both

Version= RIP-2B

Press ENTER to Confirm or ESC to Cancel:

- Make sure the IP address is the IP address that you entered in P128+(A)’s Menu 10, so the tunnel can be created correctly.

- DHCP setup should match your IP network setup.

Menu 3.3:

Menu 3.3 - Novell IPX Ethernet Setup

Seed Router= No

Frame Type 802.2= No

IPX Network #= N/A

Frame Type 802.3= Yes

IPX Network #= N/A

Frame Type Ethernet II= No

IPX Network #= N/A

Frame Type SNAP= No

IPX Network #= N/A

Press ENTER to Confirm or ESC to Cancel:

- Set that Seed Router field to ‘No’, since there’s is a Novell Server in the same network.

- Make sure that you have set the correct  Frame Type(s) to ‘Yes’.  You should enable the frame type or types that the NetWare server is running.  Please note that it is possible to set more than one frame type to ‘Yes’, but make sure that the frame type your server is configured, you Prestige must turn on that frame type.

Menu 10:

Menu 10 - Tunnel Endpoint Setup

1. P_REMOTE

2. ________

3. ________

4. ________

Enter Node # to Edit:

- Setup one tunnel endpoint to P128+(A).

Menu 10.1

Menu 10.1 - Tunnel Endpoint Profile

Endpoint Name= P_REMOTE

Active= Yes

My Host Name= p_corp

Peer Host Name= p_remote

shared Secret= ********

IP Address= 192.1.1.1

Press ENTER to Confirm or ESC to Cancel:

- Specify a name for identification of the tunnel

- Specify the ‘Host Name’ information for identifying which endpoint.  Specify the ‘Secret’ for the tunnel, and note that the secret must be the same configured in both P128+(A) and P128+(B).

- Specify the IP address that the tunnel is going to create to if P128+(A) has a static IP address from the ISP. If not, then enter any IP address that doesn’t include in your LAN network address range.  When P128+(B) receives P128+(A)’s tunnel setup request, it will use the IP address the P128+(A) is currently using to communicate with P128+(A).  ( Note, this is why that your P128A on the branch office doesn’t have to have a static IP address. )

Menu 11:

Menu 11 - Remote Node Setup

1. ISP_B

2. IPXremot

3. ________

4. ________

Enter Node # to Edit:

- The ISP_B is the remote node that you need to connect to the ISP.

- The IPXremot is the remote node that will go through the L2TP tunnel to connect to branch office’s IPX network.

Menu 11.1

Menu 11.1 - Remote Node Profile

Rem Node Name= IPXremot

Active= Yes                          Route= IPX

Call Direction= Incoming             Bridge= No

Tunneling Mode= Direct

Endpoint Index= 1                  Edit PPP Options= No

Rem IP Addr= N/A

Incoming:                            Edit IP/IPX/Bridge= No

Rem Login= remote                  Telco Option:

Rem Password= ********               Transfer Type= 64K

Rem CLID=                            Allocated Budget(min)= 0

Call Back= N/A                       Period(hr)= 0

Outgoing:                            Session Options:

My Login=                            Input Filter Sets=

My Password= ********                Output Filter Sets=

Authen= N/A                          Call Filter Sets=

Pri Phone #= N/A                     Idle Timeout(sec)= 300

Sec Phone #= N/A

Press ENTER to Confirm or ESC to Cancel:

- Setup the Tunneling Mode to Direct, and the Endpoint Index to the index in Menu 10  Tunnel Endpoint Setup.

- The Call Direction should be set to ‘Incoming’.

- Make sure the remote node is enabled by setting Active to ‘Yes’.

- Rem Login and Rem Password is the login information that the remote router will use to connect to this Prestige.

·	Route should be set to ‘IPX’.

- Set Edit IP/IPX/AppleTalk/Bridge to ‘Yes’ and press ENTER.

Menu 11.3:

Menu 11.3 - Remote Node Network Layer Options

IP Options:                          IPX Options:

Rem IP Addr: N/A                     Dial-On-Query= N/A

Rem Subnet Mask= N/A                 Rem LAN Net #= 00000000

My WAN Addr= N/A                     My WAN Net #= 00000000

Single User Account= N/A             Hop Count= 1

Server IP Addr= N/A                Tick Count= 2

Metric= N/A                          W/D Spoofing(min)= 3

Private= N/A                         SAP/RIP Timeout(min)= 3

RIP= None

Bridge Options:

Dial-On-Broadcast= N/A

Ethernet Addr Timeout(min)= N/A

Enter here to CONFIRM or ESC to CANCEL:

- Dial-On-Query should be ‘No’

- The Rem LAN Net # can be left as all 0s or the internal network number that is assigned to the Prestige P128+(A) in menu 3.3.  In our example, this would be ‘00002222’.

- W/D Spoofing can be set to a greater value, so if your ISDN connection drops for an extended period of time, the Prestige will still retain the proper SAP and RIP tables.  This value should be the same as the SAP/RIP Timeout field in the client-side Prestige.