# ZyXEL ZyNOS 2.21(I.02) for P100MH/P100WH

# Release Note /Manual Supplement

Date: July 21, 1999

This document describes the enhancements in ZyXEL Prestige product line since the last manual printing. The bug fixes section describes bug fixed since ZyNOS 2.21 (I.01).

## Supported Platforms

ZyNOS 2.21(I.02) supports Prestige models: P100MH and P100WH.

## Changes not Compatible to Previous Versions:

Login Script

In our previous script design, every sendstring will automaitcally followed by a <CR>. So, we can not only send <CR> in our login script. In some cases, people expect to send <CR> only. But our original script design can't do so and cause some troubles.

In order to make our script to support it, we implement some macro extension in send string only. These macros can appear in any place within send string. They are:

$USERNAME : username setting in remote node.

$PASSWORD: password setting in remote node.

^A ~ ^Z : means ASCII Code 0x01 ~ 0x1A (^M means <CR>, ^J means <LF>).

^^ : means '^'.

(Note: These macros only valid in Send String. Expect String doesn't have any macros).

The send string will not automatically follow by a <CR>. So, user must append "^M" at the end of sendstring.

Example: Compuserv Login script.

Menu 4.1 - Internet Setup Script

Active= Yes

Set 1:    			Set 5:

Expect=                          Expect= please...

Send= ^M			  Send=

Set 2:          			Set 6:

Expect= Host Name:       	  Expect=

Send= CIS^M               	  Send=

Set 3:

Expect= User ID:

Send= $USERNAME/GO:PPPCONNECT^M

Set 4:

Expect= Password:

Send= $PASSWORD

Press ENTER to Confirm or ESC to Cancel:

To verify the result of script execution, user can use these 2 methods:

(1) Use manual dial (In Menu 24.8, use "dev dial n", n is remote node,or by menu 24.4.3). The whole login process will be displayed.

(2) After dialing is complete, check the trace log. (In Menu 24.8, use "sys trcl disp" to display the trace log). It will show something like this:

ras> sys trcl disp

0        939 PINI dotcp

1        939 PINI doirtt

2        939 PINI dotcp

3        939 PINI dowindow

4        939 PINI dotcp

5        939 PINI doceiling

6        940 PSSV socket:af=0,type=1,prot=0

7        940 PP0a socket:af=0,type=1,prot=0

8        940 PP0a bind:name=542e40,namelen=16,s=8194

9        940 PP0a itop:s=8194

10        940 PP0a checkipaddr:fam=0,len=16

11        940 PP0a so_udp_bind:up=5f82e8

12        940 PP08 lookup_udp: socket=542dd8

13        940 PP0a recv_mbuf:from=542e30,s=8194,flags=0

14        940 PP0a itop:s=8194

15        940 PP0a so_udp_recv:up=5f82e8,fromlen=5516836

16       2322 PP07 DIALING dev=3 ch=1..........

17       2322 PP07 OUTGOING-CALL phone(0842111092)

18       2855 PP05 SCRIPT Started

19       2855 PP05 ExpectStr 1 Received

20       2945 PP05 SendStr 1 Transmitted

21       2974 PP05 ExpectStr 2 Received

22       3061 PP05 SendStr 2 Transmitted

23       3092 PP05 ExpectStr 3 Received

24       3181 PP05 SendStr 3 Transmitted

25       3271 PP05 ExpectStr 4 Received

26       3361 PP05 SendStr 4 Transmitted

27       5317 PP05 ExpectStr 5 Received

28       5401 PP05 SendStr 5 Transmitted

29       5411 PP05 ExpectStr 6 Received

30       5501 PP05 SendStr 6 Transmitted

31       5591 PP05 SCRIPT Success

32       5591 PP07 CALL CONNECT speed<115200> type<3> chan<1>

33       5592 PP08 ebp=542f2c,seqNum=1 wan01-XMIT len:29 call=1

0000: ff 03 c0 21 01 00 00 19 01 04 05 f4 02 06 00 0a

0010: 00 00 05 06 00 00 15 d7 08 02 0d 03 06

User can check if his script login is completed or not.

## Bug fixes:

- Fix bug: "sys trcl call" will display some unnecessary messages.

- Fix bug: Sometimes FTP will have trouble in SUA mode. (Send command to FTP server will fail)

# ZyXEL ZyNOS 2.21(I.01) for Prestige 100MH/100WH

# Release Note /Manual Supplement

Date: June 15, 1999

This document describes the enhancements in ZyXEL Prestige product line since the last manual printing. The bug fixes section describes bug fixed since ZyNOS 2.21.

## Supported Platforms

ZyNOS 2.21(I.01) supports Prestige models: P100MH and P100WH.

## Bug fixes/Changes

- Fix bug: Compuserv login will have problem when using packet trigger dial, but manual dial doesn't have the problem.

- Fix bug: If a channel is used to dial to ISP with script. After this ISP connection is dropped and somebody dial in, this script will apply to this incoming call.

- Fix bug: If the 1st script is set to wait for nothing (just sending), sometimes there will have problem because we send it too fast. So, we will delay 1 second before sending.

- Fix bug: User can't reset budget for remote node 5 -8 in menu 24.9.2 (Budget Management).

- Fix bug: When user use manual dial, the system tracelog will not contain the packet log.

- Fix bug: If "call back = Optional" is set in menu 14.1. When user dial in and cancel the callback, the connection will fail.

- Now, the script process will be stored in system tracelog. When the tracelog is turn on (use C/I command "sys trcl sw on" to turn on it), and after the script is executed, you can use "sys trcl disp" to check the script process.

- Add new feature: Add some new C/I commands (for supporting)

"sys trcd brief"	- Online display brief descriptions of packets in packet log.

"sys trcd parse" - Online display detail descriptions of packets in packet log.

"sys trcp brief"	- Display brief descriiptions of packets in packet log.

"sys trcp parse"	- Display detail descriptions of packets in packet log.

## How To Upgrade The Prestige

1.  Here are the files needed for upgrading your Prestige:

| Product | Router Firmware | Default Configuration File |
| --- | --- | --- |
| P100MH | P100MH.BIN | P100MH-R.ROM (Rockwell) P100MH-T.ROM (TI) |
| P100WH | P100WH.BIN | P100WH.ROM |

2.  Upgrade F/W by XMODEM via console.

(1) In SMT Main Menu, Enter menu 24.2.2 to change baudrate of Console.

(2) In SMT Main Menu, Enter menu 24.7.1 and following the instructions to upgrade Firmware.

Commands:

ATUR:  Upload Firmware file via XMODEM

3.  If you don‘t want to keep your settings, enter SMT menu 24.7.2 to upload the Default Configuration File.

Commands:

ATUR3:  Upload the Default configuration File.

Note1 :

You don't need to upload this file if you are upgrading from release 2.1x unless you want to reset all configurations to the factory default.

Note2 :

To distinguish the internal modem of the P100MH is TI-based or Rockwell Based, Please check the the FCC label on the BOTTOM side of your P100MH. If the model name on it is P100MH-R, then your modem module is Rockwell-Based. If the model name on it is P100MH-T, then your modem module is TI-Based.

# ZyXEL ZyNOS v2.21 for P100MH/P100WH/P128MH/P153/P153x

# Release Note /Manual Supplement

Date:  March 4, 1999

This document describes the enhancements in ZyXEL Prestige product line since the last manual printing. The bug fixes section describes bug fixed since ZyNOS v2.1x.

The New Features section lists the new features added to this release. Please read through the Enhancement Details section for detail information. A new section, Changes not Compatible to Previous Versions, is added to this release notes. It is important to read it too.

## New Features:

More Remote Nodes and IP Static Routes

In this release, the number of Remote Nodes and IP Static Routes have increased.

DNS Proxy

This feature allows you to use DNS Settings provided by the ISP automatically.

SUA Enhancements

In this release, SUA has been enhanced to support the popular applications: Microsoft’s tracert under Windows9x and Windows NT, CuSeeMe, IRC, RealAudio, VDOLive, Quake, and PPTP. Besides above, multiple SUA servers are supported.

Nailed-up Connection

This feature allows a dial-up line to emulate a lease line.

Upgrade internal MODEM Firmware (P100MH/P128MH Only)

Users can upgrade modem F/W using XMODEM in menu 24.7.

Upgrade Firmware via LAN

User can upgrade ZyNOS F/W by TFTP. It is not recommended you upgrade your firmware via WAN even though it may work.

Backup and Restore Configuration File via LAN

Users can backup and restore configuration by TFTP.

Upgrade internal MODEM Firmware via LAN(P100MH/P128MH Only)

Users can also upgrade Internal Modem F/W by TFTP. It is also not recommended you upgrade internal modem firmware via WAN even though it may work.

## Changes not Compatible to Previous Versions:

Filter Rules

Conceptually, there are two categories of filter rules: device and protocol. The Generic filter rules belong to the device category; they act on the raw data from/to LAN and WAN. The IP and IPX filter rules belong to protocol category; they act on the IP and IPX packets.

In ZyNOS v2.1x, device and protocol (IP or IPX) filter rules could be intermixed in a filter set. This is no longer permitted in this release. This design change was provoked by the following dilemma in applying the TCP/IP filtering to the SUA (Single User Account) connections:

In ZyNOS v2.1x, the Prestige applied the input filter rules to the incoming packets immediately after receiving them from the ISDN line; and applied the call and output filter rules to the outgoing packets immediately prior to sending them out the ISDN port. With this approach, the call and output filter rules were applied to the output IP packets whose source IP address and port number had already been converted into different values by SUA. Thus, the call and output TCP/IP filter rules did not work for a SUA connection if the filter rules were based on the local network IP address and port number. This same limitation also applied to the input filter rules as well.

Figure 1 shows the ZyNOS v2.1x logic flow for a packet from LAN to WAN (->), and a packet from WAN to LAN (->). Suppose that the packet from LAN to WAN with source IP address 192.168.1.33 and port number 1023; and the WAN IP address is 203.205.115.6 that could be dynamic or static. But the port number (4034 in this example) generated by SUA is always dynamic and unpredictable. The sequence of the logic flow for the packet from LAN to WAN is:

- LAN input filter sets.

- SUA converts the source IP address from 192.168.1.33 to 203.205.115.6 and port number from 1023 to 4034.

- WAN call and output filter sets. It does not work if the filter rules are based on IP address 192.168.1.33 and port number 1023.

- The sequence of the logic flow for the packet from WAN to LAN is:

- WAN input filter sets. It does not work if the filter rules are based on IP address 192.168.1.33 and port number 1023.

- SUA converts the destination IP address from 203.205.115.6 to 192.168.1.33 and port number from 4034 to 1023.

- LAN output filters sets.

In order to allowing users to specify the local network IP address and port number in the filter rules with SUA connections, the TCP/IP filter function has to be executed before SUA for WAN outgoing packets and after the SUA for WAN incoming IP packets. But at the same time, the Generic filter rules must be applied at the point when the Prestige is receiving and sending the packets; i.e. the WAN interface. So, the execution sequence has to be changed. The v2.21 logic flow is shown in Figure 2 and the sequence of the logic flow for the packet from LAN to WAN is:

- LAN device and protocol input filter sets.

- WAN protocol call and output filter sets. It works now because SUA does not convert the local IP address and port number to WAN IP address and port number yet.

- SUA converts the source IP address from 192.168.1.33 to 203.205.115.6 and port number from 1023 to 4034.

- WAN device output and call filter sets.

The sequence of the logic flow for the packet from WAN to LAN is:

- WAN device input filter sets.

- SUA converts the destination IP address from 203.205.115.6 to 192.168.1.33 and port number from 4034 to 1023.

- WAN protocol input filter sets. It works now because SUA has converted the destination IP address and port number to local IP address and port number.

- LAN device and protocol output filter sets.

To accommodate the above change, Generic and TCP/IP (and IPX) filter rules must now be in different filter sets. The v2.21 SMT will detect and prevent the mixing of different category rules within any filter set in Menu 21. In the following example, you will receive an error message “Protocol and device filter rules cannot be active together" if you try to activate a TCP/IP (or IPX) filter rule in a filter set that has already had one or more active Generic filter rules. You will receive the same error if you try to activate a Generic filter rule in a filter set that has already had one or more active TCP/IP (or IPX) filter rules.

Menu 21.1.1:

Menu 21.1.1 - Generic Filter Rule

Filter #: 1,1

Filter Type= Generic Filter Rule

Active= Yes

Offset= 0

Length= 0

Mask= N/A

Value= N/A

More= No                       Log= None

Action Matched= Check Next Rule

Action Not Matched= Check Next Rule

Menu 21.1.2:

Menu 21.1.2 - TCP/IP Filter Rule

Filter #: 1,2

Filter Type= TCP/IP Filter Rule

Active= Yes

IP Protocol= 0      IP Source Route= No

Destination: IP Addr= 0.0.0.0

IP Mask= 0.0.0.0

Port #= 0

Port # Comp= None

Source: IP Addr= 0.0.0.0

IP Mask= 0.0.0.0

Port #= 0

Port # Comp= None

TCP Estab= N/A

More= No            Log= None

Action Matched= Check Next Rule

Action Not Matched= Check Next Rule

Press ENTER to Confirm or ESC to Cancel:

Saving to ROM.  Please wait...

To separate the device and protocol filter categories; two new menus, Menu 11.5 and Menu 13.1, have been added, as well as some changes made to the Menu 3.1, Menu 11.1, and Menu 13. The changed fields are marked bold in the following menus:

Menu 3.1:

Menu 3.1 - General Ethernet Setup

Input Filter Sets:

protocol filters=

device filters=

Output Filter Sets:

protocol filters=

device filters=

Menu 11.1 (P100MH/P100WH):

Menu 11.1 - Remote Node Profile

Rem Node Name= test		Edit PPP Options= No

Active= No                  	Rem IP Addr= 0.0.0.0

Call Direction= Outgoing    	Edit IP= No

Edit Script Options= No

Incoming:

Rem Login= N/A          	Telco Option:

Rem Password= N/A           	  Allocated Budget(min)= 0

Rem CLID= N/A                	  Period(hr)= 0

Call Back= N/A               	  Nailed-Up Connection= No

Outgoing:

My Login= dchen             	Session Options:

My Password= ********      	  Edit Filter Sets= No

Authen= CHAP/PAP            	  Idle Timeout(sec)= 0

Pri Phone #= 10001

Sec Phone #=

Press ENTER to Confirm or ESC to Cancel:

Menu 11.1 (P153/P153x/P128MH):

Menu 11.1 - Remote Node Profile

Rem Node Name= ?  		Device Type= Modem

Active= Yes                   	Route= IP

Connection Type= Switch     	Bridge= No

Call Direction= Both         	Edit PPP Options= No

Incoming:                      	Rem IP Addr= ?

Rem Login= ?                 	Edit IP/IPX/Bridge= No

Rem Password= ?             	Edit Script Options= No

Rem CLID=                    	Telco Option:

Call Back= No               	  Allocated Budget(min)= 0

Outgoing:                      	  Period(hr)= 0

My Login=                    	  Nailed-Up Connection= No

My Password= ********      	Session Options:

Authen= CHAP/PAP            	  Edit Filter Sets= No

Pri Phone #= ?              	  Idle Timeout(sec)= 300

Sec Phone #=

Press ENTER to Confirm or ESC to Cancel:

Menu 11.5:

Menu 11.5 - Remote Node Filter

Input Filter Sets:

protocol filters=

device filters=

Output Filter Sets:

protocol filters=

device filters=

Call Filter Sets:

protocol filters=

device filters=

Menu 13 (P100MH/P100WH):

Menu 13 - Default Dial-in Setup

Telco Options:              	IP Address Supplied By:

CLID Authen= None           	  Dial-in User= No

IP Pool= Yes

PPP Options:                   	    IP Start Addr= 202.132.50.68

Recv Authen= CHAP/PAP

Compression= Yes             	Session Options:

Mutual Authen= No           	  Edit Filter Sets= No

PAP Login= N/A

PAP Password= N/A

Callback Budget Management:

Allocated Budget(min)=

Period(hr)=

Press ENTER to Confirm or ESC to Cancel:

Press Space Bar to Toggle.

Menu 13 (P153/P153x/P128MH):

Menu 13 - Default Dial-in Setup

Telco Options:           	IP Address Supplied By:

CLID Authen= None           	  Dial-in User= No

IP Pool= Yes

PPP Options:                 	    IP Start Addr= 202.132.50.65

Recv Authen= PAP             	    IP Count(1,3)= 3

Compression= Yes

Mutual Authen= No           	IPX Net Num Supplied By:

PAP Login= N/A             	  IPX Pool= No

PAP Password= N/A         	    IPX Start Net Num= N/A

Multiple Link Options:   	    IPX Count(2,16)= N/A

Max Ports= 3

Session Options:

Callback Budget Management:      Edit Filter Sets= No

Allocated Budget(min)=

Period(hr)=

Press ENTER to Confirm or ESC to Cancel:

Menu 13.1:

Menu 13.1 - Default Dial-in Filter

Input Filter Sets:

protocol filters=

device filters=

Output Filter Sets:

protocol filters=

device filters=

The SMT will also prevent you entering a protocol filter set configured in Menu 21 to the device filters field in Menu 3.1, 11.5, or 13.1, or entering a device filter set to the protocol filters field. Even though the SMT will prevent the inconsistency from being entered in v2.21, it is unable to resolve the intermixing problems existing in the filter sets that were configured before. Instead, when v2.21 translates the old configuration into the new format, it will verify the filter rules and log the inconsistencies. Please check the system log (Menu 24.3.1) before putting your device into production.

Running the Prestige with wrong filter rules may cause it to keep the WAN line perpetually active, and/or allow undesired traffic to pass to the outside world, and receive unwanted outside traffic. The first case may incur an enormous bill; the second may be a data security hazard.

In order to avoid operational problems later, the Prestige will disable its routing/bridging functions if there is an inconsistency among its filter rules.

Proofread your filter rules even if there is no warning message in system log, and observe the router's behavior carefully after upgrading/installation.

SNMP (P128MH/P153/P153x)

| SNMP Port Number | ZyNOS V2.1x | ZyNOS V2.2x |
| --- | --- | --- |
| 1 | wanif2 | enet0 |
| 2 | wanif1 | wan00 |
| 3 | wanif0 | wan01 |
| 4 | enif0 | wan02 |

In ZyNOS v2.1x, the Prestige reports logical channel status (wanif0, wanif1 and wanif2) by SNMP.. It can not reflect real port status because logic channel and physical port (wan00, wan01 and wan02) do not have a fixed mapping. A logical channel may map to multiple physical ports (Multilink PPP).

Now, the prestige reports physical port status by SNMP since ZyNOS 2.2x.

Because the SNMP port number definitions are changed, Please check the setting of your SNMP network management program (HP OpenLook, ...) and change it.

## Enhancement Details

More Remote Nodes and IP Static Routes

In this release, the number of remote nodes and numbers of IP static routes have increased as follows:

| Model | Number of remote nodes | IP static routes |
| --- | --- | --- |
| P100MH, P100WH | 8 | 8 |
| P153, P153x, P128MH | 12 | 8 |

DNS Proxy

If this feature is enabled, the Prestige will act as the DNS server to the local network. The Prestige will get the IP address of the actual DNS server from remote site via IPCP negotiation. So this feature works only if the remote site supports RFC 1877.

#### How to Configure DNS Proxy

- This feature is enabled if the value of the Primary DNS Server field in Menu 3.2 is 0.0.0.0. By default, this feature is enabled in a brand new Prestige. In this case, The Prestige will assign its IP address as the Primary DNS in the responses to the DHCP requests on the local network. A SMT restriction enforces the consistency between the Primary DNS server and Secondary DNS server fields in Menu 3.2 by skipping Secondary DNS Server field if the IP address of the Primary DNS Server field is 0.0.0.0.

- If this feature is disabled, the DHCP mechanism will be performed like before. Therefore, the Prestige will assign the values entered in Primary DNS server and Secondary DNS server fields in Menu 3.2 to the responses to the DHCP requests on the local network.

#### Functions

- Only the default route remote node will negotiate with the remote site for the DNS servers.

- If the default remote node is down and the Prestige receives a DNS request, then the Prestige will attempt to bring up the connection to the default remote node.

- If the default remote node is up, but no DNS server is negotiated, then all DNS requests to the Prestige will be dropped.

- If only one DNS server is negotiated or set in menu 3.2, then this will be the active DNS server for the rest of the session.

- If both Primary and Secondary DNS servers are negotiated or set in menu 3.2, then the Prestige will send the first DNS request to both DNS servers replacing the source IP address with its own WAN IP address.  The DNS that responds first will be the active DNS server for the rest of the session. The first response will be forwarded to the original client.  The second response will be discarded.

- If there is only one active DNS server, then all DNS request will be forwarded to that server.

- In the case that the Prestige negotiates both the Primary and Secondary DNS server, and the active DNS server has not been determined, all DNS request will be sent to both servers.

- When the Prestige receives a DNS request, it will save the request in a table to be match with the response from the DNS server. The Prestige will send the request to the DNS server(s) with a new ID that is unique to all outstanding DNS requests from the Prestige. The timeout for a request waiting for a response is 60 seconds.

SUA Enhancements

SUA has been enhanced to support the popular Internet applications: MS tracert, CuSeeMe, IRC, RealAudio, VDOLive, Quake, and PPTP. There is no configuration needed for supporting these applications.

For multiple SUA server support, the Server IP Addr field in Menu 4 and 11.2 has been removed and a new Menu 15 is added.

There are up to eight SUA servers can be configured in this menu. The first one is reserved for the default server, and it can not be changed. You only need to assign the IP address to this default server. For other SUA servers, you need to assign them both of the Port number and the IP address.

Menu 15 - Multiple Server Configuration

Port #		IP  Address

---------	-----------------

1. Default	0.0.0.0

2. 0		0.0.0.0

3. 0		0.0.0.0

4. 0		0.0.0.0

5. 0		0.0.0.0

6. 0		0.0.0.0

7. 0		0.0.0.0

8. 0		0.0.0.0

If the default server has been configured, the entire unknown (port number is not configured in this Menu, nor in SUA table) WAN incoming IP packets will be routed to this IP address. Otherwise, Prestige will discard them silently if they are neither Telnet (port number 23) nor Authentication (port number 113) packets. For Telnet packets, Prestige will perform as Telnet server in this case. For Authentication packets, Prestige will reject them.

The most often used port numbers are:

| Service | Port number |
| --- | --- |
| FTP | 21 |
| telnet | 23 |
| SMTP | 25 |
| DNS (Domain Name Server) | 53 |
| www-http (Web) | 80 |
| PPTP | 1723 |

Nailed-up Connection

If this feature is enabled in a remote node, this remote node will behave the same as a leased line connection even if the physical line is a dial-up line. A new option is added in Menu 4 and 11.1 in order to enable and disable this feature. The new field is marked in bold in the following menus:

Menu 4:

Menu 4 - Internet Access Setup

ISP's Name= ?

Pri Phone #= ?

Sec Phone #=

My Login=

My Password= ********

Single User Account= No

IP Addr= N/A

Edit Script Options= No

Nailed-Up Connection= No

Multilink= Off

Menu 11.1 (P100MH/P100WH):

Menu 11.1 - Remote Node Profile

Rem Node Name= test		Edit PPP Options= No

Active= No                  	Rem IP Addr= 0.0.0.0

Call Direction= Outgoing    	Edit IP= No

Edit Script Options= No

Incoming:

Rem Login= N/A          	Telco Option:

Rem Password= N/A           	  Allocated Budget(min)= 0

Rem CLID= N/A                	  Period(hr)= 0

Call Back= N/A               	  Nailed-Up Connection= No

Outgoing:

My Login= dchen             	Session Options:

My Password= ********      	  Edit Filter Sets= No

Authen= CHAP/PAP            	  Idle Timeout(sec)= 0

Pri Phone #= 10001

Sec Phone #=

Press ENTER to Confirm or ESC to Cancel:

Menu 11.1 (P153/P153x/P128MH):

Menu 11.1 - Remote Node Profile

Rem Node Name= ?  		Device Type= Modem

Active= Yes                   	Route= IP

Connection Type= Switch     	Bridge= No

Call Direction= Outgoing      	Edit PPP Options= No

Incoming:                      	Rem IP Addr= ?

Rem Login= ?                 	Edit IP/IPX/Bridge= No

Rem Password= ?             	Edit Script Options= No

Rem CLID=                    	Telco Option:

Call Back= No               	  Allocated Budget(min)= 0

Outgoing:                      	  Period(hr)= 0

My Login=                    	  Nailed-Up Connection= No

My Password= ********      	Session Options:

Authen= CHAP/PAP            	  Edit Filter Sets= No

Pri Phone #= ?              	  Idle Timeout(sec)= 300

Sec Phone #=

Press ENTER to Confirm or ESC to Cancel:

Functions:

- The “Nailed-Up Connection” field will be “N/A” unless your “Call Direction” field is set to “Outgoing”.

- Because the number of remote nodes is more than the number of lines in our Prestige, we always start dialing the first two remote nodes that fulfill the dedicated connection requirement.

- If it fails to make an outgoing call for the dedicated connection, the prestige will keep trying to connect the same remote node until the connection is built up.

- A remote node with dedicated connection doesn’t have any priority over any other remote nodes except it keeps dialing out until a connection is built up. That is to say, it is possible that other remote node connections may be up before the dedicated connections  (first come, first serve).

- If a dedicated connection is manually dropped, it will dial the same remote node again. But it may fail to dial because another connection has occupied the channel.

- No idle timeout for dedicated connections.

- If there are two or more remote nodes set as dedicated connections, MP connections may exist. If MP occupies all channels, the next few remote nodes with dedicated connection won‘t be dialed. That is, no dial up guarantee for MP or dedicated connection.

- Even if there is only one remote node with dedicated connection requirement and it is also set to have an MP connection, the prestige doesn’t guarantee that the second link will be up. Because it is possible that other remote node connections will be built up earlier, no bandwidth will be available for the second link of the dedicated connection.

## WAN IP Address and CLID Shown on Menu 24.1

Menu 24.1 has been separated into to two screens. The original static information in Menu 24.1 has been moved to Menu 24.2.1.

Menu 24.1 only displays the dynamic information now. For P100MH/WH, The CLID of the incoming call will be shown on this menu too. So is WAN IP address. For P153x/P153/ P128M, we only shows WAN IP address.

Menu 24.2.1 :

Menu 24.2.1 - System Maintenance - Information

Name: P100MH

RAS S/W Version: V2.21b02 | 11/16/98

Country Code: 238

LAN

Ethernet Address: 00:a0:c5:08:09:02

IP Address: 202.132.50.9

IP Mask: 255.255.255.192

DHCP: None

Press ESC or RETURN to Exit:

Menu 24.1 (P100MH/P100WH):

Menu 24.1 - System Maintenance - Status

Port Status      Speed     TXPkts     RXPkts  Errs  Tx B/s  Rx B/s     Up Time

1     Idle       0Kbps        47         28        0        0       0       0:00:00

Total Outcall Time:     0:00:00

Ethernet:                       WAN:

Status: 10M/Half Duplex 	  Port 1 IP Addr:

TX Pkts: 1910             	  Port 1 CLID:

RX Pkts: 64373

Collisions: 0

LAN Packet Which Triggered Last Call: (Type: IP)

45 00 00 38 7D 14 00 00 3F 11 C8 0B CA E7 BB 61 9D 16 01 06 00 35 00 35

00 24 39 90 79 65 00 00 00 01 00 00 00 00 00 00 06 64 6F 6D 61 69 6E 03

Press Command:

CMDS: 1-Drop MODEM1 8- Drop ALL 3-Rst Cnt  ESC-Exit

Menu 24.1 (P153/P153x/P128MH):

Menu 24.1 - System Maintenance - Status

Port Status      Speed     TXPkts     RXPkts  Errs  Tx B/s  Rx B/s     Up Time

1     Idle       0Kbps        47         28        0        0       0       0:00:00

2     Idle       0Kbps        10          8        0        0       0       0:00:00

3     Idle       0Kbps        24          8        0        0       0       0:00:00

Total Outcall Time:     0:00:00

Ethernet:                       WAN:

Status: 10M/Half Duplex 	  Port 1 IP Addr:

TX Pkts: 1910             	  Port 2 IP Addr:

RX Pkts: 64373            	  Port 3 IP Addr:

Collisions: 0

LAN Packet Which Triggered Last Call: (Type: IP)

45 00 00 38 7D 14 00 00 3F 11 C8 0B CA E7 BB 61 9D 16 01 06 00 35 00 35

00 24 39 90 79 65 00 00 00 01 00 00 00 00 00 00 06 64 6F 6D 61 69 6E 03

Press Command:

CMDS: 1-Drop Port1 2-Drop Port2 3-Drop Port3 8-Drop ALL 3-Rst Cnt  ESC-Exit

Backup and Restore Configuration File via LAN or WAN

1.  How to use TFTP to Transfer data to/from The Prestige.

1) Use the TELNET client program in your PC to login to your Prestige, and use menu 24.8 to enter C/I command mode:

Because TFTP doesn‘t have any security check, so we record the IP address that can telnet into the Prestige. Only TFTP requests from this IP address can be accepted.

2) Type C/I Command “sys stdio 0” to disable console idle timeout.

This command is to prevent console idle timeout causing TFTP data transfer failure.

3) The User can open TFTP client, set it to octet mode (binary mode) and transfer data from/to the Prestige.

2.  To backup your configuration, use TFTP client program to get file “rom-0” from the

Prestige.

3.  To restore your configuration, use TFTP client program to put your configuration in

file “rom-0” in the Prestige.

Upgrade Firmware via LAN

- To upgrade firmware, use the TFTP client program to put firmware in file “ras” in the Prestige. After data transfer is finished, the Prestige will program the upgraded Firmware into FLASH ROM and reboot your Prestige.

- To backup your firmware, use the TFTP client program to get file “ras” from the Prestige.

Upgrade Internal Firmware via Console. (P100MH/P128MH only)

- Use your terminal program to connect to the Prestige.

- In the SMT Main Menu, please enter menu 24.7 and select the modem you want to upgrade.

- Use XMODEM protocol to send MODEM F/W to the Prestige.

- After XMODEM Transfer is complete, The Prestige will program the upgraded Firmware into MODEM Module‘s Flash Memory.

- The Prestige will automatically reboot.

Upgrade Internal MODEM Firmware via LAN. (P100MH/P128MH only)

- To upgrade modem firmware in the P100MH, use the TFTP client program to put firmware to file “modem1” in the Prestige. After data transfer is finished, The Prestige will program the upgraded firmware into FLASH ROM and reboot your Prestige.

- To upgrade modem firmware in the P128MH, use the TFTP client program to put firmware to file “modem1” in the Prestige to upgrade modem 1 and put firmware to file “modem2” in The Prestige to upgrade modem 2. After data transfer is finished, the Prestige will program the upgraded firmware into FLASH ROM and reboot your Prestige.

- Backup internal modem F/W from the Prestige is not supported.

## Factory Default Values (P100MH/P100WH Only)

The factory default values have been optimized for Internet connection. There are two sets of filter rules that have been configured in Menu 21 to prevent NETBIOS triggering the call. The default filters are shown below:

Menu 21:

Menu 21 - Filter Set Configuration

Filter                                   Filter

Set #        Comments          		Set #        Comments

------  -----------------           	------  -----------------

1      NetBIOS_WAN                  	 7       _______________

2      NetBIOS_LAN                   	 8       _______________

3      _______________             	 9       _______________

4      _______________             	10       _______________

5      _______________             	11       _______________

6      _______________             	12       _______________

Menu 21.1 (NetBIOS WAN):

Menu 21.1 - Filter Rules Summary

# A Type                 Filter Rules 			  M m n

- - ---- ------------------------------------------	  - - -

1 Y IP   Pr=6, SA=0.0.0.0, DA=0.0.0.0, DP=137		  N D N

2 Y IP   Pr=6, SA=0.0.0.0, DA=0.0.0.0, DP=138	    	  N D N

3 Y IP   Pr=6, SA=0.0.0.0, DA=0.0.0.0, DP=139	   	  N D N

4 Y IP   Pr=17, SA=0.0.0.0, DA=0.0.0.0, DP=137     	  N D N

5 Y IP   Pr=17, SA=0.0.0.0, DA=0.0.0.0, DP=138	  N D N

6 Y IP   Pr=17, SA=0.0.0.0, DA=0.0.0.0, DP=139	  N D F

Menu 21.2 (NetBIOS LAN):

Menu 21.2 - Filter Rules Summary

# A Type              Filter Rules                    	  	M m n

- - ---- -------------------------------------------  	- - -

1 Y IP   Pr=17, SA=0.0.0.0, SP=137, DA=0.0.0.0, DP=53 	N D F

2 N

3 N

4 N

5 N

6 N

The filter set, NetBIOS LAN, is inserted in the protocol filters field under Input Filter Sets in Menu 3.1 in order to prevent local NetBIOS messages triggering calls to the DNS Server.

Menu 3.1:

Menu 3.1 - General Ethernet Setup

Input Filter Sets:

protocol filters= 2

device filters=

Output Filter Sets:

protocol filters=

device filters=

The filter set, NetBIOS WAN, is inserted in the protocol filters field under Call Filter Sets in Menu 11.5  in order to prevent local NetBIOS messages triggering calls to the Internet.

Menu 11.5 of ISP remote node:

Menu 11.5 - Remote Node Filter

Input Filter Sets:

protocol filters=

device filters=

Output Filter Sets:

protocol filters=

device filters=

Call Filter Sets:

protocol filters= 1

device filters=

By default, the Prestige is configured as a DHCP server, and the following features are enabled: DNS Proxy, SUA, dynamic IP address.

Menu 3.2:

Menu 3.2 - TCP/IP and DHCP Ethernet Setup

DHCP Setup:

DHCP= Server

Client IP Pool Starting Address= 192.168.1.33

Size of Client IP Pool= 32

Primary DNS Server= 0.0.0.0

Secondary DNS Server= 0.0.0.0

TCP/IP Setup:

IP Address= 192.168.1.1

IP Subnet Mask= 255.255.255.0

RIP Direction= Both

Version= RIP-1

Menu 4:

Menu 4 - Internet Access Setup

ISP's Name= ChangeMe

Pri Phone #= 1234

Sec Phone #=

My Login= ChangeMe

My Password= ********

Single User Account= Yes

IP Addr= 0.0.0.0

Edit Script Options= No

Nailed-Up Connection= No

Idle Timeout= 300

## Frequently used C/I Commands

Here is a brief description of the most frequently used CI commands. Their order in the following table is based on the v2.2x commands’ alphabetic sequence.

| C/I Commands | brief description |
| --- | --- |
| bridge stat disp | statistics on Bridge packets |
| bridge blt disp | Bridge LAN table |
| bridge brt disp | Bridge WAN table |
| dev channel disp [wan00 ¦ wan01 ¦ wan02] | shows channel information on wan00, wan01 or wan02 |
| dev channel drop [wan00 ¦ wan01 ¦ wan02] | drops channel on wan00, wan01 or wan02 |
| dev dial x | manually dials to remote node x; x is the remote node number here |
| ether config | shows the current Ethernet configuration |
| ether driver cnt disp | statistics on the Ethernet driver |
| exit | exits from CI mode |
| ip address | LAN IP address |
| ip ping {IP address} | Ping {IP address} |
| ip route stat | IP routing table |
| ip status | statistics on IP packets |
| ip sua iface [wanif0 ¦ wanif1 ¦ wanif2] disp | displays the THE SUA table for iface wanif0, wanif1 or wanif2 |
| ipx route stat | IPX routing table |
| ipx sap stat | IPX SAP table |
| ppp lcp acfc [on ¦ off] | enables/disables PPP LCP ACFC negotiation |
| ppp lcp bacp [on ¦ off] | enables/disables PPP LCP BACP negotiation |
| ppp lcp callback [on ¦ off] | enables/disables PPP LCP MS callback negotiation |
| ppp lcp pfc [on ¦ off] | enables/disables PPP LCP PFC negotiation |
| sys trcl call | shows call trace on the screen |
| sys log disp | displays the error/warning/information messages in the system log |
| sys log clear | clears the existing contents in system log |
| sys mbuf pool | displays the pool of mbuf; mbuf is the buffer pre-allocated for data transmission |
| sys mbuf status | displays mbuf status |
| sys memutil mqueue | statistics on pre-allocated system memory cell |
| sys memutil usage | statistics on the memory utilization |
| sys stdio 0 | sets SMT session timeout value to 0 –> never timeout |
| sys trcd | displays the packet trace on screen |
| sys trcl clear | clear the existing contents in logic trace log |
| sys trcl disp | displays the contents in both of logic and packet trace logs |
| sys trcl switch [on¦off] | enables/disables logic trace log mechanism |
| sys trcp disp | displays the contents in packet trace log |
| sys trcp switch [on¦off] | enables/disables packet trace log mechanism |
| wan atring <dev name> | show AT command response buffer. |
| wan cnt clear <dev name> | clear wan status counters. |
| wan cnt disp <dev name> | show wan status counters. |
| wan drop <dev name> | drop connection. |
| wan mstatus <dev name> | display internal Modem Module‘s Last Call Status. (P128MH only) |
| wan mtype <dev name> | display Internal Modem Module Type and F/W version. (P128MH only) |
| wan rate <dev name> | display traffic information of last 1 minute. |
| wan redirect <dev name> | enter redirect mode. |
| wan signal <dev name> | display RS-232 signal. |

| dev name\product | P128MH | P153/P153x | P100MH | P100WH |
| --- | --- | --- | --- | --- |
| wan00 | Port 1 | Port 1 | - | - |
| wan01 | Modem 1 | Port 2 | - | - |
| wan02 | Modem 2 | Port 3 | Modem | Port 1 |

## Internal Modem Module AT Commands Summary (P100MH, P128MH)

TI Modem Module. (P128MH, P100MH-T)

| Command Quick Reference &$ HELP, Ampersand Commands A/ Repeat Last Command A> Continuously Repeat Command AT Command Mode Prefix A Answer Call Bn n=0 V32 Mode/CCITT Answer Seq n=1 Bell Answer Seq Dn Dial a Telephone Number n=0..9#*TPR,;W@!()- DL Dial Last Phone Number DSn Dial Stored Phone Number D$ HELP, Dial Commands En n=0 No Command Echo n=1 Echo Command Chars Fn n=0 Online Echo n=1 No Online Echo Hn n=0 ON Hook (Hang Up) n=1 OFF Hook In n=0 Product Code n=1 Checksum n=2 RAM Test n=3 Product ID string n=4 Current Settings n=5 NVRAM Settings n=6 Link Diagnostics n=7 Product Configuration n=9 PnP Enumeration Screen n=11 Link Diagnostics Ln n=0 Low Speaker Volume n=1 Low Speaker Volume n=2 Med Speaker Volume n=3 Hi Speaker Volume | Mn n=0 Speaker OFF n=1 Speaker ON Until CD n=2 Speaker Always ON n=3 Speaker OFF During Dial On n=0 Return Online n=1 Return Online & Retrain P Pulse Dial Qn n=0 Result Codes Sent n=1 Quiet (No Result Codes) n=2 Verbose/Quiet ON Answer Sr=n Sets Register "r" to "n" Sr? Query Register "r" S$ HELP, S Registers T Tone Dial Vn n=0 Numeric Responses n=1 Verbal Responses Xn n=0 Basic Result Codes n=1 Extended Result Codes n=2-4 Advanced Result Codes Yn n=0 Next Reset to &W0 settings n=1 Next Reset to &W1 settings n=2 Next Reset to &F0 settings n=3 Next Reset to &F1 settings n=4 Next Reset to &F2 settings Zn n=0 Y setting determines reset n=1 Reset to &W0 settings n=2 Reset to &W1 settings n=3 Reset to &F0 settings n=4 Reset to &F1 settings n=5 Reset to &F2 settings +++ Escape Code $ HELP, Command Summary |
| --- | --- |
| Ampersand Commands &An n=0 Disable /ARQ Result Codes n=1 Enable /ARQ Result Codes n=2 Enable /Modulation Codes n=3 Enable /Extra Result Codes &Bn n=0 Floating DTE Speed n=1 Fixed DTE Speed n=2 DTE Speed Fixed When ARQ &Cn n=0 CD Always ON n=1 Modem Controls CD &Dn n=0 Ignore DTR n=1 On-Line Command Mode n=2 DTE Controls DTR n=3 Reset ON DTR Loss &Fn n=0 Load Factory 0, no FC n=1 Load Factory 1, HW FC n=2 Load Factory 2, SW FC &Gn n=0 No Guard Tone n=1 550 Hz Guard Tone n=2 1800 Hz Guard Tone &Hn n=0 Disable TX Flow Control n=1 CTS n=2 Xon/Xoff n=3 CTS and Xon/Xoff &In n=0 Disable RX Flow Control n=1 Xon/Xoff n=2 Xon/Xoff Chars Filtered n=3 HP Enq/Ack Host Mode n=4 HP Enq/Ack Terminal Mode n=5 Xon/Xoff for non-ARQ Mode &Kn n=0 Disable Data Compression n=1 Auto Data Compression n=2 Enable Data Compression n=3 Selective Data Compression &Mn n=0 Normal Mode n=4 ARQ/Normal Mode n=5 ARQ Mode &Nn n=0 Highest Link Speed n=1 300 bps n=2 1200 bps n=3 2400 bps n=4 4800 bps n=5 7200 bps n=6 9600 bps n=7 12000 bps n=8 14400 bps n=9 16800 bps n=10 19200 bps n=11 21600 bps n=12 24000 bps n=13 26400 bps n=14 28800 bps n=15 31200 bps n=16 33600 bps n=17 28000 bps n=18 29333 bps n=19 30666 bps n=20 32000 bps n=21 33333 bps n=22 34666 bps n=23 36000 bps n=24 37333 bps n=25 38666 bps n=26 40000 bps n=27 41333 bps n=28 42666 bps n=29 44000 bps n=30 45333 bps n=31 46666 bps n=32 48000 bps n=33 49333 bps n=34 50666 bps n=35 52000 bps n=36 53333 bps n=37 54666 bps n=38 56000 bps n=39 57333 bps | &Pn n=0 N.American Pulse Dial n=1 UK Pulse Dial &Rn n=1 Ignore RTS n=2 RX to DTE/RTS high &Sn n=0 DSR Always ON n=1 Modem Controls DSR &Tn n=0 End Test n=1 Analog Loopback (ALB) n=3 Digital Loopback (DLB) n=4 Grant Remote DLB n=5 Deny Remote DLB n=6 Remote Digital Loopback n=7 Remote DLB With Self Test n=8 ALB With Self Test &Un n=0 Variable link rate floor. n=1 Minimum link rate 300 bps n=2 Minimum link rate 1200 bps n=3 Minimum link rate 2400 bps n=4 Minimum link rate 4800 bps n=5 Minimum link rate 7200 bps n=6 Minimum link rate 9600 bps n=7 Minimum link rate 12000 bps n=8 Minimum link rate 14400 bps n=9 Minimum link rate 16800 bps n=10 Minimum link rate 19200 bps n=11 Minimum link rate 21600 bps n=12 Minimum link rate 24000 bps n=13 Minimum link rate 26400 bps n=14 Minimum link rate 28800 bps n=15 Minimum link rate 31200 bps n=16 Minimum link rate 33600 bps n=17 Minimum link rate 28000 bps n=18 Minimum link rate 29333 bps n=19 Minimum link rate 30666 bps n=20 Minimum link rate 32000 bps n=21 Minimum link rate 33333 bps n=22 Minimum link rate 34666 bps n=23 Minimum link rate 36000 bps n=24 Minimum link rate 37333 bps n=25 Minimum link rate 38666 bps n=26 Minimum link rate 40000 bps n=27 Minimum link rate 41333 bps n=28 Minimum link rate 42666 bps n=29 Minimum link rate 44000 bps n=30 Minimum link rate 45333 bps n=31 Minimum link rate 46666 bps n=32 Minimum link rate 48000 bps n=33 Minimum link rate 49333 bps n=34 Minimum link rate 50666 bps n=35 Minimum link rate 52000 bps n=36 Minimum link rate 53333 bps n=37 Minimum link rate 54666 bps n=38 Minimum link rate 56000 bps n=39 Minimum link rate 57333 bps &Wn n=0 Store Configuration 0 n=1 Store Configuration 1 &Yn n=0 Destructive n=1 Destructive/Expedited n=2 Nondest./Expedited n=3 Nondest./Unexpedited &Zn=s Store Phone Number &Zn? Query Phone Number #CID=n n=0 Caller ID Off n=1 Formatted Caller ID n=2 Unformatted Caller ID |
| S Register Functions S0 Ring to Answer ON S1 Counts # of Rings S2 Escape Code Char S3 Carriage Return Char S4 Line Feed Char S5 Backspace Char S6 Wait Time/Dial Tone (sec) S7 Wait Time/Carrier (sec) S8 Comma Time (sec) S9 Carrier Detect Time (1/10sec) S10 Carrier Loss Time (1/10sec) S11 Dial Tone Spacing (msec) S12 Escape Code Time (1/50sec) S13 Bit Mapped 1 = Reset ON DTR Loss 2 = Reduced Non-ARQ TX Buffer 4 = Set DEL=Backspace 8 = Do DS0 ON DTR 16 = Do DS0 ON Reset 32 = Reserved 64 = Disable Quick Retrains 128 = Escape Code Hang Up S14 Reserved S15 Bit Mapped 1 = MNP/V.42 Disabled in V.22 2 = MNP/V.42 Disabled in V.22bis 4 = MNP/V.42 Disabled in V.32 8 = Disable MNP Handshake 16 = Disable MNP Level 4 32 = Disable MNP Level 3 64 = Unusual MNP-Incompatibility 128 = Disable V.42 136 = Disable V.42 Detect Phase S16 Test Modes 1 = Reserved 2 = Dial Test 4 = Reserved 8 = Reserved 16 = Reserved 32 = Reserved 64 = Reserved 128 = Reserved S17 Reserved S18 &Tn Test Timeout (sec) S19 Inactivity Timeout (min) S20 Reserved S21 Break Length (1/100sec) S22 Xon Char S23 Xoff Char S24 Reserved S25 DTR Recognition Time (1/100sec) S26 Reserved S27 Bit Mapped 1 = V21 Mode 2 = Disable TCM 4 = Disable V32 8 = Disable 2100hz 16 = Enable V23 Fallback 32 = Disable V32bis 64 = Reserved 128 = Software Compatibility Mode | S28 V32 Handshake Time (1/10sec) S29 V.21 answer mode fallback timer S30 Reserved S31 Reserved S32 Connection bit mapped operations. 1 = V.8 Call Indicate enable 2 = Enable V.8 mode 4 = Reserved 8 = Disable V.34 modulation 16 = Disable V.34+ modulation 32 = Disable x2 modulation 64 = Disable V.90 modulation 128 = Reserved S33 V.34 & V.34+ Connection setup bit mapped control flags. 1 = Disable 2400 Symbol rate 2 = Disable 2743 Symbol rate 4 = Disable 2800 Symbol rate 8 = Disable 3000 Symbol rate 16 = Disable 3200 Symbol rate 32 = Disable 3429 Symbol rate 64 = Reserved 128 = Disable Shaping S34 V.34 & V.34+ Connection setup bit mapped control flags. 1 = Disable 8S-2D trellis encoding 2 = Disable 16S-4D trellis encoding 4 = Disable 32S-2D trellis encoding 8 = Disable 64S-4D trellis encoding 16 = Disable Non linear coding 32 = Disable TX level deviation 64 = Disable Pre-emphasis 128 = Disable Pre-coding S35 Reserved S36 Reserved S37 Reserved S38 Disconnect Wait Time (sec) S39 Reserved S40 Reserved S41 Distinctive Ring options 1 = Distinctive Ring Enabled 2 = Reserved 4 = Reserved 8 = Reserved 16 = Reserved 32 = Reserved 64 = Reserved 128 = Reserved S42 Reserved |
| Dial Commands 0-9 Digits to Dial * Auxiliary Tone Dial Digit # Auxiliary Tone Dial Digit T Tone Dialing P Pulse Dialing R Call an Originate Only Modem | , Pause (Wait for S8 Time) / Short Delay ; Remain in Command Mode After Dialing W Wait for 2nd Dial Tone (X2,X4) @ Wait for an Answer (X3-X4) ! Flash Switch Hook |

ROCKWELL Modem Module. (P100MH-R)

BASIC AT COMMANDS A/ Re-execute command. A Go off-hook and attempt to answer a call. B0 Select V.22 connection at 1200 bps. B1 Select Bell 212A connection at 1200 bps. C1 Return OK message. Dn Dial modifier. E0 Turn off command echo. E1 Turn on command echo. H0 Initiate a hang-up sequence. H1 If on-hook, go off-hook and enter command mode. I0 Report product code. I1 Report pre-computed checksum. I2 Report OK. I3 Report firmware revision, model, and interface type. I4 Report response programmed by an OEM. I5 Report the country code parameter. I6 Report modem data pump model and code revision. I7 Reports the DAA code (W-class models only). L0 Set low speaker volume. L1 Set low speaker volume. L2 Set medium speaker volume. L3 Set high speaker volume. M0 Turn speaker off. M1 Turn speaker on during handshaking and turn speaker off while receiving carrier. M2 Turn speaker on during handshaking and while receiving carrier. M3 Turn speaker off during dialing and receiving carrier and turn speaker on during answering. N0 Turn off automode detection. N1 Turn on automode detection. O0 Go on-line. O1 Go on-line and initiate a retrain sequence. P Force pulse dialing. Q0 Allow result codes to DTE. Q1 Inhibit result codes to DTE. Sn Select S-Register as default. Sn? Return the value of S-Register n. =v Set default S-Register to value v. ? Return the value of default S-Register. T Force DTMF dialing. V0 Report short form (terse) result codes. V1 Report long form (verbose) result codes. W0 Report DTE speed in EC mode. W1 Report line speed, EC protocol and DTE speed. W2 Report DCE speed in EC mode. X0 Report basic call progress result codes, i.e., OK, CONNECT, RING, NO CARRIER (also, for busy, if enabled, and dial tone not detected), NO ANSWER and ERROR. X1 Report basic call progress result codes and connections speeds (OK, CONNECT, RING, NO CARRIER (also, for busy, if enabled, and dial tone not detected), NO ANSWER, CONNECT XXXX, and ERROR. X2 Report basic call progress result codes and connections speeds, i.e., OK, CONNECT, RING, NO CARRIER (also, for busy, if enabled, and dial tone not detected), NO ANSWER, CONNECT XXXX, and ERROR. X3 Report basic call progress result codes and connection rate, i.e., OK, CONNECT, RING, NO CARRIER, NO ANSWER, CONNECT XXXX, BUSY, and ERROR. X4 Report all call progress result codes and connection rate, i.e., OK, CONNECT, RING, NO CARRIER, NO ANSWER, CONNECT XXXX, BUSY, NO DIAL TONE and ERROR. Y0 Disable long space disconnect before on-hook. Y1 Enable long space disconnect before on-hook. Z0 Restore stored profile 0 after warm reset. Z1 Restore stored profile 1 after warm reset. &C0 Force RLSD active regardless of the carrier state. &C1 Allow RLSD to follow the carrier state. &D0 Interpret DTR ON-to-OFF transition per &Qn: &Q0, &Q5, &Q6 The modem ignores DTR. &Q1, &Q4 The modem hangs up. &Q2, &Q3 The modem hangs up. &D1 Interpret DTR ON-to-OFF transition per &Qn: &Q0, &Q1, &Q4, &Q5, &Q6 Asynchronous escape. &Q2, &Q3 The modem hangs up. &D2 Interpret DTR ON-to-OFF transition per &Qn: &Q0 through &Q6 The modem hangs up. &D3 Interpret DTR ON-to-OFF transition per &Qn:. &Q0, &Q1, &Q4, &Q5, &Q6 The modem performs soft reset. &Q2, &Q3 The modem hangs up. &F0 Restore factory configuration 0. &F1 Restore factory configuration 1. &G0 Disable guard tone. &G1 Disable guard tone. &G2 Enable 1800 Hz guard tone. &J0 Set S-Register response only for compatibility. &J1 Set S-Register response only for compatibility. &K0 Disable DTE/DCE flow control. &K3 Enable RTS/CTS DTE/DCE flow control. &K4 Enable XON/XOFF DTE/DCE flow control. &K5 Enable transparent XON/XOFF flow control. &K6 Enable both RTS/CTS and XON/XOFF flow control. &L0 Select dial up line operation. &M0 Select direct asynchronous mode. &M1 Select sync connect with async off-line command mode. &M2 Select sync connect with async off-line command mode and enable DTR dialing of directory zero. &M3 Select sync connect with async off-line command mode and enable DTR to act as Talk/Data switch. &P0 Set 10 pps pulse dial with 39%/61% make/break. &P1 Set 10 pps pulse dial with 33%/67% make/break. &P2 Set 20 pps pulse dial with 39%/61% make/break. &P3 Set 20 pps pulse dial with 33%/67% make/break. &Q0 Select direct asynchronous mode. &Q1 Select sync connect with async off-line command mode. &Q2 Select sync connect with async off-line command mode and enable DTR dialing of directory zero. &Q3 Select sync connect with async off-line command mode and enable DTR to act as Talk/Data switch. &Q4 Select Hayes AutoSync mode. &Q5 Modem negotiates an error corrected link. &Q6 Select asynchronous operation in normal mode. &R0 CTS tracks RTS (async) or acts per V.25 (sync). &R1 CTS is always active. &S0 DSR is always active. &S1 DSR acts per V.25. &T0 Terminate any test in progress. &T1 Initiate local analog loopback. &T2 Returns ERROR result code. &T3 Initiate local digital loopback. &T4 Allow remote digital loopback. &T5 Disallow remote digital loopback request. &T6 Request an RDL without self-test. &T7 Request an RDL with self-test. &T8 Initiate local analog loop with self-test. &V Display current configurations. &V1 Display connection statistics &W0 Store the active profile in NVRAM profile 0. &W1 Store the active profile in NVRAM profile 1. &X0 Select internal timing for the transmit clock. &X1 Select external timing for the transmit clock. &X2 Select slave receive timing for the transmit clock. &Y0 Recall stored profile 0 upon power up. &Y1 Recall stored profile 1 upon power up. &Zn=x Store dial string x (to 34) to location n (0 to 3). %E0 Disable line quality monitor and auto retrain. %E1 Enable line quality monitor and auto retrain. %E2 Enable line quality monitor and fallback/fall forward. %L Return received line signal level. %Q Report the line signal quality. %7 Plug and Play serial number %8 Plug and Play vendor ID and product number \Kn Controls break handling during three states: When modem receives a break from the DTE: \K0,2,4 Enter on-line command mode, no break sent to the remote modem. \K1 Clear buffers and send break to remote modem. \K3 Send break to remote modem immediately. \K5 Send break to remote modem in sequence with transmitted data. When modem receives \B in on-line command state: \K0,1 Clear buffers and send break to remote modem. \K2,3 Send break to remote modem immediately. \K4,5 Send break to remote modem in sequence with transmitted data. When modem receives break from the remote modem: \K0,1 Clear data buffers and send break to DTE. \K2,3 Send a break immediately to DTE. \K4,5 Send a break with received data to the DTE. \N0 Select normal speed buffered mode. \N1 Select direct mode. \N2 Select reliable link mode. \N3 Select auto reliable mode. \N4 Force LAPM mode. \N5 Force MNP mode. \V0 Connect messages are controlled by the command settings X, W, and S95. \V1 Connect messages are displayed in the single line format. +MS Select modulation. +H0 Disable RPI/Video Ready Mode. +H1 Enable RPI and set DTE speed to 19200 bps. +H2 Enable RPI and set DTE speed to 38400 bps. +H3 Enable RPI and set DTE speed to 57600 bps. +H11 Enable RPI+ mode. +H16 Enable Video Ready Mode **0 Download to flash memory at last sensed speed. **1 Download to flash memory at 38.4 kbps. **2 Download to flash memory at 57.6 kbps. -SDR=0 Disable Distinctive Ring. -SDR=1 Enable Distinctive Ring Type 1. -SDR=2 Enable Distinctive Ring Type 2. -SDR=3 Enable Distinctive Ring Type 1 and 2. -SDR=4 Enable Distinctive Ring Type 3. -SDR=5 Enable Distinctive Ring Type 1 and 3. -SDR=6 Enable Distinctive Ring Type 2 and 3. -SDR=7 Enable Distinctive Ring Type 1, 2, and 3. ECC COMMANDS %C0 Disable data compression. %C1 Enable MNP 5 data compression. %C2 Enable V.42 bis data compression. %C3 Enable both V.42 bis and MNP 5 compression. \A0 Set maximum block size in MNP to 64. \A1 Set maximum block size in MNP to 128. \A2 Set maximum block size in MNP to 192. \A3 Set maximum block size in MNP to 256. \Bn Send break of n x 100 ms. MNP 10 COMMANDS -K0 Disable MNP 10 extended services. -K1 Enable MNP 10 extended services. -K2 Enable MNP 10 extended services detection only. -SEC=0 Disable MNP10-EC. -SEC=1,[<tx level>] Enable MNP10-EC and set transmit level <tx level> 0 to 30 (0 dBm to -30 dBm). W-CLASS COMMANDS *B Display list of permanently blacklisted numbers. *D Display list of delayed numbers. *NCn Change country to one of eight in NVRAM. CALLER ID COMMANDS #CID=0 Disable Caller ID. #CID=1 Enable Caller ID with formatted presentation. #CID=2 Enable Caller ID with unformatted presentation.

## Error Code Summary in Syslog

-3000 === remote node is connecting

-3001 === configured incoming call only, outgoing call fails

-3002 === configured outgoing call only, incoming call fails

-3003 === packet is filtered

-3004 === no iface

-3005 === no channel available

-3006 === call request fail

-3007 === remote node is waiting call back

-3020 === call dial fail

-3022 === filter groups are mixed, so call is not allowed

-3023 === received unexpected event

-3024 === state timeout

-3025 === waiting RADIUS authentication

-3026 === RADIUS call back fail

-3028 === the node is not found

-3029 === the node is inactive

-3030 === dial fail

-3031 === no budget

-3032 === radius authentication fail

-3033 === CLID is required

-3034 === CLID can not be found

-3035 === an outgoing call has already been placed for this remote node

-3036 === call is blocked

-3037 === invalid phone number

-3038 === remote side is busy

-3039 === no carrier

-3040 === no dial tone

-3041 === remote node is not active

-3042 === no answer received

-3043 === dial timeout

-3044 === redial no method

-3045 === redial stopped

-3046 === redial no number

-3047 === first call is CLID authenticated and remote node is obtained but ppp

is not up yet, second call has same CLID

-3048 === remote node does not supported L2TP

## Bug fixes

- Add DTR Drop detection in Menu 24.1. (When you unplug or close terminal program when you are still in Menu 24.1, the prestige will automatically logout -- This feature needs H/W support. Some old beta sample can‘t support this feature)

- Fix bug: P153/P153x/P128MH bridge (1 LAN + 2 WAN bridge together) won‘t work.

- The telephone number recorded in trace log is changed from first 20 digits to last 20 digits.

- Fix bug: Sometimes change the contents of Menu 11.1 in Sync-Lease mode will make Prestige reboot (P153x only)

- Fix bug: In Menu 12.3.1 (Bridge Static Route), Gateway Node can only set 1 to 4. (Should be 1 to 12). (For P153/P153x/P128MH only)

- Fix bug: when "greater" or "less" is set in “port comp” field in menu 21.x.y, SMT21.x will not show ">" or "<" sign in port display. (It shows DP1023 instead of DP>1023 or DP<1023).

- Fix bug: PING attack by large packets (size about 65020) may cause Prestige reboot.

- Fix bug: If connection type of menu 11.1 is Leased-Sync, user can‘t change any setting in this menu except user delete all settings and reconfig them again. (P153x only)

- Fix bug: STAC compression is incompatible with Microsoft Windows 98.

- Fix bug: PAP doesn‘t retry after timeout if no response.

- Fix bug: If we transfer a file with lots of 0x7E, the performance of the Prestige will drop a lot. (Sometimes data transfer will abort).

- Fix bug: Sometimes after change SMT menu 4 or 11 when remote node is connecting, this remote node will be mark as “connecting” and can‘t dial again. (When you use manual dial, you can see “remote node is connecting” error message).

- Remove unused Idle Timeout Setting in Menu 13.

- Fix bug: Press 2, 8 will drop call in SMT 24.1 (P100MH and P100WH).

- “IP ROUTE FLUSH” C/I command used to remove all of the entries in IP routing table except the local one (LAN). It has been changed to remove the dynamic entries only. It means that the Prestige will not remove the static entries defined in Menu 11 and 12.

- Fix bug: when available channels is dial-in only, BAP/BACP will not reply next available telephone number. (P153, P128MH andP153x)

- Multilink PPP “Add persist time” default value is changed from 30 secs to 15 secs. (P128MH, P153 and P153x)

- Fix bug: CHAP used to fail if the length of password exceeds 15 characters. (Now, the Max. Length of Username and Password is 31).

- Now the Prestige is capable to support rlogin and rcp even though it still has difficulty to support rsh.

- Fix bug: CHAP failed if the length of password exceeds 15 characters.

- In very large IPX networks, you may have problems accessing the desired servers.

- The TCP/IP filters configured in Menu 21 may not work properly if it is a SUA connection.  It is because the Prestige executes the call/output filter after the SUA converts the source IP address and port number of the packets from LAN, and it executes the input filter before the SUA converts the destination IP address and port number to a local address and port number.

- STAC Compression may be disabled if connected for a long time.

- Fix bug: Script is incompatible with SCO Unix. (Add 1 second delay between the expected string appears and then send out).

- Fix bug: Script is incompatible to login CompuServ.

- Fix bug: the 5th script and 6th script expect-send setting can‘t be executed.

## How To Upgrade The Prestige (from 2.1x to 2.2x)

1.  Here are the files needed for upgrading your Prestige:

| Product | Router Firmware | Default Configuration File |
| --- | --- | --- |
| P100MH | P100MH.BIN | P100MH-R.ROM (Rockwell) P100MH-T.ROM (TI) |
| P100WH | P100MH.BIN | P100WH.ROM |
| P128MH | P153.BIN | P153.ROM |
| P153 | P153.BIN | P153.ROM |
| P153x | P153.BIN | P153.ROM |

2.  If you don‘t want to keep your configuration, please goto step 5.

3.  Because the filter design is changed (Generaic Filter Rules and Protocol Filter Rules can‘t be put in the same filter rule set), please adjust your design to meet our requirements. (If you don’t change the default filter setting in the P100MH/P100WH, you can skip this step).

4.  After the Firmware is upgraded, the filter setting in menu 3.1, Menu 11 of each remote node and menu 13 will be lost. So, please write it down before upgrading.

5.  Upgrade F/W by XMODEM via console.

(1) In SMT Main Menu, Enter menu 24.2.2 to change baudrate of Console.

(2) In SMT Main Menu, Enter menu 24.7.1 and following the instructions to upgrade Firmware.

Commands:

ATUR:  Upload Firmware file via XMODEM

6.  If you want to keep your settings, Please reconfigure filter set in Menu 3.1 (General Ethernet Setup), Menu 11 for each remote node and Menu 13. (If you don‘t change the default filter setting in P100MH/P100WH, please reconfig filter sets to be the same as section “Factory default Values”. )

7.  If you don‘t want to keep your settings, enter SMT menu 24.7.2 to upload the Default Configuration File.

Commands:

ATUR3:  Upload the Default configuration File.

Note1 :

You don't need to upload this file if you are upgrading from release 2.1x unless you want to reset all configurations to the factory default.

Note2 :

To distinguish the internal modem of the P100MH is TI-based or Rockwell Based, Please check the the FCC label on the BOTTOM side of your P100MH. If the model name on it is P100MH-R, then your modem module is Rockwell-Based. If the model name on it is P100MH-T, then your modem module is TI-Based.

8.  Reconfig Filter Setting.

9.  Check the setting of your SNMP network management program (HP Openview, ...) and

modify it. (Please check the section “SNMP”.)  (P128MH, P153 and P153x)