===========================================================================

Parallel.Exe                                  Parallel Technologies, Inc.

Parallel Port Information Utility                       10603 170th Ct NE

Redmond, WA 98052

(C) Copyright 1994 Jay Lowe                                  206-869-1136

This program is Freeware                                fax  206-869-1133

subject to a Restricted License stated below            CIS    76640, 203

===========================================================================

IMPORTANT:  This software is provided AS-IS under a restricted

license agreement stated below. YOU ARE RESPONSIBLE for the safe

usage of this program. This requires that you READ THE DIRECTIONS

if you are going to use the loopback feature for IRQ detection.

We are informing you of the danger of damaging your parallel port

if you are not CAREFUL AND GENTLE in inserting the loopback wire.

To the best of our (extensive) knowledge, there is no electrical

danger to yourself or to your system or to the parallel port, since

parallel ports are by design able to withstand permanent shorts

across any of the pins. HOWEVER WE DO NOT GUARANTEE ANY OF THIS

AND CAN ACCEPT NO LIABILITY since we cannot control what you do.

Welcome to the Parallel Port Information Utility

------------------------------------------------

Parallel is a simple utility which displays the configuration of the

parallel ports found in your system. This is handy for configuring

parallel port software which is too ignorant to configure itself,

like Windows print connections or parallel port device drivers, and

for solving print spooling problems. It can also be used to simply

familiarize yourself with how parallel ports are configured.

Of course, all the parallel port device drivers written by Parallel

Technologies are smart enough to be self-configuring, if the device

hardware is not totally braindead at birth. Laughest thou not belike,

as many parallel port peripherals are based on FPGA devices, like

the Xilinx, which need to be downloaded before they can function.

So many of these devices are simply too stupid to respond to any

initialization chit-chat until they have been initialized. Just the

sort of Catch-22 situation Parallel solves for you!

Parallel is also handy for solving problems associated with print

spoolers and network print capture programs which work by redirecting

printer output.

Operation

---------

Just type PARALLEL at the command line.  Parallel will display the

parallel port type, IO address, IRQ level, BIOS name equivalence,

and an assortment of informative notes about your parallel ports.

Parallel works best when you disconnect any parallel port devices,

such as printers, external peripherals, or "LapLink" cables. Even our

4bit, 8bit, or Univeral High-Speed Turbo cables for our parallel port

networking drivers (Pipx and Pndis) should be temporarily disconnected

when you run Parallel, at least until you understand how Parallel can

be affected by interference from devices attached to the port or ports

being tested. These effects are discussed in some detail below.

Here is a sample display of a system that contains an bidirectional

8bit port located at 3BC configured to use IRQ 7 and a unidirectional

4bit port located at 378 configured to use IRQ 5. As you can see,

each port is also provided with a loopback connector (a paperclip) as

discussed in the section on Loopback.

Interpreting the display is discussed in detail.

�����������������������������������������������������������������������������Ĵ

� Parallel Port Information Utility, v1.00 (930205), use PARALLEL ? for help. �

� (C) Copyright 1993 Jay Lowe, Parallel Technologies, Inc.   We thank you.    �

�����������������������������������������������������������������������������Ĵ

�  Port Type  � Address � IRQ � BIOS Name � Notes                             �

�����������������������������������������������������������������������������Ĵ

� 8 bit port  �   3BC   �  7  �   LPT1:   � IRQ determined by loopback        �

� 4 bit port  �   378   �  5  �   LPT2:   � IRQ determined by loopback        �

� Not present �   278   �  -  �     -     � No port at this address           �

�������������������������������������������������������������������������������

Interpreting the Display

------------------------

Port Type

---------

This can be 4bit (unidirectional), 8bit (bidirectional), or EPP (an

Intel 360SL style Enhanced Parallel Port). Later we will add

recognition for ECP style ports, when they show up in user's hands.

No Port         Parallel does not see appropriate parallel port

behavior at this port address.

This means the port not does not exist, ie, no

port is installed at the given base address.

However, there is one way in which Parallel can

be fooled into thinking the port does not exist.

If an active device is connected to the data lines

and is providing enough power to "overdrive" the

port, then the port existance test will fail.

By the way, in this case, the PC's BIOS will also

fail to detect the port. Again, Parallel works

best when you disconnect any parallel port devices.

4bit ports      Can do 8bit byte output and 4bit nibble input only.

4bit ports are capable of effective transfer rates

of about 40 KBPS in typical devices and can be

pushed upwards of 80 KBPS with certain design tricks.

The port speed is also affected by the electrical

characteristics of the noise suppression circuitry.

Some ports have ridiculous arrangements of noise

suppression circuits which slow them down badly.

8bit ports      Can do both 8bit input and output. 8bit ports are

capable of speeds ranging from 80-150 KBPS, again

depending on the speed of the attached device and

on the port's electrical characteristics.

EPP ports       Can do both 8bit input and output at bus speeds.

These ports are about as fast as 8bit ISA bus cards

and can achieve transfer rates as high as 1 MBPS.

For example, our EPP-capable external SCSI disk is

faster on a Compaq LTE25 than it's internal drive.

EPP ports are supposed to conform to a well defined

electrical standard.

EPP ports are most often found on laptops using the

Intel 386SL chipset with the 360SL IO chip. You can

usually identify these laptops by the 'SL' suffix

in their name, HOWEVER this is not a hard and fast

rule, since there are non-'SL' systems with EPP ports

as well as 'SL' systems which (in our tests) have

non-functioning or sub-standard EPP ports.

Other vendors offer add-in cards for the ISA bus.

Parallel will not currently recognize non-Intel EPP

ports; we will be adding this feature. We know of

two vendors which offer ISA EPP add-in cards:

FarPoint Communications               xxx-xxx-xxxx

SMC MicroSystems                      xxx-xxx-xxxx

The FarPoint card is quite nice because it is a

"soft" design with downloadable personalities. It

comes with support for fast Windows printing and

a fast NetWare spooling NLM. We like it.

We know less about the SMC SuperIO card with EPP.

It has the advantage of being a multifunction card,

which saves bus slots.

UTILITY:        Run Parallel on any system you intend to buy to

see what kind of parallel port(s) are provided.

If you are buying a desktop with a built-in LPT

card, try to get 8bit instead of 4bit. Some LPT

cards have hardware jumpers that are required to

enable the bidirectional behavior. So, make sure you

get a manual for the LPT card when you buy a system.

If you are buying a laptop and have any desire to

use external peripherals like network adapters,

portable hard disks, CD-ROMS, tape drives, etc,

seriously consider getting a laptop with an EPP port.

They are a LOT faster running external peripherals.

Microsoft, Hewlett-Packard, and others are

starting to work on a new standard for parallel

ports, the ECP port.  This may be standardized as

part of the IEEE 1284 specification. Some vendors

are reported to have silicon for this port, but

we haven't seen it in any systems yet. The ECP port

is pretty much the same as EPP ports in terms of

performance. The major difference is that ECP has

Microsoft's blessing and they will probably support

it if it actually materializes. Quite typically,

there is no guarantee that MS will ever support EPP.

IO Address

----------

This is the base address of the port. It will be 3BC, 378, or 278 hex.

The IO address specifies where the port is in the memory address space.

Normally the first port is configured to 3BC, the second to 378, etc,

however the ports can be assigned to any of these three addresses.

On bus cards, the IO address usually set by a arrangement of jumpers

unique to the paraticular board. Again, you usually need a manual or

the ability to read PC board traces (gacck!). These jumpers are usually

arranged to allow you to select one of three addresses: 3BC, 378, 278.

Sometimes these are called LPT1, LPT2, and LPT3 in the manual, although

that may be misleading (see BIOS discussion below). Some boards offer

a more limited selection of configurations (usually 378 and 278) and

a few are actually hardwired to a specific address. Vendors do this

to save a little money and also to elimitate the chance of creating an

address collision with the "standard" address of 3BC. Because of the

increasing number of clones which are assembled from various components,

we often see systems with one LPT card, configured to address 378. In

fact, a fair number of laptops are shipping with this 378 configuration.

A slightly interesting result we've seen in several desktops spins off

when someone adds a second LPT card to a system which already has a LPT

at 378, IRQ 7. Sometimes they naively reason that since 378 is taken,

they can use 3BC and slap in the card without checking the IRQ level.

This creates a configuration of 3BC and 378 both using IRQ 7, a situation

which can lead to intermittent crashes. By the way, a few motherboards

have parallel ports and com ports built onto the motherboard itself and,

in this case, the configuration jumpers will be on the motherboard.

Notice that there are three commonly used IO Address (3BC, 378, 278)

and two available interrupt levels (IRQ 7 and IRQ 5). This means that

if you have three LPT cards in a system, two of them will probably be

sharing an hardware interrupt level. This is hardwarily permissible,

and generally doesn't create any problems since most DOS and Windows

software doesn't use interrupts for printing anyway. However, if you

are using LPT software like a print spooler or interrupt-driven LPT

devices like external hard disks, external network adapters, and so

forth, then you have to be sure that they are either (1) not trying to

use the same interrupt level, or (2) capable of "interrupt sharing".

When adding or configuring LPT cards, we suggest using 3BC, IRQ 7 for

the first card; 378, IRQ 5 for the second card; and 278, IRQ 5 for the

third card. This gives you two interrupt-capable LPT ports, and one

port (at 278) which you should use only for non-interrupt driven usage,

like status-driven spooling.

The "yes, but's" to this are: (1) you can allow LPTs to share an

interrupt level if you are sure that the software which uses them is

either non-interrupt software, or is smart enough to do interrupt

sharing, (2) you might need to keep IRQ 5 free to handle some other

non-LPT accessory like a network card, a frame grabber, or whatever in

a system that is running out of free IRQ levels.

In any case, arming with this understanding and the Parallel Utility,

you are equipped to select a configuration which works for you.

UTILITY:        The IO ADDRESS is a standard parameter in network

configuration files where it takes on different

names and syntacies to confuse innocent users.

Some simple-minded parallel port peripherals (ones

without auto-configuring capability) require that

specific the IO address, IRQ level, and sometimes

the Port Type in a vendor-specific way. Parallel

provides you with this information.

See Configuring Networks and Devices below.

This comment applies to using the IRQ data as well.

IRQ Level

---------

This is the hardware interrupt level assigned to the port. It will

always be either 7 or 5.  Normally IRQ 7 is assigned to the first

port and IRQ 5 is assigned to the second port. However, due to the

explosion of clones assembled from disparate parts, it is not uncommon

to see systems with one LPT port located at 378 with an IRQ of 7 or 5.

This is because clones are often assembled using offshore add-in LPT

cards which are shipped configured to operate as a second LPT card.

On ISA cards (bus cards in ISA slots) the interrupt level is usually set

with jumpers on the board - these jumpers are different for every card

so you need to hunt around for the manual in order to change them. If

you're lucky there will be readable jumper labels on the card itself.

On laptops, the interrupt level can usually be set using the vendor's

specific CMOS configuration (setup) program. Sometimes these CMOS setup

programs are built into the ROM; sometimes they exist as separate

programs or as part of the vendor's diagnostic program(s). Refer to

your manual to find out how to enter the CMOS setup mode. BE CAREFUL

when using the CMOS setup, since it gives you access to parameters that

can disable your system if wrongly changed. KNOW WHAT YOU ARE DOING.

Another strange feature of these software configurable ports is that

they often have very peculiar ideas about nomenclature. Often these CMOS

setup programs use vendor specific "magic code words" to select various

parallel port configurations, and these magic words can have non-obvious

effects. Sometimes the vendor's manuals don't even provide a decoder

ring telling you what their magic words really mean in terms of IO

Address, IRQ, and Port Type. Again, Parallel is useful for examining

the actual effect of the vendor's CMOS setup choices.

For example, on my Dell NL25 laptop, 4bit mode is called "Normal" and

8bit is called "Enhanced" and there is no choice at all for the

chipset's built-in Enhanced Parallel Port (EPP port). So someone might

obviously think they have enabled EPP by choosing "Enhanced".

Futhermore, the IRQ level cannot be set directly, but is set as a

by-product of choosing combinations of the IO address and the Port Type.

Most of the choices yield IRQ 7, but one lonely choice - LPT1, 278,

Normal mode results in IRQ 5. It makes a strange sort of sense when you

are experienced with parallel ports, but is likely to be confusing to

the average user (if we ever find him, or her). The Dell is not alone

in it's strange naming conventions, most laptop's have confusing,

misleading, or otherwise incomplete controls in the CMOS setup.

So, Parallel will come in handy in trying to make sense of what your

laptop's CMOS settings are actually doing to the parallel port setup.

A note about Loopback and Floating Ports

----------------------------------------

Parallel's autodetection of the IRQ is only 100% reliable when

you are using a loopback connector as described below. Parallel

may report valid IRQ detection, attributed to either loopback or

float, when there is an external device (such as a printer or

even a long cable) connected to the port. This is especially likely

if the external device is powered off (a turned-off printer).

We have never seen a case where loopback provided by an inactive

device resulted in false IRQ detection, but we recommend

disconnecting inactive devices and using loopback for best results.

Loopback and floating ports are discussed in more detail below.

BIOS Name

---------

This is the "human-readable" (hah!) name which you use to refer to the

port when using DOS commands, Windows picklists, and so forth. It will

be one of LPT1, LPT2, LPT3.

The port address is NOT the same as the BIOS name for the port. For

example, LPT1 can refer to a port at 378 (or anywhere else for that

matter).

How does the BIOS associate the LPT names with the actual ports?

Whenever your system boots, the BIOS goes out and tries to write and

readback an A5 and then a 5A from locations 3BC, 378, and 278. If it

can read back what it writes, then the BIOS decides there is a port

there and adds it to a LPT table in the "BIOS Data Area". This table is

four words located at 40:8. A command like "copy foo.txt lpt2:" means

"copy the file foo.txt to the second printer that BIOS knows" and DOS

uses a software interrupt, INT 17h, to do just that. INT 17h looks in

the BIOS Data area to find out the IO address of the "second printer",

which can be any of the IO Addresses - 3BC, 378, or 278.

The result is that LPT1 refers to the first port BIOS finds at boot

time, wherever it may be. So "LPT1" means "the first printer", not

necessarily 3BC, 378, or 278.

Another factor here is that various programs can fiddle with the

LPT table. The utility SWAPPRT, for example, can be used to swap

LPTx with LPTy which it does by moving the entries in the LPT table.

Notes

-----

The "Notes" field of the display will contain various comments,

depending on what Parallel has detected. Each note may be followed

by one or more comments giving further information or directions.

These secondary comments are indented one space under their note.

"No port found at this address"

A parallel port is not installed at this IO Address.

However, it is also possible that an active external device

is connected to the parallel port and is overdriving the port's

inputs, which can prevent the software readback test from working.

"Need loopback to determine IRQ"

The port exists, but does not respond to attempts to generate IRQs.

This is because a loopback connector, external to the port, is

required for IRQ detection in most cases.

If you correctly install the loopback connector and this message

persists, then the port's IRQ is not functioning.

"IRQ determined by loopback"

The loopback was detected, and the port IRQs are functioning.

"IRQ determined by port float"

No loopback connector was detected, but the port is generating

IRQs anyway. This probably means that the port is connected to

an inactive (unpowered) external device. Treat IRQs determined

in this way with some skepticism; better to disconnect the device

and use a loopback connector.

"IRQ determined by EPP hardware"

The IRQ is determined by the behavior of the Intel 360SL chip.

In other words, if the Port Type is determined to be EPP, then

Parallel does not use the IRQ test, but rather reports the IRQ

which is standard for an EPP port at that IO Address.

Automatic IRQ detection

-----------------------

Two methods are available for automatic detection of a port's IRQ

level: port float and loopback.

Port Float

----------

Some parallel ports will "float" in a unstable logic state when

connected to an unpowered external device such as a printer.

This unstable logic state may cause the port to generate a

limitless stream of apparent interrupts to the CPU if the port's

interrupt are enabled. This peculiar behavior is used to

advantage by Parallel.

If you have a unpowered (powered off) printer, or other device,

attached to a port, then Parallel may be able to detect the

interrupt level using this floating port behavior. In this case,

Parallel will display "IRQ determined by port float". You should

treat the IRQ determined in this way as a "best guess". The best

alternative is to disconnect the floating device and use loopback.

In some cases, attaching a unpowered device to a port will enable

Parallel to detect IRQ by loopback, since the combination of

port and device appears to Parallel indistinguishable from loopback.

FLOAT NOTE: Powered on devices usually hold be interrupt line in

a stable state and prevent IRQ autodetection. Again, it is best to

disconnect your parallel port device(s) and use the loopback

connector when you need the absolute best results from Parallel.

EXAMPLE: On my home system, an HP LaserJet II printer attached

to an 8bit port "floats" and reliably reports IRQ5. Attached to

a 4bit port on the same system, the LaserJet in indistinguishable

from a loopback connector.  So, turning the LaserJet off

converts it into very expensive loopback paperclip.

TECHNICAL NOTE: Whether or not a given device and port

combination will float, emulate loopback, or act invisibly to

Parallel is determined by the pull-ups on each end, CMOS vs TTL

logic behavior, and the power available at the port to a

connected but unpowered device. In the above example, I suspect

that just enough power is provided by the 8bit TTL port to drive

the unpowered CMOS logic in the LaserJet into oscillation, which

appears to the 8259 and CPU as a frenzy of level 5 IRQs. So, I

repeat, treat IRQs determined by "float" with a dram of salt.

Loopback

--------

The most reliable way to determine IRQ is to use an external

loopback connection. This is described in the next section.

IRQ detection using the LoopBack Feature

----------------------------------------

First, DECIDE NOT TO SCREW UP YOUR PORT by crudely jamming wires into

the wrong places with too much force. It's pretty difficult to actually

break the DB-25 connector, and parallel ports are quite resistant to

electrical surprises, but nevertheless YOU HAVE BEEN WARNED to follow

the directions carefully and to be gentle when inserting wires into the

connector. REMEMBER that the female DB-25 connector is likely to be made

out of CHEAP, BREAKABLE PLASTIC, so decide now not to break it. WE ARE

NOT LIABLE FOR ANY DAMAGE YOU MAY DO FOR WHATEVER REASON.

The actual danger is that you will push too hard when inserting the

loopback wire and force the female connector pin out of its position

in the DB-25 shell. That is why WE SUGGEST MAKING A LOOPBACK PLUG

using a male DB-25 connector as described below. This protects the

port from physical damage. The port is electrically designed to

tolerate the loopback perfectly well. Also there is a DANGER OF DAMAGE

BY STATIC DISCHARGE so make sure that you GROUND YOURSELF TO THE PORT

before inserting the loopback wire. It would be terrible if you blew

out a $25 parallel port on a $1000 486 motherboard simply because you

failed to PAY ATTENTION TO THESE PRECAUTIONS. PROCEED AT YOUR OWN RISK.

The other danger is that you will apply the loopback to a port which

is not really a parallel port. DON'T DO THIS. If you are not sure of

what you are doing STOP RIGHT NOW. We have no control over what you

might damage if you try this with a port other than a parallel port.

Whew... on with the instructions.

1. Find the parallel port(s) you want to test. They will be

female DB-25 connectors. MAKE SURE THAT YOU HAVE CORRECTLY

IDENTIFIED THE PARALLEL PORT. You may want to disconnect any

devices (printers, network adapters, external disks, etc) which

might be attached to other parallel ports on your system, but

this is not thought to be necessary. It is theoretically

possible for parallel port devices and/or parallel port

interrupt software to confuse the IRQ detection routine and,

while we have never seen a case of this, you might want to

eliminate these other possible IRQ sources, especially if you

need to be absolutely sure that the results are accurate.

2. Make sure that the connector IS NOT SOME OTHER TYPE OF

CONNECTION like an external SCSI connector or a COM port.

SCSI ports can be either female DB-25s or a larger connector,

like a 37, 40, or 50-pin connector.

COM ports are usually male DB-25 or small male DB-9 connectors,

but this is not guaranteed.

Look in your manual if there is any doubt. DON'T PROCEED unless

you are sure you have identified the parallel port(s) correctly.

3. Identify Pin 10 and Pin 9.  Refer to the illustration below.

Notice that a DB-25 plug is wider on one side.  The pins you

want are the 4th and the 5th from the left on the wider edge.

USE A FLASHLIGHT TO FIND THE MARKINGS IF YOU HAVE ANY DOUBT.

-------- Pin 10 - 4th from left on wider edge

Female        |

DB-25         |    --- Pin  9 - 5th from left on wider edge

Connector     |    |

v    v

/===========================================================\

\    13  12  11  10   9   8   7   6   5   4   3   2   1     /

\     14  15  16  17  18  19  20  21  22  23  24  25      /

\=======================================================/

Pin 10 is the interrupt line (the ACK line)

Pin  9 is the high order data line (the 80h bit)

4. Create a loopback wire. A paperclip works fine. Just straighten

it out and bend it into a "U" shape with the ends 1/8 inch apart.

5. GROUND YOURSELF to the PC and to the parallel port by touching

the loopback wire first to the PC's metal chassis near the

parallel port connector and then to one of the connector nuts

or the connector metal housing itself. Do this to DISCHARGE

ANY STATIC ELECTRICITY you may have about you (created for

example by scooting across the rug in a silk negligee on the

way to your PC or by stroking your cat with woolen gloves).

Parallel ports are usually resistant to static discharge, but

not all ports are created equally, so don't ask for trouble.

6. Carefully insert the paperclip into the parallel port's DB-25

connector so that Pin 9 is connected to Pin 10. DON'T FORCE IT.

It will go in at most about half an inch (about 1 cm) and there

should be absolutely no physical resistance.

Better yet, if you are handy with a soldering iron, get a male

DB-25 plug and jump solder or otherwise wire Pin 9 to Pin 10.

This will prevent any physical damage to your port, which is

the major source of risk with this procedure, and the reason

for our incessant nagging about precautions. Such a loopback

plug will be no more dangerous to your system than a printer,

and is a handy accessory if you frequently use Parallel.

7. Now run Parallel. It will automatically use the loopback to

determine which IRQ level the port(s) are actually using. You

may loopback more than one, or all, ports at the same time.

8. Remove the loopback when you are done.

9. Congratulate yourself on using reasonable handling precautions

when working with expensive equipment.

Configuring Network Software

----------------------------

Armed with the information that Parallel reports, you are equipped

to perform manual configuration of network adapters and other

parallel port peripheral devices, should they require such manual

configuration. Here are a few examples of how Parallel's IO address

and IRQ level are used in popular network software configuration

files when configuring them for use with parallel port adapters.

Note that these examples also apply to our Pipx and Pndis drivers

which allow NetWare Lite, Windows for Workgoups, and LAN Manager

networking using simple parallel port cables between two systems.

Test Drive versions of Pipx and Pndis are available on CompuServe,

or you can buy the commercial versions directly from us. These

drivers allow you to use your favorite network software between

two systems through the parallel port, without network adapters.

This is a handy capability for laptops, and in other situations

where you need truly portable and universal networking at low cost.

Pipx and Pndis support several different types of cables, ranging

from the low-performance "lap-link" type cable you may already

have up to our fabulous Universal High-Speed Turbo Cable (whew!)

which can do parallel port networking at 200 KBPS or more.

In the following examples, assume Parallel has reported a parallel

port at IO address 3BC, IRQ 7.

These examples are not meant to be cookbook instructions, but

are offered to give you a feel for how to apply the information

displayed by Parallel.

NetWare NET.CFG file for ODI drivers

------------------------------------

[in the Link Driver section]

PORT 3BC

INT  7

LAN Manager or Windows Workgroups PROTOCOL.INI file

---------------------------------------------------

[in the appropriate adapter section]

IOBASE = 0x3BC

IRQ    = 7

Lantastic (parameters on command line, maybe in STARTNET.BAT)

-------------------------------------------------------------

C:\> <drivername> IRQ=7 IOBASE=3BC

Other device configuration files and programs

---------------------------------------------

If you are having trouble installing a parallel port peripheral,

be prepared to run Parallel when calling your vendor for technical

support. They will be better able to help you if you can tell

them your exact parallel port configuration.

Technical Support

-----------------

Parallel is Freeware, and comes with Freeware support.

We are willing to offer friendly assistance on an as-available

basis, but only via CompuServe mail (CIS 76640, 203).

We do not offer free telephone assistance to the general public.

If you are in dire need of technical support in solving parallel

port related problems, you may open a support account with us by

contacting:

Parallel Technologies, Inc.

1234 Redmond Way, Suite 789

Redmond, WA 98052

(206) 869-1136

(206) 869-1133 (fax)

CompuServe ID# 76640, 203

Internet: 76640.203@compuserve.com

The Parallel software, and other parallel port software utilities

and developer's libraries, example drivers, hardware prototyping

tools, contract development services, and a wide range of OEM

parallel port products are available from Parallel Technologies.

Custom versions of the Parallel Port Information Utility are available.

Please call us if your firm would benefit from a custom or

device-specific version of the Parallel Port Information Utility.

Technical Notes and Comments

----------------------------

(1) EPP TESTING. Parallel currently will detect only EPP ports in

systems with Intel SL chipsets. This generally means laptops

that have an SL suffix on the name.

WISHLIST: detect and report presence of other EPP ports supported

by an EPP BIOS extension

(2) PRESERVING STATE. Parallel completely preserves the state of

your system AND of the parallel ports themselves.  Parallel

installs and deinstalls vectors for IRQ 5 and 7 to attempt

loopback detection of the IRQ level. Parallel may also perform

data and control writes to the port, but the lines are left in

their original state and no "strobed" data is written.

(3) PRINTERS and PARALLEL PORT PERIPHERALS. Parallel writes a short

alternating sequence of 00 and FF bytes as part of the loopback

test. These writes MAY cause printers to initialize or other

parallel port peripherals (network adapters, disks, etc) to get

into funny states. Our tests have not revealed any cases of the

program actually affecting any attached devices.

You may wish to remove any attached devices before running the

program, but this is not necessary.

Software License Agreement

--------------------------

Parallel Technologies, Inc.

Freeware Software License Agreement

IMPORTANT

Read this agreement before installing or using the Software. Installing

and/or using the Software on your computer indicates your acceptance of

these terms. If you do not agree to these terms and conditions, delete

the Software and all it's associated files from your computer. This

Software License Agreement gives you ONLY RESTRICTED RIGHTS to use the

Software (and all it's associated files and documentation) as follows:

YOU MAY

(1) Use the Software without charge on as many systems as you wish,

(2) Share the Software with others, but only if you share it intact,

without any changes or modifications of any kind, and complete with

all it's documentation and associated files. You must pass it on

exactly in the same form as we release it, or not pass it on at all.

(3) Make backup copies of the Software, so long as you place our copyright

notice on the backup copies,

(4) Request and receive limited technical support as provided in the

Freeware Product Support Policy below.

YOU MAY NOT

(1) Use the Software in any way except as provided in this agreement,

(2) Modify, alter, copy, transfer, or reproduce the Software in any way,

except as expressly allowed in this agreement,

(3) Translate, de-assemble, de-compile, reverse program, or reverse

engineer the Software or any of its components,

(4) Charge any kind of fee for distributing the Software without our

written permission,

(4) Sublicense, lease, rent, sell, distribute, or otherwise dispose of

the Software without our written permission.

This license is effective from the date of purchase and shall remain in

force until terminated. You may terminate this agreement by  destroying

any and all copies of the Software and its documentation. Parallel

Technologies, Inc. may terminate this agreement if you fail to comply

with any of the terms and conditions of it. You agree to destroy any and

all copies of the Software and its documentation, in whatever form, if

we terminate this license agreement, and you agree to provide us written

certification of such destruction upon our request.

Freeware Product Support Policy

Parallel Technologies will provide limited technical support for

the Software via CompuServe Email (76640,203) on a purely "as available"

basis at Parallel Technologies discretion. Technical support is limited

to assistance only with the correct operation of the Software as provided

for in the program documentation and does not necessarily extend to

assistance with network, hardware, or software configuration.

Additional technical support may be provided on a time and materials basis.

Call Parallel Technologies (206-869-1136) for more information.

Statement of Exclusion of Warranty

ALL PRODUCTS DELIVERED PURSUANT TO THIS AGREEMENT ARE DELIVERED ON AN

"AS-IS" BASIS AND PARALLEL TECHNOLOGIES, ITS DEALERS AND DISTRIBUTORS,

EXPRESSLY DISCLAIM ANY AND ALL WARRANTIES OR CONDITIONS OF MERCHANTABLE

QUALITY OR FITNESS FOR A PARTICULAR PURPOSE.

Limitation of Liability

PARALLEL TECHNOLOGIES DISCLAIMS ALL WARRANTIES RELATING TO THIS

SOFTWARE, WHETHER EXPRESS OR IMPLIED, INCLUDING BUT NOT LIMITED TO ANY

IMPLIED WARRANTIES OF MERCHANTABILITY AND FITNESS FOR A PARTICULAR

PURPOSE, AND ALL SUCH WARRANTIES ARE EXPRESSLY AND SPECIFICALLY

DISCLAIMED.  NEITHER PARALLEL TECHNOLOGIES NOR ANYONE ELSE WHO HAS BEEN

INVOLVED IN THE CREATION, PRODUCTION, OR DELIVERY OF THIS SOFTWARE SHALL

BE LIABLE FOR ANY INDIRECT, CONSEQUENTIAL, SPECIAL, PUNITIVE, OR

INCIDENTAL DAMAGES OR DAMAGES DUE TO LOST PROFITS, DATA, OR INFORMATION

ARISING OUT OF THE USE OR INABILITY TO USE SUCH SOFTWARE EVEN IF

PARALLEL TECHNOLOGIES HAS BEEN ADVISED OF THE POSSIBILITY OF SUCH

DAMAGES OR CLAIMS.  IN NO EVENT SHALL PARALLEL TECHNOLOGIES' LIABILITY

FOR ANY DAMAGES EVER EXCEED THE PRICE PAID FOR THE LICENSE TO USE THE

SOFTWARE, REGARDLESS OF THE FORM OF CLAIM.  THE PERSON USING THE

SOFTWARE BEARS ALL RISKS AS TO THE QUALITY AND PERFORMANCE OF THE

SOFTWARE.  Some states do not allow the exclusion of the limit of

liability for consequential or incidental damages, so the above

limitation may not apply to you.

This agreement shall be governed by the laws of the State of Washington

and shall inure to the benefit of Parallel Technologies, Inc. and any

successors, administrators, heirs and assigns.  Any action or proceeding

brought by either party against the other arising out of or related to

this agreement shall be brought only in a STATE or FEDERAL COURT of

competent jurisdiction located in King County, Washington.

=================================================================

=                       FBI WARNING                             =

=      Failure to observe the restrictions of this license      =

=                     is a violation                            =

=       of 17 United States Code sections 101 through 810       =

=     which carries severe personal and corporate penalties.    =

=================================================================

Record of Testing

-----------------

Version Vendor  Model   Vendor's Configuration          Parallel's Result

------- ------  ------  ---------------------------     -------------------

0.20   Dell    NL25    Off                             OK - No ports detected

0.20   Dell    NL25    LPT1, 3BC, Normal               OK - 4bit, 3BC, 7

0.20   Dell    NL25    LPT1, 3BC, Enhanced             OK - 8bit, 3BC, 7

0.20   Dell    NL25    LPT1, 378, Normal               OK - 4bit, 378, 7

0.20   Dell    NL25    LPT1, 378, Enhanced             OK - 8bit, 378, 7

0.20   Dell    NL25    LPT1, 278, Normal               OK - 4bit, 278, 5

0.20   Dell    NL25    LPT1, 278, Enhanced             OK - 4bit, 278, 7

0.20   Dell    NL25    LPT1, 278, Enhanced, EPP        OK - EPP,  378, 7

0.20   Clone   486/33  ISA 4bit card, 378, 7           OK - 4bit, 378, 7

0.20   Clone   486/33  ISA 8bit card, 3BC, 5           OK - 8bit, 3BC, 5