DATE:	1/8/96

SUBJECT:	Release Notes for AMI System Configuration Utility 3.30 and Overlay 1.20

## About This Release:

1.	This is the release of Overlay 1.20 and the AMI System Configuration Utility  3.30 for use on the following platforms:

Xxpress 15”  -- int3190

Xxpress 12”  -- int3180

AltServer  -- int31A0

Alder  -- int31C0

2.	NOTE:	AMI System Configuration Utility 3.30 and Overlay 1.20 should only be used with the following BIOS versions:

Platform	       First compatible BIOS        Latest Compatible Release

Xxpress 15” 	Release 1.00.6			R1.00.6

Xxpress 12” 	Release 1.00.4			R1.00.4

AltServer  	None			None (as of R1.00.17)

Alder (Alder)		Beta 0.01.1		No Release (B0.05.1)

## Issues Resolved in this Release:

- If all A/D Channels are set to disable and saved, the SCU will correctly save the settings and will reload settings correctly on the next run (previously would reset to defaults.) DCS #2219

- The SCU performs a resource check on the standard VGA BIOS location (C0000h-C8000h) and warns the user if this resource is not reserved by a video device.  (DCS #2103)

- The AMI SCU may be run without DOS loaded high. (DCS #2388)  The AMI SCU will no longer report “XMS Memory Paging Failure” if DOS is loaded low, but the user may experience low memory problems if too little conventional memory is available.

- Addition of EISA cards possessing OVLs capable of multi-initialization will not report “CFG File Errors.”  (DCS #2273)

- The correct resource and bit settings are reported for Serial/Console Redirection. (DCS #2125, #2280)

- When reading the diskette volume serial number to verify the correct diskette during data transfers/copies, the AMI SCU will insure that no DMA transfers across 64KB boundaries are attempted.  This will prevent the AMI SCU from erroneously identifying the AMI SCU diskette and/or source diskettes.  (DCS #2312, #2422)

- Help text was added to the A/D Channel Switch Menu and the ISA ROM Memory Shadow Options Menu to indicate ‘SPACE’ will toggle the selection.  DCS #2357

- Some error messages containing misspellings were corrected.  (DCS #2409)

- The maximum amperage for EISA/ISA cards was increased to 3A/slot.  (DCS #2390)

- Text for ‘Lock Toggle” button changed to ‘ISA/EISA Lock Toggle’ (DCS #2226)

## Features Added or Changed in this Release:

- The Overlay Lite for the AMI SCU is not available with this release of the SCU, the issues forcing its use have been resolved.  Future releases of the AMI SCU will not include or necessarily be compatible with Overlay Lite.

- The baseboard OVL will warn the user when attempting to restore an unidentifiable CMOS image or a CMOS image from a BIOS version other than the current system BIOS.  CMOS images created by baseboard OVLs older than 1.20 (except 1.02) will be unidentifiable.  An override feature is available to force the restoration. (DCS #2402)

- A “PCI System Error Detection” option was added to the baseboard OVL to allow enabling and disabling of PERR and SERR on the PCI bus.  (DCS #2285)

- The Global Resource Map displays the user defined board name for user defined ISA cards instead of a generic label.  (DCS #2410)

- A toggle switch was added to allow toggling between Celsius and Kelvin in the Voltage/Temperature Limits menu.  (DCS #2369, #2210)

- An option was added to the boot device menu to allow specification of CD-ROM hard disk image or CD-ROM diskette image.  (DCS #2314)

- A menu was added to allow enabling or disabling of the PCI locking feature (Xxpress 15” only.)  (DCS #2315)

- The text for the password menus will display “New Password” when no password is present and “Change Password” when a password is present.  (DCS #2371)

- Several enhancements to the PnP card functions have been made to the AMI SCU and associated BIOS.

## Outstanding issues:

- The SCSI BIOS version information is not available in the System Information Log. (DCS #2183)

- IRQ 12 is always assigned to the Mouse and can not be freed for other resources.

- The AMI SCU may not used with extended memory managers such as “EMM386.”

- If himem.sys does not load, viewing help when editing functions in an adapter card or baseboard may result in an “Out of Memory” error.  If less than 600KB of conventional memory is available to the AMI SCU, viewing help when editing functions in an adapter card or baseboard may result in an “Out of Memory” error.

In order to help track memory usage, the AMI SCU includes two memory features:

- Pressing <CTRL-M> at the main menu will display the largest free memory block (this value is effectively the amount of free memory in the system.)

- Pressing <CTRL-F> at the main menu will toggle-on (or toggle-off) a dynamic memory meter that appears in the upper left-hand corner of the screen.  This “gas gauge” meter will be erased at every screen update by the AMI SCU and is sometimes covered up.  Asking for “Help (F1)” will usually provide a menu that displays this meter without being “covered up” right away.

- The <CTRL-M>  and <CTRL-F> features are only available at the main menu.

AMI SCU Usage

Main Boot Menu for AMI SCU

Perform the following steps to obtain a working version of the AMI SCU:

- Format and ‘sys’ a new diskette with MS-DOS.

- Copy the contents of the provided System Configuration Diskette onto the newly created bootable diskette.

- Copy the file “himem.sys” from the “DOS” directory onto the bootable diskette.

- Boot the server system using the diskette.

- When the system boots, the user will be presented a main menu with a number of choices:

1. Execute AMI SCU

2. Execute AMI SCU (baseboard configuration only)

3. Execute AMI SCU using the Overlay Lite

4. Execute SCSISelect (TM) Utility (int31A0/ALTServer baseboard only)

The user must choose one of the options presented.

Execute AMI SCU Selection

By selecting this option, the AMI SCU will automatically locate all EISA/ISA PnP/PCI devices and configure them into the system per the Plug-N-Play and PCI specifications.

Execute AMI SCU (baseboard configuration only) Selection

This option is no longer available.  Overlay Lite is no longer available--the issues forcing its use have been resolved.

Execute AMI SCU Using the Overlay Lite Selection

This option is no longer available.  Overlay Lite is no longer available--the issues forcing its use have been resolved.

Execute SCSISelect(TM) Utility Selection

This option is only for use on the ALTServer system platform.  This option was installed to select the onboard AIC-7870 fast/wide SCSI controller.  The SCSISelect utility enables and configures the AIC-7870 SCSI controller on the ALTServer platform.  The SCSISelect configuration menu can be enabled on either Xtended Xpress system platforms by pressing <CTRL-A> during system initialization.