NETBIOS for the National Semiconductor Corporation Ethernet Adapters

INSTALLATION:

To Install the NETBIOS using the accompanying INSTALL.BAT:

Change drives to the appropriate drive containing this diskette.   This

may be either A: or B:.

At the DOS prompt, type "install d" and press <return> where d is the

destination drive.  For example, to install from drive B: to drive C:,

type "install c" and press return.  This will copy the NETBIOS.EXE file

to the root directory of drive C:.

To Install the NETBIOS without using the accompanying INSTALL.BAT:

Change drives to the drive from which the NETBIOS will be executed.

Using the DOS COPY command, copy NETBIOS.EXE from the diskette to the

root directory on drive C:.  For example, if the diskette is in drive A:

and NETBIOS.EXE will be executed from drive C:, you would perform the

following steps:

c: <return>

copy a:\netbios\netbios.exe c:\netbios.exe <return>

COMPATIBILITY:

This NETBIOS will work on the NATIONAL SEMICONDUCTOR's Ethernet Adapters.

NETBIOSes from different manufacturers may not be compatible with this

NETBIOS.  The NETBIOS provided by National Semiconductor is compatible

with NETBIOSes provided by Performance Technology, Inc.  Performance

Technology provides interoperable NETBIOS support for NE1000 and NE2000

adapters, as well as Western Digital WD8003 and WD8013, Pure Data PD8023 and

PD8023-16, 3COM 3C503 and 3C505, and Xircom's Pocket Ethernet Adapters

and adapters compatible with them.  In addition, interoperable NETBIOSes for

both XENIX and OS/2 are available for some of the above adapters including

NATIONAL SEMICONDUCTOR's Ethernet Adapter.

For more information regarding other NETBIOSes, contact Performance

Technology at (512)349-2000.

OPTIONS:

Usage:      NETBIOS <option> ...

<option>:   /LN      run as adapter 1 for bridge

/SES:<n> set number of sessions (default 32)

/NAM:<n> set max number of names (default 17)

/LP:<n>  set base port address (default 300)

/LI:<n>  set IRQ number (default 3)

/H       see help screen