float - floating point number formats

Regulus supports two formats of single  precision  floating

point  numbers.  They are the Motorola 'Fast Floating Point' and

the 'IEEE standard'.  Both types are stored in 32-bit  entities.

They are not compatible.

The FFP format consists of a 24  bit  mantissa,  1  bit  of

sign,  and  7  bits of exponent.  The exponent is stored in it's

two's complement form.

[ | | | | | | | ~~~~ | | | | | | | | | | | | | | | | ]

|------------ Mantissa --------------|S|---- Exp ----|

The IEEE format consists of 1 sign bit, 8 bits of exponent,  and

23  bits  of mantissa.  The exponent is store in it's Excess 127

format.

[ | | | | | | | | | | | | | | | ~~~~ | | | | | | | | ]

|S|----- Exp -----|---------- Mantissa --------------|

The floating point numbers represented by the IEEE format are  :

(-1)^S  *  1.F  * 2^E, the FFP floating point numbers are of the

form : (-1)^S * 0.F * 2^E.

The IEEE format will be used if the -e flag is given to the

compiler  or  if  the  -lE flag is given to the loader.  The FFP

format will be used if the -f flag is given to the  compiler  or

if  the  -lF  flag  is  given  to the loader.  These flags cause

floating point constants of the appropriate flavor  to  be  gen-

erated and the appropriate library to be loaded.

The IEEE floating point values lose some precision  due  to

the fact that calculations use the fast floating point routines,

and a  translation  to  and  from  the  FFP  format.   cc(cmnd),

lo(cmnd)